3GPP TSG-RAN WG4 Meeting #116b                                                                             R4-2514072

Prague, Czech, Oct 13th – 17th, 2025

Title:	CSI compression on Low‑Complexity Encoders under SGCS‑4

Source:	Korea Testing Laboratory

Agenda item:			7.9.3

Document for:	Discussion

- Introduction

This contribution discusses Option 3, Track 2 considering low‑complexity encoders. It follows the way‑forward captured at RAN4#116 and aligns with Topic #2 agreements on differentiating backbone vs. complexity effects, reporting learning‑curve stability, and addressing fairness in N‑to‑1 Multi‑encoder structure‑aware training. The focus is SGCS convergence, not absolute numbers.

We evaluate a data‑side method only: an eigenvector‑based subspace pre‑processing applied to the mixed dataset while leaving the encoders, decoder, and SGCS‑4 procedure unchanged. The question is whether such pre‑processing can lift weak compatibility cases without altering the training recipe.

We also reuse the SGCS‑4 training procedure and loss form previously described by [2], including the encoder‑agreement term used to limit anchor bias during Step‑1.

Table 1. Ad-hoc summary for [116][132]NR_AIML_air_part2  [1]

- Recommended WF(Agreements) - It is important to differentiate in this evaluation between the impacts of different encoder backbones and the impacts of different complexity levels - The following observations are valid with the mixed dataset; there can be dataset dependency for other datasets - 1-1 joint training: - For 1-1 joint training, some, although not all, companies observed that the low complexity MLP encoder (Encoder 4) had a reduced performance compared to the other encoders. - Training based on the Samsung frozen decoder - When trained using the Samsung Frozen decoder, many, although not all, companies observed that the MLP encoder had a loss compared to the other encoders - One company saw SGCS loss for all of the low complexity encoders (Transformer, CNN and MLP) when trained with the Samsung frozen encoder. - Based on the majority company results, it might be concluded that the lower complexity transformer and CNN encoders are compatible with the Samsung frozen decoder (which was trained using the agreed “high complexity” encoder), but the MLP may not be. - Training based on N-1 decoder - It is proposed to name training based on the N-1 decoder as “N-to-1 Multi-encoder structure aware training” - With N-to-1 Multi-encoder structure aware training, most companies observed that the performance of the MLP low complexity encoder could improve relative to the other encoders (depending on company, possibly still a slight loss) - One company observed that if lower complexity CNN encoders are used with N-1 joint training then the SGCS becomes lower for the low complexity CNN encoders. - One company observed that the choice of anchor and method used to train the N-to-1 Multi-encoder structure aware training may impact the performance and capability with different encoders. - Summary conclusion: - If a high complexity test decoder is used, it seems to be compatible with different encoder backbones (at least CNN and transformer) - Whether a low complexity decoder can be compatible with different encoder structures was not studied - The benefits of N-to-1 Multi-encoder structure aware training are not fully clear, however it may improve the performance of the lowest complexity MLP encoder

- Simulation

2.1 Dataset subspace pre‑processing

- Compatibility Verification of Low-Complexity Encoders and Given Models for Option 3 with SGCS-4 training manner with data set subspace feature engineering

  - Motivation:

- In the summary of last meeting, ZTE proposes that ‘The dataset that is captured in the WI should be Eigenvectors’.

- We try to apply subspace feature engineering for the given data set, and retrain the low complexity encoders, verifying whether implementing subspace feature engineering for training date is benefit for the compatibility of the low complexity encoders.

  - Methodology for the dataset subspace feature engineering

    - Reduce channel redundancy and promote physically meaningful low-rank structure without changing the tensor shape seen by the network.

    - The encoders/decoder and the SGCS-4 manner remain untouched; only the input to encoders is replaced by a shape-identical low-rank reconstruction of the original samples.

    - The shape of given data set in ‘MIXEDR4_114dsei00.zip’ is (500000, 2, 13, 32), where ‘500000’ is the number of data samples; ‘2’ represents two channels of real and imaginary part of the complex channel; ‘13’ represents the number of subcarriers; and ‘32’ represents the number of antennas equipped on the BS.

    - Processing steps:

      - For each sample H∈R2×13×32, we implement the following operations:

      - Step 1: Preprocess

        - Complexing casting: Cast the real and imaginary parts, transform the original data into complex value.

H→Hc∈C13×32

        - Normalization: Normalize Hc by rescaling the root mean square (RMS).

Hn=Hc113×32HcF,

Where ||*|​F denotes the Frobenius norm.

      - Step 2: Egin extraction

        - Subspace extraction (SVD): Compute the compact SVD

Hn=Udiag(σ)VH,

Where σ represents the singular value vector.

        - Rank selection: Define the maximum rank of Hn as r, select the top r singular value σ[1:r] and its corresponding singular vector.

      - Step 3: Training data reconstruction

      - Low-rank reconstruction: Form

H=U:,1:r​diagσ[1:r]​V:,1:rH​.

      - Residual mixing: Blend the reconstruction with the normalized input:

H'=α⋅H+1-αHn,  α=max(α0, min(1, i=1rσi2j=rσj2))

with α0​∈[0, 1] (default 0.7). This adaptive α raises reliance on H when the retained-energy ratio is high and reduces it otherwise—mitigating distribution shift while preserving denoising benefits.

      - RMS rescaling:

Hn'=H'*113×32HcF,

      - Re/Im packing (shape identical to original)

Hn'→H∈R2×13×32

    - Keep the new data and denote as ‘MIXEDR4_114dsei00_subspace.zip’.

  - Decoder: Fixed by agreed decoder: The decoder given by Samsung.

  - Encoder:

    - Build new model for three low complexity encoders given in 3GPP TSG RAN WG4 Meeting #115 R4-2508084:

- Encoder 2 (Transformer-based),

- Encoder 3 (CNN-based),

- Encoder 4 (MLP-based).

- Reuse the encoders given by each company as the Encoder 1, respectively.

  - Training:

    - Perform training with SGCS-4 manner:

Fig. 1. Training procedure of SGCS-4.

Fig. 2. Detailed training procedure of SGCS-4 step-1

  - Data set:

    - Train = The first 70% of reconstructed data set(subspace) (‘MIXEDR4_114dsei00_subspace’) (from the 1st to 350000th sample).

    - Test = last 10% of original data set (‘MIXEDR4_114dsei00’) (from the 450000th to 500000th sample).

  - Provide result for

    - The SGCS values by testing on 2-step trained Encoder 1/2/3/4.

      - We implemented encoder given by Samsung and VIVO as Encoder 1, respectively.

      - For step 1, we implemented 10 epoch training.

      - For step 2, we implemented 10 epoch training.

k

Table 3. . Result for comparison of testing on low complexity encoders by row channel (Samsung’s decoder).

| Category | Testing on Encoder 1 (Step-2) | Testing on Encoder 2 (Step-2) | Testing on Encoder 3 (Step-2) | Testing on Encoder 4 (Step-2) | Testing on given two-side model |
| --- | --- | --- | --- | --- | --- |
| VIVO | 0.0289 | 0.1810 | 0.1434 | 0.5282 | 0.0447 |
| Nokia | 0.2286 | 0.2609 | 0.2670 | 0.5204 | 0.4942 |
| Samsung | 0.5698 | 0.2569 | 0.2567 | 0.5430 | 0.5374 |
| CATT | 0.2325 | 0.1643 | 0.0764 | 0.5314 | 0.5517 |

Table 4. Result for comparison of testing on low complexity encoders by eigenvectors(subspace) (Samsung’s decoder).

| Category | Testing on Encoder 1 (Step-2) | Testing on Encoder 2 (Step-2) | Testing on Encoder 3 (Step-2) | Testing on Encoder 4 (Step-2) | Testing on given two-side model |
| --- | --- | --- | --- | --- | --- |
| VIVO | 0.0786 | 0.1835 | 0.1571 | 0.5578 | 0.0447 |
| Nokia | 0.2253 | 0.2751 | 0.2658 | 0.5219 | 0.4942 |
| Samsung | 0.5660 | 0.2613 | 0.2572 | 0.5528 | 0.5374 |
| CATT | 0.2519 | 0.1842 | 0.0955 | 0.5333 | 0.5517 |

s

Observation 1: Subspace pre‑processing yields small, repeatable gains on weaker encoder–decoder pairs while preserving the overall ranking among low‑complexity encoders (E4 > E2 > E3 under the short training budget).

Observation 2:  The low-rank reconstruction projects each sample onto its principal subspace and suppresses high-order or ill-conditioned components, effectively denoising the input. This helps weak encoder-decoder pairs most, and improvement is modest because the dominant bottleneck remains the decoder compatibility, not the input representation itself.

Observation 3:  Subspace preprocessing is essentially a near-lossless reparameterization (energy preserved in top singular directions), it does not change the encoders’ inherent capacity under the fixed training budget.

- Discussion

In accordance with the Topic‑#2 Way‑Forward captured at RAN4#116, we keep the agreed test‑decoder architecture and the SGCS‑4 training recipe fixed so that any observed changes are attributable solely to the data representation, which is the specific factor the meeting asked us to isolate. Building on KTL’s observation that Step‑1 outcomes depend on the chosen anchor and that an encoder‑agreement term in the composite loss can mitigate this, we will report the anchor‑spread (defined as max–min SGCS across anchors) also when using the eigenvector‑based subspace dataset to check whether the pre‑processing impacts fairness under exactly the same SGCS‑4 procedure.

Because endpoint values can reorder under longer training budgets, we follow the #116 agreements and report SGCS versus epochs to establish learning‑curve stability rather than relying on single endpoints; as good practice (not an agreement) we will average over multiple random seeds (e.g., ≥3) and report mean and variance for both Step‑1 and Step‑2. This addresses the budget‑induced ranking artifacts already highlighted for low‑complexity encoders.

Consistent with the #116 summary and the KTL[2] TDoc, we pair all subspace results with a dataset‑alignment check (e.g., PSE), noting that aligned train/test conditions typically yield only few‑percent variations, whereas misalignment can produce tens‑of‑percent swings; we therefore disclose alignment alongside results. To avoid ambiguity, we label and separate matched‑representation evaluations (Row  Row, Eigen  Eigen) from cross‑representation checks (Row Eigen, EigenRow) so that the impact of representation alignment is transparent.

Finally, while the minutes recognize that the benefits of N‑to‑1 training are conditional, and Ericsson reported mixed outcomes depending on decoder complexity (gains with the agreed high‑complexity decoder but degradations under a low‑complexity decoder), our present study does not alter SGCS‑4 or decoder training at all; it changes only the input representation via subspace pre‑processing. This provides a conservative, standards‑friendly way to probe compatibility improvements without touching the training recipe, and it naturally motivates the Rel‑20 WI work on datasets and generalization. In particular, given that the Rel‑19 mixed dataset is centered on UMa with 32‑Tx and may not be sufficient to ensure broad generality, it is technically consistent to move from this discussion to a proposal on curating new datasets for the WI.

Proposal 1: RAN4 is encouraged to introduce (Rel-20) WI training/validation datasets covering bands, environments, mobility, antenna setups, and SNR ranges.

Proposal 2: RAN4 is encouraged to define reporting with clear train/validation/test splits and dataset-alignment metrics (e.g., PSE) in all submissions. (Rel-20) WI

Proposal 3: RAN4 is encouraged to accept raw-channel and/or eigenvector latent data (with formats) as valid dataset representations. (Rel-20) WI

- Conclusion

Observation 1: Subspace pre‑processing yields small, repeatable gains on weaker encoder–decoder pairs while preserving the overall ranking among low‑complexity encoders (E4 > E2 > E3 under the short training budget).

Observation 2:  The low-rank reconstruction projects each sample onto its principal subspace and suppresses high-order or ill-conditioned components, effectively denoising the input. This helps weak encoder-decoder pairs most, and improvement is modest because the dominant bottleneck remains the decoder compatibility, not the input representation itself.

Observation 3:  Subspace preprocessing is essentially a near-lossless reparameterization (energy preserved in top singular directions), it does not change the encoders’ inherent capacity under the fixed training budget.

Proposal 1: RAN4 is encouraged to introduce (Rel-20) WI training/validation datasets covering bands, environments, mobility, antenna setups, and SNR ranges.

Proposal 2: RAN4 is encouraged to define reporting with clear train/validation/test splits and dataset-alignment metrics (e.g., PSE) in all submissions. (Rel-20) WI

Proposal 3: RAN4 is encouraged to accept raw-channel and/or eigenvector latent data (with formats) as valid dataset representations. (Rel-20) WI

- Reference

[1] Ad-hoc summary for [116][132]NR_AIML_air_part2, R4-2511893

[2] RAN4 #116, Bengaluru, India, R4-2510184, “CSI compression on Low‑Complexity Encoders under SGCS‑4”

5. Contact points

- Kangyu Gao(gaokangyu@hanyang.ac.kr); Zhaokun Shao(shaozhaokun@hanyang.ac.kr);  Sang-Woon Jeon (sangwoonjeon@hanyang.ac.kr)-Hanyang University; Jae-suk Lee(jslee@ktl.re.kr); Hoon-Geun Song(hgsong@ktl.re.kr)- KTL