3GPP TSG RAN WG4 Meeting #116-bis	R4-2514082

Prague, Czech, 13 – 17 October, 2025

Agenda item:			7.8.4

Source:	vivo

Title:	Discussion on LB CA via switching enhancement

Document for:	Discussion

# Introduction

In the RAN#109 meeting, a new WID [1] on LB CA via switching enhancement was approved. The core part objectives for the WI are copied below.

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

In this contribution, we provide our views on RRM impacts on LA CA via switching enhancement.

# Discussion

The main difference between Rel-19 and Rel-20 LB-CA via switching is the scenarios to be supported. There are 3 cases being considered in 3GPP.

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

Switching between Case 1 and Case 2 were supported in Rel-19. It is to standardization switching between Case 1 and Case 3 in Rel-20. For case 3, dual downlink of FDD + SDL are supported but there is no uplink for both downlinks. The uplink transmission is only possible when UE switches to Case 1.

The LB CA via switching for Rel-19 and Rel-20 are illustrated in Fig 1 and Fig 2, respectively.

Fig. 1 LB-CA via switching in Rel-19

Fig. 2 LB-CA via switching in Rel-20

Deployment constraints are also important in terms of RRM impact. It is summarized below for both Rel-19 and Rel-20.

| Rel-19 | Rel-20 |
| --- | --- |
| - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers | - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected |

It can be seen the only difference is that there is no in-gap interference between DL carriers. With this assumption, there will be no extra RRM impact due to deployments constraints compared to Rel-19.

Observation 1: The main difference between Case 2 and Case 3 is that dual DL is supported in Case 3.

Regarding the switching pattern, it is to reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching according to the WID. The switching pattern in Rel-19 is a 40bits bitmap and its field description is as below.

switchingPattern Indicates which carrier is configured for the corresponding slot for low NR band carrier aggregation via switching between FDD and supplementary downlink (SDL) carrier. ‘0’ indicates FDD carrier (PCell), and ‘1’ indicates SDL carrier (SCell). The pattern starts from the beginning of SFN 0 of PCell.

It can be seen this bit map configuration cannot be directly used for switching between Case 1 and Case 3. A simple approach is to introduce a new UE capability in Rel-20 and reuse the field description of Rel-19 switching pattern with changing the meaning of ‘1’ indication to that it indicates both FDD downlink carrier (PCell DL) and SDL carrier (SCell).

Proposal 1: LS to RAN1 and RAN2 on switching pattern to support switching between case 1 and case 3.

## PCell related requirements impact

For PCell operation, downlink is always available when switching between case 1 and case 3 as illustrated in Fig 2. Therefore, there should be no impact to legacy PCell operation. The requirements for PCell operation under Rel-19 LB-CA via switching is not appliable anymore. On the other hand, legacy requirements should be applicable.

The impacted requirements in Rel-19 including RLM, BFD/CBD, L1-RSRP, L1-SINR, PL-RS, TCI state switching and intra-frequency measurement without gap. For UE supporting switching between Case 1 and Case 3, application of legacy requirements should be clarified.

Proposal 2: For UE supporting switching between Case 1 and Case 3, legacy requirements for RLM, BFD/CBD, L1-RSRP, L1-SINR, PL-RS, TCI state switching and intra-frequency measurement without gap should be applicable for PCell operation.

## SCell related requirements impact

For SDL SCell operation, downlink is not always available when switching between case 1 and case 3 as illustrated in Fig 2. Comparing Fig 1 and Fig 2, there is no difference between Rel-19 switching and Rel-20 switching, Therefore, the requirements for SDL SCell operation under Rel-19 LB-CA via switching is still appliable in Rel-20.

The impacted requirements in Rel-19 including SCell activation/deactivation, BFD/CBD, L1-RSRP, L1-SINR, TCI state switching and intra-frequency measurement without gap. For UE supporting switching between Case 1 and Case 3, application of Rel-19 LB-CA via switching requirements should be clarified.

Proposal 3: For UE supporting switching between Case 1 and Case 3, Rel-19 requirements for LB-CA via switching for BFD/CBD, L1-RSRP, L1-SINR, TCI state switching and intra-frequency measurement without gap should be applicable for SDL SCell operation.

# Summary

In this contribution, we further provided views on provide our views on specification improvement and RAN4 operation related aspects. Following proposals are provided.

Observation 1: The main difference between Case 2 and Case 3 is that dual DL is supported in Case 3.

Proposal 1: LS to RAN1 and RAN2 on switching pattern to support switching between case 1 and case 3.

Proposal 2: For UE supporting switching between Case 1 and Case 3, legacy requirements for RLM, BFD/CBD, L1-RSRP, L1-SINR, PL-RS, TCI state switching and intra-frequency measurement without gap should be applicable for PCell operation.

Proposal 3: For UE supporting switching between Case 1 and Case 3, Rel-19 requirements for LB-CA via switching for BFD/CBD, L1-RSRP, L1-SINR, TCI state switching and intra-frequency measurement without gap should be applicable for SDL SCell operation.

# References

- RP‑252906	Moderator's summary for other RAN4 REL-20 5G-Adv topics, TELUS