# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514097

# Prague, Czech, 13th – 17th October, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for less than 5MHz in NTN

Agenda item:	6.6.2.1

Document for:	Approval

Introduction

RRM core requirements for Rel-19 NTN enhancements are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. We can see the main task on RRM performance part for less than 5MHz NTN as follows:

Less than 5MHz for NTN - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

In this paper we will provide our views on RRM performance requirements on less than 5MHz NTN.

Discussion

In last meeting, we have already confirmed the measurement accuracy requirements and common testing configuration. For test cases, there are still some remaining issues to be discussed in following table.

| Test case | Requirements | Configurations | Core requirements to be defined |
| --- | --- | --- | --- |
| Common configuration |  | 15KHz, 3MHz BW, 12 PRB SSB, GSO/NGSO The SSB configuration for less than 5MHz can be used (SSB.13 FR1). |  |
| HO-1: | 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | unknown cell | Define HO delay requirements, which are taken extension of delay for TN less than 5MHz as baseline. TΔ is extended to [3Trs] |
|  |  | Intra-frequency |  |
|  |  | Same testing procedure as NTN can be reused |  |
|  |  | FFS: Other aspects |  |
| CHO-1 | 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover or 6.1C.2.3 NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurements | With L3 measurement | - for CHO with L3 measurements: For CHO with L3 measurement, when Tmeasure is extended with [3] for intra-frequency case, T∆ in Tinterrupt can be extended to 2 Trs. - for CHO without L3 measurements: T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] |
|  |  | Time based |  |
|  |  | Unknown target cell |  |
|  |  | Intra-frequency |  |
|  |  | Other aspects |  |
| SatSwitch-1 | 6.1C.3 NR SAN Satellite switching with re-synchronization | Hard switch | Same as HO requirements |
|  |  | Unknown target cell |  |
|  |  | RACH based |  |
| RLM-1 | 8.1C Radio Link Monitoring for Satellite Access | RLM OOS | TN less than 5MHz is reused |
|  |  | FR1 PCell only |  |
|  |  | non-DRX |  |
|  |  | Other aspects |  |
| RLM-2 | 8.1C Radio Link Monitoring for Satellite Access | RLM IS | TN less than 5MHz is reused |
|  |  | FR1 PCell only |  |
|  |  | DRX |  |
| BFR-1 | 8.5C Link Recovery Procedures for Satellite Access | SSB based BFD | TN less than 5MHz is reused |
|  |  | FR1 PCell only |  |
|  |  | non-DRX |  |
|  |  | Other aspects |  |
| MR-1 | 9.2C NR intra-frequency measurements for SAN | with SSB index detection | The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended by 4 and 3 for intra-frequency (Es/Iot≥-6 dB) and inter-frequency (Es/Iot≥-4 dB) respectively. Same as legacy. |
|  |  | Non-DRX |  |
|  |  | No gap |  |
|  |  | Other aspects |  |
| MR-2 | 9.3C NR inter-frequency measurements for SAN | With SSB index detection | The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended by 4 and 3 for intra-frequency (Es/Iot≥-6 dB) and inter-frequency (Es/Iot≥-4 dB) respectively. Same as legacy. |
|  |  | DRX |  |
|  |  | gap |  |
|  |  | Other aspects |  |
| IDLE-1 | 4.2C Cell Re-selection for NR UE for Satellite Access | - FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth | No RRM requirements impact |

In this paper, we would like to provide the views from our side to remaining issues based on the above table.

we have agreed the part of test cases configuration, the remaining configuration shall be discussed in this meeting based on above table.

For test case on handover, some test parameters such as BW channel, PDSCH Reference measurement channel and CORESET Reference channel, we found that these parameters shall be confirmed based on legacy less than 5M in TN network and legacy NTN test parameters:

- PDSCH Reference measurement channel: In TN, the type of this parameter is SR.1.3 for both 15PRB CBW and 12PRB CBW. Actually, we have no idea whether SR.1.2 shall be used for 12 PRB CBW based on clause A3.1.1. From our understanding, SR.1.3 FDD shall be considered.

- CORESET Reference Channel: CR.1.2 FDD shall be confirmed.

- SSB Configuration: In the last meeting, RAN4 has reached one agreement that the SSB configuration shall be SSB.13 FR1.

- BWP: We confirm 12PRB.

Proposal 1: The following table shall be captured as part of the test case:

| PDSCH Reference measurement channel |  |  | SR.1.3 FDD |  |
| --- | --- | --- | --- | --- |
| CORESET Reference Channel |  |  | CR.1.2 FDD |  |
| TRS configuration |  |  | TRS.1.1 FDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC Configuration |  |  | SMTC.1 |  |
| SSB Configuration |  |  | SSB.13 FR1 |  |
| BWPchannel |  | MHz | 3: NPRB,c = 12 | 3: NPRB,c = 12 |
| BWP BW |  | MHz | 3: NPRB,c = 12 | 3: NPRB,c = 12 |

In core part, we have defined the handover delay which includes RRC processing delay and interruption. For test requirements, the RRC procedure delay equals to 10ms, the formulation of interruption is as below:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms,

Where, Tsearch = 20ms; TIU = 20 ms; Tprocessing = 20 ms; T∆ = 60 ms; Tmargin = 2 ms.

Thus, the UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T2.

Proposal 2: The UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T2.

For satellite switch in NTN, test cases are defined for hard satellite switch and soft satellite switch. The legacy UE capability has been defined as below:

softSatelliteSwitchResyncNTN-r18 Indicates whether UE supports soft satellite switch with re-sync, as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate support of hardSatelliteSwitchResyncNTN-r18.

Based on above UE capability, UE supporting soft switch will also support hard satellite switch. Consequently, we prefer to consider soft satellite switch which can cover hard satellite switch. If only hard switch is tested, how do we test the duration t-service-t-seviceStart. From our perspective, soft satellite shall be tested.

Observation 1:  UE supporting soft switch will also support hard satellite switch.

Proposal 3: For satellite switch in NTN, soft satellite switch shall be tested.

For cell re-selection, although there is no core part impact on cell re-selection requirements, regarding there is no exact Tsi-nr given in current spec, only the testing parameter will be given. Thus, we deem that the testing parameter shall be defined. In legacy TN less than 5M WI, the reason to define new test case for cell re-selection is that UE needs to read SIB/MIB from the target cell, which is same in HO. RAN4 did not define new core requirements because the exact delay for UE to read SIB/MIB is included in Tsi but the exact value is not specified. In the test, Tsi is defined as 1280ms as below:

Legacy TN test requirements for Tsi: TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

Since the legacy delay as 1280ms is not enough to accommodate longer delay for SIB/MIB reading due to punctured PBCH, thus, the legacy test requirements on less than 5M TN can be seen as below:

Legacy TN with less than 5M for Tsi: TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

We observe that the exact value in test requirements for legacy NTN on Tsi is as below:

Legacy TN test requirements for Tsi: TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20 ms period and 80 ms period, respectively.

The same logic shall be considered when defining cell re-selection test requirements with less than 5M in NTN scenario.

Observation 2: The legacy test requirements without less than 5MHz in TN scenario, the legacy delay as 1280ms is not enough to accommodate longer delay for SIB/MIB reading due to punctured PBCH.

Observation 3: The legacy test requirements with less than 5MHz in TN scenario, TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

Proposal 4: The concrete test requirements could be seen as below:

TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20 ms period and 80 ms period, respectively.

Conclusion

In this paper we provided our views on RRM performance requirements on R19 NR less than 5M NTN enhancement.

Proposal 1: The following table shall be captured as part of the test case:

| PDSCH Reference measurement channel |  |  | SR.1.3 FDD |  |
| --- | --- | --- | --- | --- |
| CORESET Reference Channel |  |  | CR.1.2 FDD |  |
| TRS configuration |  |  | TRS.1.1 FDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC Configuration |  |  | SMTC.1 |  |
| SSB Configuration |  |  | SSB.13 FR1 |  |
| BWPchannel |  | MHz | 3: NPRB,c = 12 | 3: NPRB,c = 12 |
| BWP BW |  | MHz | 3: NPRB,c = 12 | 3: NPRB,c = 12 |

Proposal 2: The UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T2.

Observation 1:  UE supporting soft switch will also support hard satellite switch.

Proposal 3: For satellite switch in NTN, soft satellite switch shall be tested.

Observation 2: The legacy test requirements without less than 5MHz in TN scenario, the legacy delay as 1280ms is not enough to accommodate longer delay for SIB/MIB reading due to punctured PBCH.

Observation 3: The legacy test requirements with less than 5MHz in TN scenario, TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

Proposal 4: The concrete test requirements could be seen as below:

TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20 ms period and 80 ms period, respectively.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133

- R4-2508261, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi.