3GPP TSG-RAN WG4 Meeting #116bis                                        R4-2514132 Prague, Czech Republic, October 13 - 17, 2025

Source: 	Huawei, HiSilicon

Title: 	On BS conformance for new requirements for SBFD

Agenda Item:	6.13.2.1

Document for:	Approval

Introduction

During RAN4#116, more analysis on SBFD conformance testing are presented. Discussion progress can be found in WF [1].

##### Issue 1: System parameters of conformance testing Agreement: - For system parameters of conformance testing: - The operating bands supported by SBFD need to be added to TS 38.141-1 and TS 38.141-2. - The channel bandwidths of SBFD capable BS and SBFD subband configuration(s) per supported channel bandwidth need to be added to TS 38.141-1 and TS 38.141-2. ##### Issue 2: BS type for SBFD conformance testing Agreement: - For BS type for SBFD conformance testing: - The general principle that SBFD doesn’t support type 1-C needs to be captured in TS 38.141-1. ##### Issue 3: Declaration of supported SBFD configuration Agreement: - Declaration of supported SBFD configuration: - At least the following declarations shall be captured in TS 38.141-1/-2: - TX power for SBFD; - Bands supported for SBFD; - Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; - SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands. - Other declarations are not precluded ##### Issue 4: Testing applicability for supported SBFD configuration Way forward: - Further study on the following proposals on test configuration: - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required. For the restriction on guardband size of SBFD - Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations. ##### Issue 5: RF channels to be tested for SBFD operation Way forward: - Further study on the following proposals on RF channels to be tested for SBFD operation: - When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band. ##### Issue 6: Conformance testing related to transient period between SBFD and non-SBFD Way forward: - Further study on the following proposals on conformance test for transient period between SBFD and non-SBFD: - For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD. ##### Issue 7: Conformance testing related to in-channel adjacent subband leakage Way forward: - Further study on the following proposals on conformance test for in-channel adjacent subband leakage: - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges. - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2. - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2. ##### Issue 8: Conformance testing related to in-channel adjacent subband blocking Way forward: - Further study on the following proposals on conformance test for in-channel adjacent subband blocking: - For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on. - For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement. - For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.

In this contribution, we would like to share our views regarding this topic.

Discussion

Transient period between SBFD and non-SBFD

According to the WF, whether new test model is needed should be studied for the transient period between SBFD and non-SBFD. From our understanding, the existing TDD configuration can be used as baseline and necessary time-domain SBFD configuration should be included. The existing test procedure for transmitter transient period i.e., clause 6.4.2 in TS 38.141 can be used as starting point as well.

Table 4.9.2.2-1: Configurations of TDD for BS type 1-C and BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

Proposal 1: For the transition period between SBFD and non-SBFD symbols, reuse the existing TDD testing configuration and the existing test procedure for transmitter transient period as baseline. Necessary SBFD flavour should be added into testing configuration e.g. SBFD time domain configuration.

In-channel adjacent subband leakage

For this new RF requirement for SBFD, the existing test model should be updated since the DL signal is no longer transmitted in the whole carrier. Specifically, the number of RBs allocated for PDSCH should be the RB number assigned for DL subband i.e. NRB, SBFD_DL for DU/UD case and NRB, SBFD_DL_1, NRB, SBFD_DL_2 for DUD case.

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-C and BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

Proposal 2: For the In-channel adjacent subband leakage, reuse the test model NR-FR1-TM1.1 and NR-FR2-TM1.1 and change the NRB as NRB, SBFD_DL for DU/UD case and NRB, SBFD_DL_1, NRB, SBFD_DL_2 for DUD case. The test procedure for absolute ACLR limit can be used as baseline.

In-channel adjacent subband blocking

Unlike the above new RF requirement for SBFD, we think the difference between it and the legacy in-band blocking is not that much, so we propose to use the existing test procedure for in-band blocking as basis and check if modification for SBFD is needed.

Proposal 3: For the In-channel adjacent subband blocking, use the existing test procedure for in-band blocking as baseline.

Conclusion

In this contribution we discussed on the conformance testing for new requirements for SBFD. We have the following proposals:

Proposal 1: For the transition period between SBFD and non-SBFD symbols, reuse the existing TDD testing configuration and the existing test procedure for transmitter transient period as baseline. Necessary SBFD flavour should be added into testing configuration e.g. SBFD time domain configuration.

Proposal 2: For the In-channel adjacent subband leakage, reuse the test model NR-FR1-TM1.1 and NR-FR2-TM1.1 and change the NRB as NRB, SBFD_DL for DU/UD case and NRB, SBFD_DL_1, NRB, SBFD_DL_2 for DUD case. The test procedure for absolute ACLR limit can be used as baseline.

Proposal 3: For the In-channel adjacent subband blocking, use the existing test procedure for in-band blocking as baseline.

- References

- R4-2512665, “Way Forward for [116][306] NR_duplex_evo_General”, Samsung, CATT, Charter, CMCC, Ericsson, Huawei, HiSilicon, Nokia, Qualcomm, Tejas Network, ZTE, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.

- R4-2511255, “CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS”, Samsung, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.