# 3GPP TSG-RAN WG4 Meeting #116bis                                R4-2514149

# Prague, CZ, 13rd – 17th, Oct. 2025

# Title: 	Discussion on inter-vendor training collaboration and interoperability for two-sided models

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.9.3

# Document for:	Approval

# 1 Introduction

In RAN #108 meeting, the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In the following section, we will provide the initial discussions for inter-vendor training collaboration and interoperability for two-side models.

# 2 Discussion

In the Rel-19, RAN4 discussed the testability issues for two side model, and achieve the following agreement in the last meeting as guidance for the WI discussion for two-side model.

Based on the discussion of Rel-19, RAN4 verified the feasibility of option 3 track 2 with full specified test decoder, and option 4 with partial knowledge of test decoder. However, based on the WID [1] mentioned that only the fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration. Therefore, RAN4 only needs to consider the full specified test decoder for Rel-20.

Model structure scalability

As illustrated in the above of WID, the responsibility of RAN4 requires feedback of the Direction C, Direction A, sub-option 3a-1 in RAN 110 meeting. Regarding Direction C, which is defined in RAN1 with fully specified reference model, structure and parameters, and Option 3a-1 is based on the only specified the model structure. Considering the limited time for the check point, RAN4 does not need to discuss the more detailed model structures and the scalability issues. RAN4 only needs to make sure that the model structure scalability issues will be considered in the Rel-20 requirements definition. The detailed solutions for scalability can be discussed later according to the work plan of WID.

Observation 1. Considering the limited timeline for checkpoint, RAN4 needs to confirm that the scalability will be fully consider in the Rel-20, and the related solutions can be discussed later.

Proposal 1. Based on the agreements of RAN1/RAN4, RAN4 can fully consider the model structure scalability in Rel-20 requirements definition.

Base on the discussion of the last meeting [2], the following topics need to be discussed in the upcoming meeting.

The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and align on the dataset assumptions and dataset - Discuss and align on assumptions for the test decoder and reference encoder structures - Also may need to consider how many test decoders and encoders - Discuss and align on the scalability alternatives - Discuss and agree on expectations on whether the decoder/encoder for RAN4 are the same as for option C - Discuss whether to decide this before or after agreeing the model - Develop a means to capture model structures and model parameters in 3GPP - Discuss and agree the test metric for the performance requirements. - Discuss and agree channel model for the requirements. - For the dataset, discuss the following: - Eigenvectors or raw channel (or both) ? - What it is important to capture within the dataset - metrics for checking alignment - For example,. PSE - Consider examples from SI datasets that did not align

Dataset assumptions and dataset

In the Rel-19 study, the training dataset was based on system-level simulations to verify whether the SGCS could be aligned. From the perspective of requirements definition, RAN4 always use TDL channel mode as testing environment to define requirement in legacy procedure. RAN4 needs to discuss or evaluate whether the mode based on CDL channel model can be used for TDL channel model to check the test metric.

Observation 2. Currently in Rel-19, the dataset assumption based on the CDL channel model. However, the requirements definition of legacy procedure based the TDL channel model.

Proposal 2. RAN4 needs to discuss whether the dataset based on the CDL channel can be used for TDL channel model.

Reference model

In Rel-19, RAN4 at least studied the performance of CNN, transformer, simplified CNN, and MLP model structures, as shown in the following table.

Table 1. Different encoder structures with complexity

| Different encoder backbones | Flops |
| --- | --- |
| Encoder-1 (CNN) | 47M |
| Encoder-2 (Transformer) | 3.63M |
| Encoder-3 (CNN) | 4.14M |
| Encoder-4 (MLP) | 0.22M |

Also based on the performance evaluation of SGCS, the transformer model has the best performance and the MLP has the worst performance. On the other hand, the transformer is also considered a reference model for studying performance and scalability, based on the RAN1 discussion and evaluation. Therefore, the transformer structure can be used as a reference encoder in Rel-20 at least.

Proposal 3. The transformer structure can be considered as a reference encoder in Rel-20 at least.

Model structure scalability

Regarding the aspects of model structure scalability, the following alternatives were agreed in RAN1.

- For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters

Based on the evaluation of RAN1, the feasibility of the above alternatives has been confirmed for direction A 3a-1 and direction C. Thus, some redundant discussions can be avoided in RAN4, especially some simulation works.

Proposal 4. RAN4 needs to discuss and pick up the easiest alternatives to implement.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for AI based CSI compression, the conclusions are:

Observation 1. Considering the limited timeline for checkpoint, RAN4 needs to confirm that the scalability will be fully consider in the Rel-20, and the related solutions can be discussed later.

Observation 2. Currently in Rel-19, the dataset assumption based on the CDL channel model. However, the requirements definition of legacy procedure based the TDL channel model.

Observation 3. The feasibility of scalability alternatives has been confirmed for direction A 3a-1 and direction C in RAN1.

Proposal 1. Based on the agreements of RAN1/RAN4, RAN4 can fully consider the model structure scalability in Rel-20 requirements definition.

Proposal 2. RAN4 needs to discuss whether the dataset based on the CDL channel can be used for TDL channel model.

Proposal 3. The transformer structure can be considered as a reference encoder in Rel-20 at least.

Proposal 4. RAN4 needs to discuss and pick up the easiest alternatives to implement.

# 4 References

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements.

- R4-2511849, WF on [116] [132] NR_AIML_air_part2, Ericsson.