3GPP RAN4 Meeting #116 bis		R4-2514154

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:	7.11.2

Source:	Ericsson

Title:	Discussion on RF requirements for A-IoT device

WI/SI:	FS_Ambient_IoT_Outdoor

Release:	Rel-20

Document for:	Approval

# Introduction

During RAN #108 a new SID [1] on enhancements for solutions for A-IoT in NR outdoor for active devices was agreed. This contribution provides our views related to the RF requirements for active A-IoT devices.

# Discussion

In the SID the following A-IoT devices are considered:

The overall objectives of the SI are the following:

And the RAN4 led objectives are the following:

As it can be seen, RAN4 work is mainly focused on doing coexistence study between A-IoT and NR/LTE. However, the outcome of the coexistence study is highly dependent on the coexistence simulation assumptions including device output power, device sensitivity/SNR, device ACS and device transmission/ reception bandwidth.

## Output power

In RAN1-led objective, the SID considers that device 2b has a power in the range -20dBm to -10dBm and device C in the range -3dBm and +5dBm. The device output power will have an impact on the coexistence study outcome in A-IoT UL scenario in two cases; A-IoT device is the interferer and NR/LTE BS is the victim, and NR/LTE device is the interferer and A-IoT reader is the victim. The higher the device output power, the wors the interference from A-IoT device to NR/LTE BS. On the other hand, the higher the A-IoT device output power, the better the SNR at the A-IoT reader. The sensible choice is to perform the coexistence investigation for both ends of the power range for both device type. It should be noted that while A-IoT devices have significantly lower power compared to NR UEs, A-IoT can still pose a coexistence problem to NR in some cases, e.g. there are many A-IoT devices, an A-IoT device is significantly closer to NR BS compared to a NR UE, etc.

Observation 1. The high-end of the device out power represents the bottleneck in the case (A-IoT device > NR/LTE BS), while the low-end of the device output power represents the buttle neck in the case (NR/ LTE UE > A-IoT reader). The sensible choice is to perform the coexistence investigation for both ends of the power range for both device types.

Proposal 1. Perform co-existence study considering an output power of -20dBm and -10dBm for device 2b and -3dBm and 5dBm for device C

## Sensitivity/ SNR

The coexistence study outcome in the A-IoT DL scenario (NR/LTE BS is the interferer and is A-IoT device is the victim) will depend on the device required SNR. RAN1 will perform Link Level Simulations (LLS), RAN4 can use the SNR value corresponding to 10%BLER in the LLS for coexistence study.

Observation 2. RAN1 will perform Link Level Simulations (LLS), RAN4 can use the SNR value corresponding to 10% BLER in the LLS for coexistence study.

Proposal 2. Use the SNR value corresponding to 10% BLER from RAN1 LLS to perform co-existence study

## Interference blocking capabilities

As it can be seen from the device types above, both device 2b and C use IF Envelope Detector (ED) receivers compared to device 1 that uses RF ED.

An ED receiver is a very simple architecture (essentially just an RF tuned circuit feeding a diode + RC filter). Its main drawback is poor selectivity and poor strong-signal handling. Adding an IF mixer stage will turn the design into a superheterodyne ED:

- The RF is first mixed down to an intermediate frequency (IF).

- At IF, you can insert narrow, high-Q filters (ceramic, crystal, or LC) that are much more selective than what you could achieve at RF.

- This filtering dramatically improves in-band blocking performance, because strong adjacent or off-channel interferers can be rejected before they overload the detector.

The IF receiver interference blocking will depend on the following:

- Mixer nonlinearity: If the mixer is poor quality or driven too hard, strong out-of-band signals can still create intermodulation products that fall in-band. The blocking performance then depends on mixer linearity as well as IF filtering.

- IF choice: Lower IF makes envelope detection easier but filtering harder; higher IF allows sharper filters but complicates detection.

Observation 3. Both device 2b and C use IF ED receiver that provide in-band selectivity, so in-band selectivity and ACS requirements for both devices can be discussed

Observation 4. Both device 2b and C may also use an RF BPF, so out of band blocking can also be discussed

Proposal 3. In-band selectivity, ACS and out of band blocking for device 2b and C should be discussed as part of coexistence study

## Device transmission/ reception bandwidth

A-IoT device has a transmission bandwidth of 1 PRB to a couple of PRBs. Since device 2b and C have IF receivers, they can also be assumed to have receiver bandwidth matched with the reader transmission bandwidth (i.e. 1 PRB).

Proposal 4. Consider an A-IoT device with a transmission/ reception bandwidth of 1 PRB for the coexistence evaluation

# Conclusions

This contribution provides our views related to the RF requirements for active A-IoT devices. The contribution makes the following observations and proposals.

Observation 1. The high-end of the device out power represents the bottleneck in the case (A-IoT device > NR/LTE BS), while the low-end of the device output power represents the buttle neck in the case (NR/ LTE UE > A-IoT reader). The sensible choice is to perform the coexistence investigation for both ends of the power range for both device types.

Proposal 1. Perform co-existence study considering an output power of -20dBm and -10dBm for device 2b and -3dBm and 5dBm for device C

Observation 2. RAN1 will perform Link Level Simulations (LLS), RAN4 can use the SNR value corresponding to 10% BLER in the LLS for coexistence study.

Proposal 2. Use the SNR value corresponding to 10% BLER from RAN1 LLS to perform co-existence study

Observation 3. Both device 2b and C use IF ED receiver that provide in-band selectivity, so in-band selectivity and ACS requirements for both devices can be discussed

Observation 4. Both device 2b and C may also use an RF BPF, so out of band blocking can also be discussed

Proposal 3. In-band selectivity, ACS and out of band blocking for device 2b and C should be discussed as part of coexistence study

Proposal 4. Consider an A-IoT device with a transmission/ reception bandwidth of 1 PRB for the coexistence evaluation

# References

[1]	RP- 251840, “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, 3GPP RAN #108.