3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514161

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on performance requirements for NTN in Ku band

Source: 	Huawei, HiSilicon

Agenda item:	5.10.2

Document for:	Discussion

Introduction

RRM core requirements for NTN in Ku band are discussed in RAN4#116, and the outcomes are captured in WF [1]. Based on the work plan, RAN4 should start performance part of the WI.

In this paper, we will provide our views on performance requirements for NTN in Ku band.

Discussion

Ku bands include bands in both FR1-NTN and FR2-NTN. RAN4 needs to discuss how to test each of them.

FR2-NTN

For core part, it is agreed that the existing requirements defined for FR2-NTN (in R18 for Ka band) apply also for VSAT in Ku bands working with FR2 numerology. There are also 2 additional requirements:

- Satellite switch with re-sync

- UL timing for mobile VSAT served by NGSO

For RRM testing, as most RRM requirements are band agnostic it is straightforward that existing test cases defined for FR2-NTN (in R18 for Ka band) are re-used for Ku bands in FR2-NTN. This means RAN4 does not need to define new test cases for Ku bands in FR2-NTN except for the additional requirements. It is noted that measurement accuracy test cases may need to be updated because they are band specific.

For satellite switch with re-sync, RAN4 should define new test cases for FR2-NTN. FR1-NTN test cases can be used as baseline. For UL timing, we understand RAN4 only needs to add a new test configuration for mobile VSAT served by NGSO in Table A.14.3.1.2.1-1. The test setup and requirements in A.14.3.1.2 can be re-used. It is noted that the interval between epoch times in the test is 2.56s which meets the side condition for the UL timing requirements for mobile VSAT served by NGSO.

Table A.14.3.1.2.1-1: Supported test configurations for FR2-NTN PCell

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| 2 | NGSO, NR FDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| 3 | GSO, NR FDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| NOTE: For mobile VSAT UE type, it is only requied to pass test config 3. |  |

Proposal 1: For Ku bands in FR2-NTN, the existing test cases defined for FR2-NTN (in R18 for Ka band) are re-used, except the following

- Measurement accuracy test cases are updated to include the new bands

- New test cases are defined for satellite switch with re-sync

- UL timing test cases are updated to include test configuration for mobile VSAT served by NGSO

FR1-NTN

For core part, it is agreed that the by default existing requirements defined for FR2-NTN (in R18 for Ka band) apply also for VSAT in Ku bands working with FR1 numerology. However, the existing test cases defined for FR2-NTN (in R18 for Ka band) cannot be re-used for Ku bands in FR1-NTN because they are with different numerologies.

In our view, there are 2 options to test Ku bands in FR1-NTN:

- Option 1: define new test cases corresponding to those defined for FR2-NTN but with FR1 numerology. It is straightforward and clear, but requires large efforts in RAN4 as we need to repeat the same work for FR2-NTN in R18.

- Option 2: re-use existing test cases for FR1-NTN corresponding to those defined for FR2-NTN. It is possible because most of the RRM requirements are same for FR1-NTN and FR2-NTN, so the test setup and requirements can be re-used for testing VSAT. The exception is blind HO and satellite switch with re-sync, for which the test requirements can be defined separately for FR1-NTN UE and VSAT.

We slightly prefer option 2 to save the standardization efforts and to avoid redundancies in spec, but we are open to discuss e.g. if there exists some FR2-NTN test case without corresponding FR1-NTN test case.

Proposal 2: For Ku bands in FR1-NTN, RAN4 to discuss whether it is feasible to re-use existing test cases for FR1-NTN corresponding to those defined for FR2-NTN, except the following

- Test cases for blind HO and satellite switch with re-sync, for which the test requirements are updated to add requirements for VSAT in FR1-NTN.

Test applicability

It is straightforward that VSAT supporting only Ku bands in FR1-NTN needs to pass the test cases for FR1-NTN, and VSAT supporting only Ku bands in FR2-NTN needs to pass the test cases for FR2-NTN. It is unclear if there could exist VSAT supporting Ku bands in both FR1-NTN and FR2-NTN. If so, RAN4 needs to discuss whether this VSAT needs to pass 2 sets of test cases, or only needs to pass 1 set of test cases, either FR1-NTN or FR2-NTN.

Proposal 3: RAN4 to discuss the test applicability for VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, if such VSAT exists.

Conclusions

In this paper we provided our views on performance requirements for NTN in Ku band.

Proposal 1: For Ku bands in FR2-NTN, the existing test cases defined for FR2-NTN (in R18 for Ka band) are re-used, except the following

- Measurement accuracy test cases are updated to include the new bands

- New test cases are defined for satellite switch with re-sync

- UL timing test cases are updated to include test configuration for mobile VSAT served by NGSO

Proposal 2: For Ku bands in FR1-NTN, RAN4 to discuss whether it is feasible to re-use existing test cases for FR1-NTN corresponding to those defined for FR2-NTN, except the following

- Test cases for blind HO and satellite switch with re-sync, for which the test requirements are updated to add requirements for VSAT in FR1-NTN.

Proposal 3: RAN4 to discuss the test applicability for VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, if such VSAT exists.

Reference

- R4-2512121, WF on RRM requirements for NR_NTN_Ku_bands, Apple