3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514174

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on performance requirements for other enhancement in NTN

Source: 	Huawei, HiSilicon

Agenda item:	6.19.3.2

Document for:	Discussion

Introduction

RRM requirements for other enhancement in NR NTN are discussed in RAN4#116, and the outcomes are captured in WF [1]. As the core part of the WI is closed, RAN4 needs to work on the performance part for the objective of DL coverage enhancement, which has impacts to core requriements.

In this paper, we will provide our views on RRM performance requirements for DL coverage enhancement in NR NTN.

Discussion

For SSB periodicity extension, RAN4 has defined new RRC re-establishment requirements:

- When SMTC is not provided for an inter-frequency carrier, UE assumes the SMTC period is 160ms rather than 20ms in legacy.

- When serving cell Es/Iot is < -8dB, delay requirements are defined for blind search based on SMTC with period up to 160ms rather than 20ms in legacy.

In RAN4#114-bis, there was some discussion on whether to define test case for the new requirements. In R15, RAN4 has defined test case in clause A.6.3.2.1.3	for RRC Re-establishment without serving cell timing. In R17, this test case (without serving cell timing) was not defined for NTN. In our understanding, the reason is that RAN4 did not repeat all TN test cases for NTN. As the R19 requirements are dedicated for this case, we suggest RAN4 to define one test case for RRC Re-establishment with 160ms SSB.

Proposal 1: Define test case for RRC Re-establishment with 160ms SSB periodicity and without serving cell timing based on existing test case in A.6.3.2.1.3.

Besides, RAN4 has agreed to support SMTC enhancements introduced by RAN2.

On supporting up to 7 SMTCs per carrier in IDLE/INACTIVE mode, RAN4 is still discussing whether and how to define requirements in core part maintenance. In our view, if RAN4 agrees to define how UE selects SMTC when the configured number of SMTCs exceeds its capability, then RAN4 can consider to define a test case for cell reselection where more than 4 SMTCs are configured, and we verify whether UE correctly selects the SMTC and perform measurement in the selected SMTC.

Proposal 2: Wait for conclusion on core part discussion for supporting up to 7 SMTCs per carrier in IDLE/INACTIVE mode before discussing whether to define new test cases on cell reselection.

On supporting 2 SMTC periodicities with different offsets per carrier, RAN4 has agreed on several impacts to the core requirements. In our view, RAN4 should define L3 measurement (event triggered report) test case for this enhancement. Although there are also impacts to other requirements like HO, the impacts are due to use of MO configuration in those procedures, so the key procedure that should be tested is L3 measurement. Also, the enhancement will impact cell reselection requirements, but as the cell reselection measurement is based on DRX cycle and not sensitive to SMTC periodicity, the value to test cell reselection is not high. The requirements in CONNECTED is anyway much more stringent.

Proposal 3: Define test cases for L3 event triggered reporting for supporting 2 SMTC periodicities with different offsets per carrier.

Conclusions

In this paper we provided our views on RRM performance requirements for DL coverage enhancement in NR NTN.

Proposal 1: Define test case for RRC Re-establishment with 160ms SSB periodicity and without serving cell timing based on existing test case in A.6.3.2.1.3.

Proposal 2: Wait for conclusion on core part discussion for supporting up to 7 SMTCs per carrier in IDLE/INACTIVE mode before discussing whether to define new test cases on cell reselection.

Proposal 3: Define test cases for L3 event triggered reporting for supporting 2 SMTC periodicities with different offsets per carrier.

Reference

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)