3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514182

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on performance requirements for IoT NTN TDD

Source: 	Huawei, HiSilicon

Agenda item:	6.21.2

Document for:	Discussion

Introduction

RRM core requirements for Rel-19 IoT NTN TDD are discussed in RAN4#116, and the outcomes are captured in WF [1]. Based on the work plan, RAN4 should start the performance part of the WI.

In this paper, we will provide our views on RRM performance requirements for IoT NTN TDD.

Discussion

It is clear that RAN4 needs to define accuracy requirements for NRSRP/Q and channel quality report for IoT NTN TDD. In our view, the existing requirements for NB-IoT NTN can be re-used except the side condition. Based on the agreement in core part, in this WI the target SNR is -1dB, and number of measurement samples is determined to achieve similar accuracy as in existing requirements. In addition, accuracy is not impacted by the interval between measurement samples.

Proposal 1: RAN4 to define accuracy requirements for NRSRP/Q and channel quality report for IoT NTN TDD by re-using the existing requirements for NB-IoT NTN, except the side condition.

For IoT NTN, RAN4 also defined requirements for PHR reporting, including the evaluation period (1 slot with NPUSCH) and the reporting delay (0ms). In our view, those requirements can be directly re-used for IoT NTN TDD.

Proposal 2: RAN4 to define PHR requirements for IoT NTN TDD by re-using the existing requirements for NB-IoT NTN.

RAN4 also needs to define RRM test cases for IoT NTN TDD. Considering that a UE may support only band 249, this UE will not be tested against existing test cases for NB-IoT NTN, so it is better to make sure the test cases defined for IoT NTN TDD cover the same requirements as NB-IoT NTN.

On the other hand, the WI focuses on quite specific deployment scenario, so some existing test cases for NB-IoT NTN are not applicable, e.g. those defined for enhanced coverage, time triggered measurement relaxed measurement. Based on existing test cases in A.13, we suggest the following test case list in Proposal 3.

Proposal 3: RAN4 to define the following TCs for IoT NTN TDD.

- Intra-frequency cell reselection

- Inter-frequency cell reselection

- Re-establishment

- Random access

- Transmit timing and TA accuracy

- RLM OOS and RLM IS, both without DRX and with DRX

- Intra-frequency neighbor cell measurement, with radio condition trigger and with location trigger

- Inter-frequency neighbor cell measurement with radio condition trigger

- Channel quality reporting accuracy, in both Msg3 and connected mode

Among all the test cases, RLM test cases may require further RAN4 discussion because RAN4 needs to determine the Qin and Qout which will impact the SNR levels used in the test case. Qin and Qout depends on the NPDCCH parameter, more specifically the Rmax. The Rmax used in existing test cases are much larger than 2, the value assumed for this WI, so RAN4 may need to conduct a simulation campaign.

Proposal 4: RAN4 to discuss the Qin and Qout for RLM test cases based on Rmax =2.

Conclusions

In this paper we provided our views on RRM performance requirements for IoT NTN TDD.

Proposal 1: RAN4 to define accuracy requirements for NRSRP/Q and channel quality report for IoT NTN TDD by re-using the existing requirements for NB-IoT NTN, except the side condition.

Proposal 2: RAN4 to define PHR requirements for IoT NTN TDD by re-using the existing requirements for NB-IoT NTN.

Proposal 3: RAN4 to define the following TCs for IoT NTN TDD.

- Intra-frequency cell reselection

- Inter-frequency cell reselection

- Re-establishment

- Random access

- Transmit timing and TA accuracy

- RLM OOS and RLM IS, both without DRX and with DRX

- Intra-frequency neighbor cell measurement, with radio condition trigger and with location trigger

- Inter-frequency neighbor cell measurement with radio condition trigger

- Channel quality reporting accuracy, in both Msg3 and connected mode

Proposal 4: RAN4 to discuss the Qin and Qout for RLM test cases based on Rmax =2.

Reference

- R4-2512149, WF on RRM requirements for IoT_NTN_TDD, Moderator (Qualcomm Incorporated)