3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514206

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on AI-enabled Measurement event prediction

Agenda item:			7.12.4

Document for:	Discussion

- Introduction

The updated WID with the following measurement event prediction objective was approved in RAN #109 [1].

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management, if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

Based on the description highlighted in yellow, the study in feasibility of direct event prediction core requirement will be postponed to Q2 2026, from this, in this contribution, some views on indirect will be provided.

From our understanding, based on the indirect measurement event prediction method defined in 38.744, such indirect event prediction is strongly dependent on measurement prediction. From this, the core requirement impact analysis to some extent can be shared between the two types of predictions. So in this contribution, we will focus on performance analysis.

Proposal 1: Indirect event prediction is strongly dependent on measurement prediction, the core requirement impact analysis (e.g., measurement/prediction) in measurement prediction can be inherited as much as possible.

- Discussion

In RAN4 #115 and #116 meetings, the following agreements on performance metric for event prediction use case were achieved, respectively

#115 Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP. #116 Issue 3-1: Performance metric for indirect measurement event prediction Agreement: - On top of the agreement made in RAN4 #115, RAN4 to discuss whether/how to define additional performance metrics for indirect measurement event prediction in WI phase. - Capture the candidate proposals in the WF for information purpose.

From the definition on absolute and/or relative accuracy of predicted RSRP perspective, the technical principle/conclusion obtained in RRM measurement prediction can be inherited

Proposal 2: For indirect measurement event prediction, the requirement on absolute and/or relative accuracy of predicted RSRP identified for measurement prediction can be reused.

In RAN2, the definition of indirect event prediction corresponding to temporal domain case A/ temporal domain case B/ frequency domain were agreed in RAN2 #128, captured as below,

Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model.

For measurement event prediction based on intra-frequency temporal domain case A, there are two options to establish handover model, that is option 1: Relying on legacy measurement event and option 2: Relying on predicted measurement event.

- Option 1                                                                                                   (b) option 2

Based on the 2 options, if to define additional performance metrics, we don’t think missed measurement event prediction rate is necessary. Since even if UE miss an event that will be happened in the future using AI, legacy HO procedure, that is HO decision based on real/actual measurement result still works, no negative performance impact, just worst case, no AI gain can be obtained, wherein  Counter n2 could be interpreted as missed measurement event prediction

Proposal 3: For indirect measurement event prediction, if to define additional performance metrics for indirect measurement event prediction, missed measurement event prediction rate is unnecessary.

- Conclusion

In this contribution, we have provided our viewpoints on AI-enabled Measurement event prediction. The following observations and proposals are obtained:

Proposal 1: Indirect event prediction is strongly dependent on measurement prediction, the core requirement impact analysis (e.g., measurement/prediction) in measurement prediction can be inherited as much as possible.

Proposal 2: For indirect measurement event prediction, the requirement on absolute and/or relative accuracy of predicted RSRP identified for measurement prediction can be reused.

Proposal 3: For indirect measurement event prediction, if to define additional performance metrics for indirect measurement event prediction, missed measurement event prediction rate is unnecessary.

4. Reference

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#109