3GPP TSG-RAN WG4 Meeting #116-bis 	R4-2514245

Prague, CZ, Oct 13th – 19th, 2025

Agenda Item: 5.9.3

Source: MediaTek inc.

Title: Discussion on RRM impacts on simultaneous CA_n5_n8

Document for: Discussion

# Introduction

On the topic of supporting band combination CA_n5-n8, RAN4 reached the following agreement [1]:

- RAN4 agrees to specify the support of the following implementation architecture in Rel-19 for CA_n5-n8 (i.e., Option 2) - 2UL/2DL CA is supported using non-concurrent n5 DL and n8 UL - Send LS to RAN2 to complete necessary signalling designs - Stop discussion on Option 3 in Rel-19. If option 3 is discussed in Rel-20 or later, new band numbers are considered.

After few meetings, RAN4 provided a comprehensive overall description background in the reply LS to R2 [2].

For carrier aggregation involving a low-low band combination where the UL of one band partially overlaps with the DL of the other band, such as CA_n5-n8, RAN4 conducted studies in Rel-18 on enhancements for 700/800/900MHz band combinations. The studies concluded that three options are feasible for implementing the CA_n5-n8 band combination, as detailed in TR 38.872.: 1) Full band n5 and n8 RF filters implementation with option 1 and option2: - Option 1: Only support 1UL/2DL CA. Single UL in n5 - Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL 2) Dedicated RF filters implementation with partial frequency range - Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL without any scheduling restriction on both bands. In particular, Option 1 has already been specified in Rel-18, while RAN4 has agreed in RAN4#112bis to specify only Option 2 to support full band frequency range of each band in Rel-19 (See WF R4-2417125) with Option 3 excluded in Rel-19 which enables simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restrictions. Moreover, standardizing Option 2 in Release 19 may necessitate a future-proof signalling design to accommodate the introduction of simultaneous operation capabilities for additional NR FDD-FDD inter-band CA combinations. These combinations may involve partial UL and DL overlap between different FDD bands or face duplexer-related challenges, as requested in the original liaison statement R4-2420410. Additionally, RAN4 has continued the discussion with regards to reply to RAN2 where several potential solutions for this band combination with new issues identified for Option 2, e.g., interrupt needed for RF front end configuration change, and PA ON/OFF etc, therefore, several solutions are discussed, each representing a different level of improvement. • Solution #1 – switching by activation and deactivation: § If UE reports support of 2UL for CA_n5-n8, network only activate either n5DL or n8UL at a time § Change of cases is done in such a way that to no extra interrupts/switching time is needed § Staying in Case 1 (n5DL+n8DL/n5UL) longer than Case 2 (n5DL/n5UL+n8UL) § RAN2 capability signaling is required • Solution #2 - New capability: introduce a UE capability of switching gap (i.e., switching period and location) to cover necessary switching period including RFFE configuration switching, PA On/Off etc., § Can be the same for both directions (n5DL -> n8UL, n8UL->n5DL), or one for each direction § Network accommodates the switching gap by scheduling § Cross-carrier scheduling might be needed (e.g., from Case 2 with only 1 DL n8DL to Case 1 with two DL) § No RAN1 spec change is required o Solution #3 - Further improvement, e.g., under L-L CA via switching framework § Not considered in Rel-19 § Not precluded in Rel-20 or later RAN4 has decided to support Solution #1 in Rel-19, where the required RAN2 capability signaling aligns with the original request. The RAN2 capability indicates two independent concurrent capabilities for a band combination consisting of two FDD bands, referred to as band nA and band nB: o Capability #1: A value of “1” indicates concurrent nA uplink and nB downlink operation; a value of “0” indicates that nA-UL and nB-DL are non-concurrent. o Capability #2: A value of “1” indicates concurrent nA downlink and nB uplink operation; a value of “0” indicates that nA-DL and nB-UL are non-concurrent. If 2UL is supported for the band combination CA_n5-n8, the two capabilities should be set to (1,0). Regarding the questions raised, in-line answers could be found below: Q1: Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version? Answer 1: If only Option 2 is assumed, it becomes impossible to implement simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without introducing scheduling restrictions. Furthermore, a UE would not be able to support simultaneous Rx/Tx in any portion of the overlapping FDD bands. However, as noted above, a UE implemented with Option 3 can support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restrictions, or enable simultaneous Rx/Tx in specific portions of the overlapping FDD bands. Nevertheless, RAN4 has excluded Option 3 in Rel-19. Q2: If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band? Answer 2: This can be further discussed when deciding to specify Option 3 in Rel-20 or later.

Furthermore, RAN4 reached the following agreement in previous meeting, which captured in the chairman notes:

Agreement [R4 chairman notes meeting#116]: For CA_n5-n8, in RAN4#116bis, an agenda item in RRM session will be introduced to discuss and determine if there is RRM impact. If RRM impact is identified and agreed, this topic will not be further pursued. If not, the discussion will continue in main session under R19 maintenance. - When n5DL is operated, n8UL RF path needs to be switched off and the same for the other way around. - Proponent companies are encouraged to bring detailed analysis, including the possible gain/loss. Before the conclusion on RRM impact is made, no further discussion is needed in the main session.

Therefore, in this document we provide discussion on the RRM impact due to the non-concurrent operation of n5 DL and n8 UL.

# Discussion

The bands n5 and n8 details are given below:

| Band | Uplink (MHz) | Downlink (MHz) |
| --- | --- | --- |
| n5 | 824 – 849 | 869 – 894 |
| n8 | 880 – 915 | 925 – 960 |

The operation of the two cases provided in the WF can be depicted in a sketch as:

In this contribution, we provide our observations on the following aspects:

Switching between Case 1 and Case 2: When a UE switches its operation between Case 1 and Case 2, RF switching time is required (either 35 us or 145 us, depending on the UE’s capability). The timing of this RF switching can have a significant impact on the entire PUSCH/PDSCH packet. Although the RF switching may only affect a few OFDM symbols within a packet, even partial data loss can make the entire packet unrecoverable. Therefore, RAN4 RRM should further investigate whether interruption requirements need to be considered for such switching events.

- The RRM group should further assess whether to define interruption requirements to address RF switching between Case 1 and Case 2 for non-concurrent CA_n5-n8 operation.

Measurement on DL n5 vs UL grant on UL n8: When the UE is operating as non-concurrent for n5 and n8, a collision scenario between a measurement on n5 DL might collide in time with an UL grant (PUSCH/PUCCH) transmission on n8. The impact can be seen as either scheduling restriction on UL grants or extending measurement delay requirements.

- The RRM group should investigate the impact of overlapping/collision of measurements on n5 DL and UL grant transmissions (PUSCH/PUCCH) on n8 UL.

Inter-UE interference (inter-UE CLI): Another issue associated with the n5 and n8 band combination is inter-UE interference. This occurs when two UEs are in close proximity—one UE is transmitting on n8 UL while the other is receiving on n5 DL. The transmission from the first UE on n8 UL can interfere with the reception of the second UE on n5 DL. This interference becomes more severe if the first UE is transmitting while the second UE is receiving the SSB, which can negatively affect measurement accuracy. Therefore, RAN4 RRM should further study this impact and consider whether scheduling restrictions are necessary.

- The RRM group should also examine inter-UE interference that may occur when two UEs operate simultaneously, with one in Case 1 and the other in Case 2.

# Summary

In summary:

Proposal 1: The RRM group should further assess whether to define interruption requirements to address RF switching between Case 1 and Case 2 for non-concurrent CA_n5-n8 operation.

Proposal 2: The RRM group should investigate the impact of overlapping/collision of measurements on n5 DL and UL grant transmissions (PUSCH/PUCCH) on n8 UL.

Proposal 3: The RRM group should also examine inter-UE interference that may occur when two UEs operate simultaneously, with one in Case 1 and the other in Case 2.

# References

- R4-2417125, “WF on UL CA_n5-n8”, CATT, China Telecom, Huawei, HiSilicon, Hefei, China, Oct 2024.

- R4-2506064, “Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx”, St Julian, Malta, May 2025.