3GPP TSG-RAN WG4 Meeting #116b			R4-2514250

Prague, Czechia, 13th - 17th Sep 2025

Agenda item:	4.27.4

Title:	On remaining issues of IoT_NTN_TDD related to RRM core

Source:	Nordic Semiconductor ASA

Document for:	Discussion

1. Introduction

WF on RRM requirements for IoT_NTN_TDD has been approved in RAN4#116 in [R4-2512149]. There have been few leftovers to resolve:

To be further discussed and concluded in RAN4#116-bis: - Change ‘110 ms’ in the following bullets to an integer number of 90ms + DL-to-UL gap within the 90 ms TDD pattern, where the exact value is TBD upon completion of RAN1 spec. - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended. - TBD for the values of 10 ms, 40 ms, and 8 ms in the following bullets, and whether to retain the DL-Bitmap-NB IE upon completion of RAN1 and RAN2 spec: - TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. - Option 1: TDL-UL switch is 8 ms. - Option 2: TDL-UL switch is 50 ms (THALES, Iridium) Agreement (online - Wed): - Tsearch_NB1-NC,i in RRC re-establishment requirement is - If the target cell is known, then Tsearch_NB1-NC,i = 0 ms. If the target cell is unknown, Tsearch_NB1-NC,i = Ksatellite,i *2.52 sec. - FFS the definition of Ksatellite,i

### 6.9B.2 Requirements The UE shall be capable of performing the RRC connection redirection to the non-anchor carrier within Tconnection_redirect_non-anchor. The time delay (Tconnection_redirect_non-anchor) is the time between the end of the last subframe in the repetition period of NPDSCH containing the IE, “CarrierConfigDedicated-NB” received on the anchor carrier and the end of the last subframe in the repetition period of NPUSCH transmitted on the target non-anchor carrier. The time delay (Tconnection_redirect_non-anchor) shall be less than: Tconnection_redirect_non-anchor = TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - Tperiod_DL_bitmap: 0 ms. - TUL_grant: It is the time required to acquire uplink grant from the non-anchor carrier for transmitting NPUSCH on the non-anchor carrier. The value of TUL_grant depends on Tperiod_DL_bitmap and th e number of repetitions of NPDCCH used in the non-anchor carrier. - TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 50 ms. - TNPUSCH: It is the time required to transmit NPUSCH on the non-anchor carrier. The value of TNPUSCH depends on the number of repetitions of NPUSCH used in the non-anchor carrier. When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended.

2. Discussion

The non-anchor re-direction for IoT NTN TDD should be updated and simplified according to Figure 1. Respecting the legacy RRC delay of 110ms, the first DL burst to receive UL grant on a non-anchor is the second DL burst after the burst containing RRC message (CCD).  This being 172ms including all the switching gaps.  After that procedure and its requirement continues as in legacy.

Figure 1  Redirection to non-anchor

Proposal-1: Update requirement 6.9.B2 as the following:

### 6.9B.2 Requirements The UE shall be capable of performing the RRC connection redirection to the non-anchor carrier within Tconnection_redirect_non-anchor. The time delay (Tconnection_redirect_non-anchor) is the time between the end of the DL burst containing the last subframe in the repetition period of NPDSCH containing the IE, “CarrierConfigDedicated-NB” received on the anchor carrier and the end of the last subframe in the repetition period of NPUSCH transmitted on the target non-anchor carrier. The time delay (Tconnection_redirect_non-anchor) shall be less than: Tconnection_redirect_non-anchor = TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 172 ms. - Tperiod_DL_bitmap: 0 ms. - TUL_grant: It is the time required to acquire uplink grant from the non-anchor carrier for transmitting NPUSCH on the non-anchor carrier. The value of TUL_grant depends on Tperiod_DL_bitmap and the number of repetitions of NPDCCH used in the non-anchor carrier. - TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 50 ms. - TNPUSCH: It is the time required to transmit NPUSCH on the non-anchor carrier. The value of TNPUSCH depends on the number of repetitions of NPUSCH used in the non-anchor carrier. When the NPUSCH ACK transmission for the received RRC message takes longer than 110 148 ms, the overall RRC connection redirection delay is extended by multiples of 90ms may be extended.

In legacy R18 specification it states:

The parameter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

Proposal-2: Definition of Ksatellite can be reused for R19 IoT NTN TDD.

3. Conclusion

In this contribution we have two proposals:

Proposal-1: Update requirement 6.9.B2. in 36.133

Proposal-2: Definition of Ksatellite can be reused for R19 IoT NTN TDD.