3GPP TSG-RAN WG4 Meeting #116bis	R4-2514254

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	LG Electronics

Title:	Discussion on low NR band carrier aggregation via switching

Agenda item:	7.8.3

Document for:	Discussion

Introduction

In 3GPP RAN #109, WID[1] was approved. In this paper, we provide our views on enhancement of low NR band aggregation via switching.

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

Discussion

Clarification for architecture of Rel-20 LBCA

WID for low‑band CA via switching (LB‑CA) was approved at RAN#109, and the objectives were decided as per the WID table above. From Rel‑20, the scope is expanded to the scenario between Case 1 and Case 3, as referenced in prior RAN4 discussions on switching‑based CA. When we compare with the Rel‑19 scope, the most important point is the UE architecture for Rel‑20 LB‑CA via switching from an RF‑requirement perspective. In Rel‑19, the assumed architecture is as shown in Figure 1, i.e., a single‑chain SPDT‑based realization.

Fig1. Architecture for Rel-19

In the Rel‑19 WI phase, RAN4 considered certain parameters for defining RF requirements, such as the switching period for supporting low‑band carrier aggregation via the switching feature. In fact, there is no explicit RF requirement related to RX switching in the NR RF specifications, so RAN4 reflected the additional operation only in the on/off time mask. DL‑related information is captured in TR 38.768 as study content. The switching period means the time required to retune RF components to support a specific band operation, including LO/PLL settling time, synthesizer retuning time, and any filter reconfiguration, etc. The values are 35 µs, 70 µs, and 140 µs. When the UE supports low‑band carrier aggregation, the switching period should be taken into account for requirement derivation and test configuration selection.

Then, what is the difference between Rel‑19 and Rel‑20? The UE can support both DL carriers, one in each band, concurrently under the Rel‑20 scope. So one difference in Rel‑20 compared with Rel‑19 is support for an additional DL in FDD mode, as follows.

Observation 1: The key difference between Rel‑19 and Rel‑20 is the support of FDD DL enabling DL‑DL operation, with additional CA combinations such as CA_n12‑n29 and CA_n28‑n67.

(a)

(b)

Fig. Scenario for (a)Rel-19 and (b)Rel-20

Rel-20 pursue UE could support 2 DL at same time. And other difference is to support other band( CA_n12-n29, CA_n28-n67).

(a)CA_n12-n29                                                                         (b)CA_n28-n67

Fig. Example CA band combination for low band carrier aggregation via switching

Although the gap between the UL and DL spectrum may be very small (about 1 MHz) or even overlap (up to ~10 MHz), the Rel‑20 low‑band CA operation precludes UL/DL from operating simultaneously within this feature scope. There is therefore no UL–DL interference in the scenarios considered by assumption. We think the most important question is which architecture can support the Rel‑20 feature from a RAN4 RF‑requirement standpoint. When we look back at our Rel‑19 architecture, only a single chain is supported, as follows, with a shared front end and SPDT path selection. The UE cannot physically support both bands simultaneously due to path isolation.

Observation 2: In Rel‑19, the architecture used in RAN4 cannot support both bands

Fig. Assumed architecture in Rel-19

physically because, when the SPDT is switched to make one path for a specific band, the other path is isolated. So the UE cannot support dual‑band operation because the paths are physically isolated. But to expand and support DL in FDD, RAN4 needs to consider a new architecture to support the new feature while keeping the specifications. We think there are options for supporting the Rel‑20 feature that enable Rx on both FDD carrier 1 and SDL carrier 2, with no Tx on FDD carrier 1, as shown in the following figure.

- (b)

Fig. Example of simplified structure (a)single chain (b)dual chain for Rel-20 LB CA

When we consider the architecture for Rel‑20 to support LB‑CA via switching, the first option(Figure(b)) is a single chain, which is similar to Rel‑19. But the difference is to use broadband bandpass filter(n29(SDL)+n12(FDD DL)). We can use the other option (Figure (c)); this uses a dual chain implemented with two transceivers (i.e., two independent receive chains). We usually state the number of T/Rx by counting the number of transceivers. A transceiver means the core up‑/down‑conversion chain, such as the LO (local oscillator), mixers, PLL/synthesizer, and ADC/DAC. For more specific structural clarification, we can refer to the following structure and block diagrams.

(a)

- (c)

Fig. Specific LB CA architecture (a)single chain for Rel-19 (a)candidate single chain for Rel-20 (b)candidate dual chain for Rel-20

As we can see in the figure above, when we use a single chain in Figure (b), the band-pass filter should cover both bands (wide BPF coverage). I think we need to update switching period according to change BPF characteristic. when RAN4 uses a dual chain as in Figure (c), it is unnecessary to introduce any further RX-switching-specific RF requirements beyond the existing CA framework. We can reuse the NR legacy on/off time mask.

Proposal 1: RAN4 should study whether switching period is updated or not according to architecture

Proposal 2: RAN4 should decide whether to adopt a single chain or a dual chain for Rel-20 LB-CA as the baseline assumption for requirement.

Single chain                                                                              dual chian

Proposal 3: RAN4 should determine whether the on/off time mask is reused or not according to the architecture

‘Switching from’ or ‘Switching to’ for time mask

In Rel‑19, RAN4 discussed where the switching should be applied within the time‑mask definition. One reason for deciding on “switching from” was to avoid loss of PDCCH. If the switching gap were applied to “switching to,” it could harm PDCCH symbols by truncating or erasing control symbols. So, likewise, Rel‑20 could reuse the Rel‑19 approach for consistency.

Proposal 4: The switching gap should be applied to “switching from,” as in Rel‑19, to maintain control‑channel robustness.

Capability

With respect to capability, the capability defined in Rel‑19 must either be updated or a new capability must be created. The Rel‑19 capability cannot support LB‑CA that switches between Case 1 and Case 3.

Proposal 5: Capablity is needed for supporting Rel-20 LBCA(Case1<->Case3)

- 2. Conclusion

In this contribution, we want to share some views on low NR band aggregation via switching and proposals are made as following:

Observation 1: The key difference between Rel‑19 and Rel‑20 is the support of FDD DL enabling DL‑DL operation, with additional CA combinations such as CA_n12‑n29 and CA_n28‑n67.

Observation 2: In Rel‑19, the architecture used in RAN4 cannot support both bands

Proposal 1: RAN4 should study whether switching period is updated or not according to architecture

Proposal 2: RAN4 should decide whether to adopt a single chain or a dual chain for Rel-20 LB-CA as the baseline assumption for requirement applicability.

Single chain                                                                              dual chian

Proposal 3: RAN4 should determine whether the on/off time mask is reused or not according to the architecture

Proposal 4: The switching gap should be applied to “switching from,” as in Rel‑19, to maintain control‑channel robustness.

Proposal 5: Capablity is needed for supporting Rel-20 LBCA(Case1<->Case3)

- 3. Reference

[1] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, RAN#109, September,2025

- Annex

Time mask for Rel-20

- FDD(UL/DL) to SDL/FDD(DL) time mask for single chain(LB CA time mask can be reused or update switching period)

- FDD(UL/DL) to SDL/FDD(DL) time mask for dual chain(NR requirement can be reused)

- SDL/FDD(DL) to FDD(UL/DL) time mask for single chain(LB CA time mask can be reused or update switching period)

- SDL/FDD(DL) to FDD(UL/DL) time mask for dual chain(NR requirement can be reused)