3GPP TSG-RAN WG4 Meeting#116bis                                R4-2514276

Prague, CZ, 13th – 17th, Oct. 2025

Agenda item:	6.13.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on conformance testing of the legacy RF requirement for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some initial views on conformance testing for SBFD BS.

Issue 10: BS conformance testing - manufacturer declaration - Way forward - FFS in next meeting - All the declarations in the WF is needed - In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Tx requirements shall be updated to include the relevant clauses for SBFD capable BS. - It is necessary to add the definitions of output power requirements for SBFD capable BS to TS 38.141-1 and TS 38.141-2. - It is necessary to add the definition of total power dynamic range requirement for SBFD capable BS to TS 38.141-1 and TS 38.141-2. - The definitions of EVM and averaged EVM for SBFD capable BS need to be added to TS 38.141-1 and TS 38.141-2. - The OTA measurement chamber setup, calibration, and measurement procedures defined for NR BS can serve as the baseline for testing Tx requirements of SBFD capable BS. - In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS. - The OTA measurement chamber setup and calibration (and measurement) procedures of Rx requirements testing for SBFD capable BS are different from those of NR systems, and these differences must be added to TS 38.141-2. Issue 11: BS conformance testing - RF channels - Way forward - FFS in next meeting - New test models need to be set for SBFD BS. - Guarantee SBFD carrier is configured at the left-most, right-most and middle of the operating band Issue 12: BS conformance testing - Test model - Way forward - FFS in next meeting - New parameters such as guard-band size should be described clearly in the spec. - The test model for SBFD DL subbband needs to be scalable. - New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband. - Signal statistics may change for SBFD test models and need to be investigated. The impact of DL subband on signal statistics needs to be analyzed. - In TS 38.141-1 and TS 38.141-2, the test models specified for NR BS can be reused for the existing Tx requirements testing of SBFD capable BS, but with the different allocated channel in tests. - A new test model which transmitter and the receiver operate simultaneously must be set up for Rx requirement testing of SBFD capable BS. - Test requirements for each Rx requirement of NR BS in TS 38.141-1 and TS 38.141-2 can serve as the baseline for SBFD capable BS. - In test requirements, NR channel bandwidth needs to be updated to the UL subband bandwidth of the SBFD capable BS. - Any test requirements for SBFD capable BS that differ from NR BS test requirements must be specified for SBFD capable BS in TS 38.141-1 and TS 38.141-2. Issue 13: BS conformance testing - Test signal - Way forward - FFS in next meeting - define the test signal for SBFD conformance testing in the following way: - Table 4.7.2-1: Signal to be used to build NR SBFD TCs Operating Band characteristics; Band n39; Other FR1 bands. TC signal; BWchannel; 20 MHz (Note 1); 40 MHz (Note 1). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing. NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used.. - Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O Operating band characteristics; FDL_high – FDL_low ≤ 3250 MHz. TC signal; BWchannel; 100 MHz (Note 1, Note 2). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing declared per operating band (D.7). NOTE 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used.. Issue 14: BS conformance testing - Beam configuration - Way forward - FFS in next meeting - When test Tx requirements of SBFD Capable BS, receivers can be turned off. - In the test of Tx requirements of SBFD Capable BS, the Tx antennas beam is test output beam. - When test OTA Rx requirements of SBFD capable BS, the transmitters must be turned on. - For OTA Rx requirements testing of SBFD capable BS, the beam configurations of Tx and Rx antennas need to consider the isolation requirement between them. - For OTA Rx requirements testing of SBFD capable BS, it is necessary to consider the directional correspondence between the test input antenna beam and the Rx antenna beam.

Discussion

## 2.1. Test signal and test configuration

First of all, the minimum supported bandwidth for SBFD BS has been agreed in the following way, in other word, the current signal for SBFD conformance testing should be updated according to the latest agreement.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. Note: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39.. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264. Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz..

Proposal 1: propose to define the test signal for SBFD conformance testing in the following way:

Table 4.7.2-1: Signal to be used to build NR SBFD TCs

| Operating Band characteristics |  | Band n39 | Other FR1 bands |
| --- | --- | --- | --- |
| TC signal | BWchannel | 20 MHz (Note 1) | 40 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Secondly, there are lots of test configuration for NR BS, however lots of test configuration which is not relevant with SBFD BS operation e.g. multi-band operation and also non-contiguous spectrum operation;

Proposal 2: propose to define the test configuration for SBFD BS in the following way:

| NRTC1: Contiguous spectrum operation | NRTC1: Contiguous spectrum operation |
| --- | --- |
| NRTC2: Contiguous CA occupied bandwidth | NRTC2: Contiguous CA occupied bandwidth |
| NRTC3: Non-contiguous spectrum operation | NRTC3: Non-contiguous spectrum operation |
| NRTC4: Multi-band test configuration for full carrier allocation | NRTC4: Multi-band test configuration for full carrier allocation |
| NRTC5: Multi-band test configuration with high PSD per carrier | NRTC5: Multi-band test configuration with high PSD per carrier |

## 2.2. Test model

For test model for SBFD symbol in the specification, the existing test model in the specification is based on the transmission bandwidth configuration of the test signal, however for SBFD symbols, this NRB should be updated as the declared as DL sub-band;

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR1-TM1.2

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs boosted | x=40% |
| # of QPSK PDSCH RBGs which are boosted | NRBG=minxNRB−3P,12NRB−3+3modP−PP−NRB−3+3modP−PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB−3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of PDSCH RBGs which are boosted | NRB−3+3modPP−1 and if NRBG>1, 1, 3, …, 2NRBG−2+1 |
| Level of boosting (dB) | 3 |
| # of QPSK PDSCH PRBs which are deboosted | NRB−3−PNRBG |
| Locations of PDSCH RBGs which are deboosted | Starting at RB#3 and excluding PRBs of RBGs which are boosted |
| Level of deboosting (dB) | 10log10NRB−3−103/10PNRBGNRB−3−PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

Table 4.9.2.2.3-1: Specific physical channel parameters of NR-FR1-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3−1. 3n+1; NRB2; n=0,…,10×2μ−13−1. 3n+2; NRB−1; n=0,…,10×2μ−23−1. |
| # of PDSCH PRBs which are not allocated | NRB−1 |

Table 4.9.2.2.7-1: Specific physical channel parameters of NR-FR1-TM3.2

| Parameter | Value |
| --- | --- |
| Target percent of 16QAM PDSCH PRBs deboosted | x = 60% |
| # of 16QAM PDSCH RBGs within a slot for which EVM is measured | NRBG=minxNRB−3P,12NRB−3P−NRB−3Pmod2+1NRBG=minxNRB−3P,12NRB−3+3modP−PP−NRB−3+3modP−PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB−3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of 16QAM RBGs which are deboosted | NRB−3+3modPP−1 and if NRBG>1, 1, 3, …, 2NRBG−2+1 |
| Level of deboosting (dB) | -3 |
| # of QPSK PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB−3−PNRBG |
| Locations of PDSCH RBGs which are boosted | Starting at RB#3 and excluding PRBs of RBGs which are deboosted |
| Level of boosting (dB) | 10log10NRB−3−10−3/10PNRBGNRB−3−PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 for which EVM is not measured | QPSK |

Table 4.9.2.2.8-1: Specific physical channel parameters of NR-FR1-TM3.3

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs deboosted | x = 50 % |
| # of QPSK PDSCH RBGs within a slot for which EVM is measured | NRBG=minxNRB−3P,12NRB−3+3modP−PP−NRB−3+3modP−PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB−3 as the size of the bandwidth part and NBWPstart=3 |
| Level of deboosting (dB) | -6 |
| Locations of QPSK RBGs which are deboosted | NRB−3+3modPP−1 and if NRBG>1, 1, 3, …, 2NRBG−2+1 |
| Level of deboosting (dB) | -6 |
| # of QPSK PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB−3−PNRBG |
| Locations of QPSK RBGs for power balancing | Starting at RB#3 and excluding PRBs of RBGs which are deboosted |
| Level of boosting (dB) | 10log10NRB−3−10−610PNRBGNRB−3−PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 for which EVM is not measured | QPSK |

Observation1: there are following two ways to define the test model for SBFD symbols:

- Option 1: to replace the legacy NRB of test signal with the declared PRB number of DL sub-band;

  - NOTE: this approach will need the new test model implemented in the baseband to cater for conformance testing of SBFD symbols;

- Option 2: to reuse the existing NRB of test signal with the frequency range of UL sub-band and Guard band to be blanked

  - NOTE: this approach will allow to reusing the existing test model implemented with blanking frequency range of UL sub-band. This is expected to be easiest upgrade for the conformance testing of SBFD symbols;

- Proposal 3:For test model for SBFD symbols, propose to reuse the existing NRB of test signal with the frequency range of UL sub-band and Guard band to be blanked

## 2.3. RF channel

For RF channels for conformance testing, usually this should be discussed case by case for different RF requirement to achieve the balance between test coverage and conformance testing of wors case. In addition, for FR1 WA and higher power MR above 36 dBm with the implementation of Rx analog filter, it is not possible to have multiple RF channels for the conformance testing.

- Proposal 4:For RF channels for SBFD conformance testing, propose to discuss the case by case.

- Proposal 5: For RF channels for FR1 WA BS and MR BS with output power higher than 36dBm, this should be treated differently with other BS class.

## 2.4. Test procedure

Regarding the test procedures for SBFD symbols, except for additional In-channel adjacent subband leakage and In-channel adjacent subband blocking for SBFD and Transient period between SBFD and non-SBFD operation, other RF requirement for SBFD symbols are aligned with non-SBFD symbols, in other words, the existing test procedures for non-SBFD symbols could be also reused for SBFD symbols.

Proposal 6: For the test procedure for SBFD symbols, except for In-channel adjacent subband leakage and In-channel adjacent subband blocking for SBFD and Transient period between SBFD and non-SBFD operation, propose to reuse the existing test procedures for the SBFD symbols’ conformance testing.

## 2.5. MU and test tolerance

Regarding the test procedures for SBFD symbols, except for additional In-channel adjacent subband leakage and In-channel adjacent subband blocking for SBFD and Transient period between SBFD and non-SBFD operation, it’s expected to reuse the same test setup or test chamber for the conformance testing between non-SBFD and SBFD symbols. If the transmitter turn on during Rx conformance testing don’t have the impacts on Rx MU analysis, then it should be fine to reuse the existing MU for SBFD symbols’ testing requirement.

Proposal 7: If the activated transmitter during Rx conformance testing don’t have the impact on Rx MU analysis, propose to reuse the existing MU for SBFD symbols’ testing requirement.

Conclusions

In this contribution, we want to share some initial views on conformance testing for SBFD BS and proposals/observations are made as following:

Proposal 1: propose to define the test signal for SBFD conformance testing in the following way:

Table 4.7.2-1: Signal to be used to build NR SBFD TCs

| Operating Band characteristics |  | Band n39 | Other FR1 bands |
| --- | --- | --- | --- |
| TC signal | BWchannel | 20 MHz (Note 1) | 40 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for SBFD BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Proposal 2: propose to define the test configuration for SBFD BS in the following way:

| NRTC1: Contiguous spectrum operation | NRTC1: Contiguous spectrum operation |
| --- | --- |
| NRTC2: Contiguous CA occupied bandwidth | NRTC2: Contiguous CA occupied bandwidth |
| NRTC3: Non-contiguous spectrum operation | NRTC3: Non-contiguous spectrum operation |
| NRTC4: Multi-band test configuration for full carrier allocation | NRTC4: Multi-band test configuration for full carrier allocation |
| NRTC5: Multi-band test configuration with high PSD per carrier | NRTC5: Multi-band test configuration with high PSD per carrier |

Observation1: there are following two ways to define the test model for SBFD symbols:

- Option 1: to replace the legacy NRB of test signal with the declared PRB number of DL sub-band;

  - NOTE: this approach will need the new test model implemented in the baseband to cater for conformance testing of SBFD symbols;

- Option 2: to reuse the existing NRB of test signal with the frequency range of UL sub-band and Guard band to be blanked

  - NOTE: this approach will allow to reusing the existing test model implemented with blanking frequency range of UL sub-band. This is expected to be easiest upgrade for the conformance testing of SBFD symbols;

- Proposal 3:For test model for SBFD symbols, propose to reuse the existing NRB of test signal with the frequency range of UL sub-band and Guard band to be blanked;

- Proposal 4:For RF channels for SBFD conformance testing, propose to discuss the case by case;

- Proposal 5: For RF channels for FR1 WA BS and MR BS with output power higher than 36dBm, this should be treated differently with other BS class;

Proposal 6: For the test procedure for SBFD symbols, except for In-channel adjacent subband leakage and In-channel adjacent subband blocking for SBFD and Transient period between SBFD and non-SBFD operation, propose to reuse the existing test procedures for the SBFD symbols’ conformance testing.

Proposal 7: If the activated transmitter during Rx conformance testing don’t have the impact on Rx MU analysis, propose to reuse the existing MU for SBFD symbols’ testing requirement.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413283, On modification of existing TX RF requirements for SBFD, Huawei

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved

- R4-2411640,On the potentially new requirements for SBFD operation for FR1 and FR2-1, Samsung

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.

- R4-2512665,Way Forward for [116][306] NR_duplex_evo_General

- R4-2512680,Way Forward for [116][307] NR_duplex_evo_BSRF