3GPP TSG-RAN WG4 Meeting#116bis                                R4-2514278

Prague, CZ, 13th – 17th, Oct. 2025

Agenda item:	7.2.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Rel-20 Co-location requirement

Document for:	Approval

Introduction

In the RAN#109 meeting, a new WID: New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2 has been approved with Rel-19 left over issue: Co-location requirement included. In this contribution, we would like to share our initial understanding for Rel-20 Co-location requirement enhancement.

- Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Consider impact on the following OTA co-location requirements: - Transmitter OFF power requirement - Transmitter co-location spurious emission requirement - Transmitter intermodulation requirement - Co-location OOBB requirement - Enhance OTA co-location requirements - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered - Option 1: Investigate and improve CLRA concept - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS), if any

Discussion

## 2.1. General

According to the objective in WID, the target frequency is above 3.3GHz. As shown in Figure 2.1-1/2/3, the targeted frequency above 3.3GHz includes license bands n77, n78, n48 (CBRS band), n79, n104 and unlicensed bands n46, n96 and n102 and V2X band n47.

Figure 2.1-1. Band definition of n77/n78 and n48 at 3.5GHz

Figure 2.1-2. Band definition of n79

Figure 2.1-3. Band definition n46/n47/n96/n102 and n104

For the unlicensed bands, in the current specification, it is restricted to MR BS type 1-C due to the regulatory maximum transmission power limitation. In addition, FR1 WA BS type 1-O is not expected to be co-located with unlicensed MR BS type 1-C, therefore we propose not to consider the unlicensed band in Rel-20 BS RF enhancement.

Proposal 1: In Rel-20 BS RF enhancement, propose not to consider the FR1 unlicensed band for co-location requirement improvement.

In the following Table 2.1-1, we summarized the potential RF requirement impacts on NR BS supporting band n77/78/79 and n104. For band n48, from our understanding, we don’t need to consider the Tx co-location spurious emission requirement and co-location blocking requirement with band n79 and n104 since band n48 is mainly deployed in north American market, but North American market don’t have band n79 and n104 deployed or to be deployed so far from our understanding.

In addition, according to the measurement and simulation results​in TR 37.941 clause 6.4.4, only the yellow-highlighted cases have been investigated in Rel-19, therefore we propose to continue the investigation on the rest of co-located cases which are not covered in Rel-19.

Proposal 2: In Rel-20 BS RF enhancement, propose to continue the investigation on the rest of co-located cases which are not covered in Rel-19.

Table 2.1-1. The impacts on co-location requirement improvement on BS type 1-O supporting band n78/79/n104

| Supported band | Transmitter OFF power | Tx intermodulation requirement | Tx co-location Spurious emission | Receiver co-location blocking |
| --- | --- | --- | --- | --- |
| n77/n78 | Intra-band co-channel | Intra-band | n79 and n104 | n79 and n104 |
| n79 | Intra-band co-channel | Intra-band | n77/78 and n104 | n77/78 and n104 |
| n104 | Intra-band co-channel | Intra-band | n77/78 and n79 | n77/78 and n79 |

- In addition, in the previous Rel-19 co-location analysis, it is primarily based on the assumption of 10cm horizontal separation distance between two BSs to mimic 30dBc coupling loss in the past. But when we check the practical AAU installation guidance in field as shown in Figure 2.1-4 for horizontal placement and Figure 2.1-5 for vertical placement, RAN4 assumption for the conformance testing in the horizontal domain with 10cm edge-to-edge separation distance is much more stringent than the recommended installation in field. In other words, we should also consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption.

Figure 2.1-4. The illustration of horizontal co-location deployment for two NR BSs from different operators

Figure 2.1-5. The illustration of vertical co-location deployment for two NR BSs from different operators

- Proposal 3: For Rel-20 NR BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption which is used to mimic 30dBc coupling loss.

## 2.2. OTA co-location improvement

- In the following section, we want to highlight the impacts of the improved co-location requirements on the RF implementation according to our understanding.

| Requirement | Remarks |
| --- | --- |
| Transmitter OFF power | The current -106dBm/MHz is based on -85dBm/MHz and 30dB coupling loss between the aggressor and Co-location reference antenna in the TS 38.104, then the minimum requirement is defined as -(115+9)dBm/MHz=-106dBm/MHz=-116dBm/100kHz. Indeed it should be noted that transmitter OFF power from UTRA phase is not derived based on the co-location deployment assumption. Transmit OFF power is measured within the carrier instead of out of carrier for the protection of different operators’ receiver performance and it was developed with two BSs deployed as following assumption with the observation of MCL as 30dBc in the past; This 30dBc derived from the following Figure is aligned with BS2BS co-location MCL analysis by coincidence at the end. It should be noted that the transmitter OFF power for the UTRA phase is not derived based on the co-location deployment assumption. The transmit OFF power is measured within the carrier to protect own system’s receiver performance rather than outside the carrier to protect the receiver performance of different operators. This was developed under the following assumption of two base stations (BSs) being deployed as shown in the following Figure, with a historical observation of a minimum coupling loss (MCL) of 30 dBc. The 30 dBc value, derived from the following figure, coincidentally aligns with the BS-to-BS co-location MCL analysis at the conclusion. Later on, considering the -85dBm+9dB=-76dBm/MHz is too lower for the measurement in the OTA chamber, then co-location reference antenna is used to transform the OTA TRP measurement to Co-location reference antenna based conducted measurement. In the Rel-20, basically we need to address the following two issues firstly: - More appropriate MCL1 between two BSs according to the legacy deployment assumption from the single operator as shown in above Figure and relevant BS’s antenna configuration to derive the core conductive requirement (no intention to change the conducted core requirements according to the WID objective) ; - More appropriate co-location reference antenna for the OTA requirement improvement and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement; - If MCL1 assumption for FR1 higher bands above 3.3GHz identified in issue 1 is relaxed to higher value due to the higher operating frequency, indeed this will not have the impacts on RF implementation from technically speaking since there is no much implementation difficulties to turn off the transmitter or configure large digital attenuation for the transmitter to emulate the transmitter OFF status. But this might be useful for the conformance testing, if transmitter OFF power is improved to be relatively high value which might make the TRP measurement for OFF power in the OTA chamber feasible. |
| Transmitter intermodulation | For the transmitter intermodulation requirement, NR BS need to comply with the out-of-band emission/ACLR/spurious emission requirement according to the interference signal injected from co-location reference antenna with the input power as min(46 dBm, Prated,t,TRP). Indeed if the NR BS is implemented with circulator for external interference signal as following, then IMD impacts due to transmitter intermodulation requirement should be quite impacted. It is also true that legacy passive co-location reference antenna at FR1 high bands might be not relevant/valid to the practical deployment and also make the testing antenna different from the commercial deployment. Another aspects, due to the higher isolation for intra-band case especially for above 4GHz, we could further discuss the necessity of removal of this requirement; According to the MCL results around 3.5GHz as captured in TR 37.941, this is still close to the legacy 30dBc, except for improving the current co-location reference antenna, otherwise we don’t see much necessity to change the requirement for band n77/n78 Tx intermodulation requirement at least. |
| Transmitter spurious co-location emission | For the transmitter co-location spurious requirement, considering the larger MCL1 observed for inter-band case, the corresponding requirement could be potentially improved which will have direct impacts on front-end filter design especially for the harmonic signal within the spurious frequency region. In Rel-20, we basically need to consider the following two aspects: - More appropriate MCL1 between co-located BS from different bands to figure out the core requirement, then use the following equation to derive the corresponding TRP based transmitter spurious emission requirement and check its feasibility of TRP measurement in the OTA chamber; - = -174dBm/Hz + 5dB + 50dB－7dB + MCL+9dB (scaling factor) - More appropriate co-location reference antenna and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement; |
| Receiver out-of-band blocking | For receiver out-of-band blocking requirement, NR BS needs to comply with EISminSENS + 6 together with the interference signal injected from co-location reference antenna with the input power as 46dBm for FR1 WA BS. If we don’t need to align the requirement between BS type 1-H and 1-O, then it might be okay just to provide more relevant testing antenna as alternative with total transmission power as 46dBm (e.g..AAS BS with 46dBm TRP). If we need to align the requirement between BS type 1-H and 1-O, the coupling loss between DUT and improved co-location reference antenna will have impacts on the expected Total transmission power from the interfering signal source. In addition, this improved requirement will potentially have the impacts on receiver’s analog filter design. |

- Observation 1: For OTA transmitter OFF power improvement, don’t see too much implementation burden on the existing requirement except for the measurement for conformance testing;

Proposal 4: For OTA transmitter OFF power in Rel-20, we propose to consider following aspects:

- Step 1: to identify more appropriate MCL1 between two BSs according to the legacy deployment assumption from single operator and relevant BS’s antenna configuration to derive the core conductive requirement(no intention to change the conducted core requirements according to the WID objective) ; then check whether TRP based OFF power measurement in the OTA chamber is feasible or not;

- Step 2: If TRP based OFF power measurement in step 1 is not feasible, then to define more appropriate/relevant co-location reference antenna for the OTA requirement improvement and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement;

Observation 2: For OTA transmitter intermodulation requirement, if NR BS is implemented with circulator for protection of external interference signal, IMD impacts due to transmitter intermodulation requirement should be quite impacted.

Proposal 5: For OTA transmitter intermodulation requirement in Rel-20, we propose to consider the following aspects:

- Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment;

- Step 2: Discuss whether the transmitter intermodulation requirement for band n79 and n104 except for band n77/n78 could be optionally removed from regions which don’t mandate the Tx intermodulation testing;

Proposal 6: For OTA transmitter spurious co-location emission in Rel-20, we propose to consider following aspects:

- Step 1: To identify more appropriate MCL1 between co-located BS from different bands, then use the following equation to derive the corresponding TRP based transmitter spurious co-location emission requirement and check its feasibility of TRP measurement in the OTA chamber firstly;

- = -174dBm/Hz + 5dB + 50dB－7dB + MCL1+9dB (scaling factor)

- Step 2: If TRP based OTA transmitter spurious co-location emission in step 1 is not feasible, to discuss more appropriate co-location reference antenna and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement;

Proposal 7: For OTA Receiver out-of-band blocking in Rel-20, we propose to consider following aspects:

- Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment;

  - NOTE:Larger inter-band MCL observed in TR 37.941 will cause less REFSENS degradation compared with the legacy 30dBc MCL assumption;

- Step 2: Discuss whether we need to align the requirement for BS type 1-H and BS type 1-O;

Conclusions

In this contribution, we want to share some initial views on Rel-20 co-location requirement improvement and proposals/observations are made as following:

Proposal 1: In Rel-20 BS RF enhancement, propose not to consider the FR1 unlicensed band for co-location requirement improvement.

Proposal 2: In Rel-20 BS RF enhancement, propose to continue the investigation on the rest of co-located cases which are not covered in Rel-19.

- Proposal 3: For Rel-20 NR BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption which is used to mimic 30dBc coupling loss.

- Observation 1: For OTA transmitter OFF power improvement, don’t see too much implementation burden on the existing requirement except for the measurement for conformance testing.

Proposal 4: For OTA transmitter OFF power in Rel-20, we propose to consider following aspects:

- Step 1: to identify more appropriate MCL1 between two BSs according to the legacy deployment assumption from single operator and relevant BS’s antenna configuration to derive the OTA core requirement(no intention to change the conducted core requirements according to the WID objective), then check whether TRP based OFF power measurement in the OTA chamber is feasible or not.

- Step 2: If TRP based OFF power measurement in step 1 is not feasible, then to define more appropriate/relevant co-location reference antenna for the OTA requirement improvement and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement.

Observation 2: For OTA transmitter intermodulation requirement, if NR BS is implemented with circulator for protection of external interference signal, IMD impacts due to transmitter intermodulation requirement should be quite impacted.

Proposal 5: For OTA transmitter intermodulation requirement in Rel-20, we propose to consider following aspects:

- Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment.

- Step 2: Discuss whether the transmitter intermodulation requirement for band n79 and n104 except for band n77/n78 could be optionally removed from regions which don’t mandate the Tx intermodulation testing.

Proposal 6: For OTA transmitter spurious co-location emission in Rel-20, we propose to consider following aspects:

- Step 1: To identify more appropriate MCL1 between co-located BS from different bands, then use the following equation to derive the corresponding TRP based transmitter spurious co-location emission requirement and check its feasibility of TRP based measurement in the OTA chamber firstly.

- = -174dBm/Hz + 5dB + 50dB－7dB + MCL1+9dB (scaling factor)

- Step 2: If TRP based OTA transmitter spurious co-location emission in step 1 is not feasible, to discuss more appropriate co-location reference antenna and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement.

Proposal 7: For OTA Receiver out-of-band blocking in Rel-20, we propose to consider following aspects:

- Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment.

  - NOTE:Larger inter-band MCL observed in TR 37.941 will cause less REFSENS degradation compared with the legacy 30dBc MCL assumption;

- Step 2: Discuss whether we need to align the requirement for BS type 1-H and BS type 1-O.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- RP-252905, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, ZTE Corporation.

- RP-252586, CR to TR 37.941: Rel-19 CLTA-related agreements, Approved.

- R4-2409402, Co-location requirements background and discussion, Huawei.

- R4-2504594

Annex

Table 6.4.4.2-1: Intra band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical Separation | SA-to-SA | M: 54 | M: 53 | M: 56 |  |
| Horizontal Separation | SA-to-SA | M: 31 | M: 34 | M: 46 |  |
|  | A-to-SA |  | M: 35 S: 55±15 | M: 51 M: 69 | M: 55 S: 70±15 |
|  | A-to-A |  |  |  | S: 77±15 |
| M: measurement results S: simulation results. |  |  |  |  |  |

Table 6.4.4.2-2: Inter band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |
| --- | --- | --- |
|  |  | 3.5 GHz to 7 GHz |
| Horizontal separation | A-to-SA | S: 57±10 |
|  | A-to-A | S: 54±10 |
| M: measurement results S: simulation results. |  |  |