3GPP TSG-RAN WG4 Meeting #116bis	R4-2514282

Prague, Czech Republic, October 13 – 17, 2025

Source:	Nokia

Title:	NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC

Agenda item:	4.14.2

Document for:	Discussion

Introduction

The work on Rel-19 NR demodulation performance Phase 5: BS Demodulation MMSE-IRC has been concluded in RAN4#116 with the resulting requirements to appear in the newest version of the corresponding Rel-19 specifications. While the SNR requirements have been captured without any square brackets in the specifications, during the maintenance phase, RAN4 still allows companies to make further simulation results alignments and further discussions on the results as documented in [1] below:

- Wayfoward: - For this meeting, use the latest SNR values for requirement with all [ ] removed. - Companies can further align simulation results in the maintenance part. - Further discuss the solution in the maintenance part if the results are not well-aligned.

In addition to the above-mentioned way forward, [1] captured also early observations from few companies on the simulation results collected in RAN4#116, which is documented in [2].

Prior to RAN4#116bis, the draft of the newest version of the specifications were made available at 3GPP portal. As there are few inconsistencies, draftCRs need to be submitted in RAN4#116bis to make proper corrections.

This contribution provides our brief observations on the simulation results collected in RAN4#116 and brings the attention of participating companies to improve the quality of the Rel-19 BS MMSE IRC requirements in the respective specifications.

Discussion

The topics of discussions in this Section are simulation results alignment and corrections to the specifications. In the following, they are discussed in each of the respective subsections.

Simulation results alignment

The collected simulation results in [2] shows some cases of the ideal simulation results with span more than 2 dB. Specifically, for the 8Rx case, from 16 cases, all of them have span more than 2 dB, ranging from the minimum of 2.49 dB to the maximum of 6.42 dB. One of the observations documented in [1] is as follows:

Regarding companies may still use different MMSE-IRC implementation, our simulation results shared in the last meeting show that: for 2Rx and 4Rx, there is no performance difference if companies use different MMSE-IRC implementation. Therefore at least for 2Rx and 4Rx, we do not think that will be the reason if the results are not aligned.

Referring to such an observation from the observer’s own simulation results, different MMSE-IRC implementations are expected to show performance differences in 8Rx case. This is somehow reflected by the collected simulation results in [2] for 8Rx, in which all 16 cases have span more than 2 dB.

In Rel-19 BS MMSE IRC, the Interference-to-Noise-Ratio (INR) is derived from the Dominant-Interference-Proportion (DIP) from LTE, which was a result from system-level simulations from a group of contributing companies as documented in TR 36.884 [3]. Interestingly, if one would consider only the results of that group of companies which have contributed to TR 36.884 [3], the results for 8Rx are quite aligned with span less than 2 dB for all the 16 cases. Nonetheless, RAN4 shall take into account all simulation results from all contributing companies and, hence, if alignment is not possible, need to find the outliers to be removed to have a span less than 2 dB when defining the SNR requirements.

It is unknown prior to RAN4#116bis whether there are companies which will bring updated simulation results for alignment in RAN4#116bis. In case there are, but alignment is still not reached or there is no company updating the results, the way forward from RAN4#116 can be reused again as the way forward from RAN4#116bis. Anyway, in RAN4#116bis, companies could only bring draftCR to be endorsed, hence, RAN4 can keep the discussion about the simulation results alignment open until RAN4#117 when formal CR is allowed to be submitted for agreement in RAN4.

If simulation alignment could not be reached in RAN4#116bis, RAN4 to keep the way forward from RAN4#116 and continue the discussions in RAN4#117 before making final conclusions on the SNR requirements.

Corrections to the specifications

Prior to RAN4#116bis meeting, the drafts of the updated specifications were made available. Reviewing the current state of Rel-19 BS MMSE-IRC requirements in 38.104, 38.141-1 and 38.141-2, alignment across them and essential corrections are still needed. The following draftCRs are making essential corrections and submitted for endorsement in RAN4#116bis, namely, [4] for TS 38.104, [5] for 38.141-1 and [6] for 38.141-2.

There are few issues in the draft of the related specifications for Rel-19 BS MMSE-IRC shared at 3GPP portal prior to RAN4#116bis meeting. Hence, essential corrections need to be made for the corresponding specifications.

Conclusion

In this contribution, we provide our views on Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC. The following Observations and Proposals were made:

Proposal 1: If simulation alignment could not be reached in RAN4#116bis, RAN4 to keep the way forward from RAN4#116 and continue the discussions in RAN4#117 before making final conclusions on the SNR requirements.

Observation 1: There are few issues in the draft of the related specifications for Rel-19 BS MMSE-IRC shared at 3GPP portal prior to RAN4#116bis meeting. Hence, essential corrections need to be made for the corresponding specifications.

- References

- R4-2512606, Way Forward for [116][321] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#116, China Telecom, Bengaluru, India, August 25-29, 2025

- R4-2509405, Simulation results summary for BS requirement with MMSE_IRC receiver_v5_Samsung, 3GPP RAN4#116, Samsung, Bengaluru, India, August 25-29, 2025

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, V13.1.0, 09-2016

- R4-2514283, DraftCR on 38.104 for Rel-19 BS MMSE-IRC, 3GPP RAN4#116bis, Nokia, Prague, Czech Republic, October 13-27, 2025

- R4-2514284, DraftCR on 38.141-1 for Rel-19 BS MMSE-IRC3GPP RAN4#116bis, Nokia, Prague, Czech Republic, October 13-27, 2025

- R4-2514285, DraftCR on 38.141-2 for Rel-19 BS MMSE-IRC3GPP RAN4#116bis, Nokia, Prague, Czech Republic, October 13-27, 2025