# 3GPP TSG RAN WG4 Meeting #116	bis 														R4-2514301

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.17.2.2

Source: 	Ericsson

Title:	RRM performance requirements for CSI-RS based L1 measurements

Document for:	Discussion

# Introduction

In this contribution, we will address several aspects regarding the test case design for CSI-RS based L1 measurement requirement.

# Discussion

## CSI-RS based L1 measurement test cases

In last meeting following WF is agreed.

  - For CSI-RS based L1 RSRP measurement reporting:

    - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

    - For UE supporting event triggered reporting, UE only needs to pass 2)

    - For UE not supporting event triggered reporting, UE only needs to pass 1)

For CSI-RS-based L1-RSRP, we have agreed not to specify requirements involving a measurement gap and RTD > CP.

As discussed in the last meeting, if the UE supports both periodic, semi-persistent CSI-RS L1-RSRP for LTM and event-triggered L1-RSRP for LTM, we consider it sufficient to test the event-triggered reporting. This is because event-triggered reporting requires the UE to perform more frequent measurements and meet more stringent requirements compared to periodic or semi-persistent reporting.

Hence, to reduce the number of test cases and the testing burden on the UE, we suggest:

- Testing only the event-triggered L1-RSRP test cases for CSI-RS.

- If the UE does not support event-triggered reporting for CSI-RS, then it must pass the periodic CSI-RS-based L1-RSRP test cases.

For event-triggered test cases for CSI-RS-based L1-RSRP for LTM, we propose the following list of test cases based on the Rel-18 LTM test cases.

- We recommend that RAN4 agree on the following list of test cases for CSI-RS-based L1-RSRP.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

- To reduce the number of test cases and the testing burden on the UE, we suggest:

- Testing only the event-triggered L1-RSRP test cases for CSI-RS.

- If the UE does not support event-triggered reporting for CSI-RS, then it must pass the periodic CSI-RS-based L1-RSRP test cases.

# Summary

The following have been proposed in the current contribution:

- We recommend that RAN4 agree on the following list of test cases for event triggered CSI-RS-based L1-RSRP.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

- To reduce the number of test cases and the testing burden on the UE, we suggest:

- Testing only the event-triggered L1-RSRP test cases for CSI-RS.

- If the UE does not support event-triggered reporting for CSI-RS, then it must pass the periodic CSI-RS-based L1-RSRP test cases.

# References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.