3GPP TSG RAN WG4 Meeting #116	bis 														R4-2514303

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.10.3

Source: 	Ericsson

Title:	Discussion on Rel-20 MIMO RRM requirements

Document for:	Decision

# Introduction

In RP-252936, WID for MIMO phase 6 is agreed. RAN4 objective is copied here for reference.

RAN4:

Specify necessary core requirements for the enhancements listed above.

In this contribution we analyze the potential RAN4 RRM impacts for NR MIMO Phase 6.

# Discussion

From the WID, we can observe that RAN4 scope depends on the RAN1 enhancements introduced for each objective. In this contribution, we look at each objective and the corresponding RAN4 impact based on the RAN1 agreements in the below sections.

## Objective 1: Enhancing UL capacity and coverage

Objective is copied below for reference.

On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

There are two sub-objectives, outlined below with a high-level overview of their intent:

- Enabling multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop, specifically for RB-level partial frequency sounding.

- Enabling cross-slot SRS resource usage between the end of a U slot and the beginning of an S slot, within a single SRS resource set.

For the first sub-objective, we anticipate the main impact to lie within the RF specifications, particularly in relation to discussions on phase coherency and phase continuity across frequency hops. These aspects may require updates to ensure consistent phase behavior between frequency hops.

From the RRM specification perspective, transmission requirements for multi-slot SRS, cross-slot SRS, or frequency hopping of the SRS are currently not defined. The RRM spec instead focuses on interruption requirements associated with SRS antenna switching or SRS carrier switching. Since both sub-objectives operate on the same carrier and do not involve antenna switching, we believe there is no impact on RRM requirements.

- Objective 1 (enhancing UL coverage and capacity) do not have RRM requirements impact.

## Objective 2: Enhancing DL CSI acquisition (Early CSI acquisition)

Objective is copied below for reference.

On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

To analyze the RRM requirements impact, we first look at few RAN1 agreements.

Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support at least aperiodic SRS-AS transmission triggered via MSG4 of 4-Step RACH. Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support aperiodic CSI reporting triggered via MSG4 of 4-Step RACH based on the followings: - The aperiodic CSI reporting is transmitted on PUSCH. - Support at least aperiodic CSI-RS for CSI associated with the aperiodic CSI reporting - Support PMI-based reporting with wideband PMI based on Rel-15 Type-I SP codebook and wideband CQI - FFS: Which report quantity(s) can be configured - Support PMI-free reporting with wideband CQI FFS: Which report quantity can be configured

Based on recent RAN1 agreements, the UE supports at least aperiodic transmission of both SRS and CSI reporting during the transition from IDLE or INACTIVE to CONNECTED mode. These transmissions are triggered via MSG4 of the 4-Step RACH procedure. Aperiodic SRS helps with downlink CSI acquisition, while aperiodic CSI reporting is sent on PUSCH and depends on associated aperiodic CSI-RS for channel state acquisition. CSI reporting can be done in two ways: PMI-based reporting, which uses wideband PMI from the Rel-15 Type-I Single Panel codebook along with wideband CQI, and PMI-free reporting, which includes only wideband CQI.

In addition, RAN1 has agreed on a four-step timeline for CSI acquisition. Same is copied below for reference.

Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3 - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting Note: The term “capability” above does not mean legacy RRC based UE capability. Note: The term “capability” above does not mean legacy RRC based UE capability.

Although PMI and CQI reporting requirements are not typically defined by RAN4 RRM, it is important for RAN1 to consider UE transmission and reception limitations during certain stages of the CSI acquisition process. To support this, RAN4 should share its understanding with RAN1. Specifically, RAN4 assumes that the UE can only acquire CSI after fine timing synchronization is completed. This assumption is key to ensuring that CSI-RS transmission from the network aligns with the UE’s ability to perform CSI measurements during connection setup.

IDLE to connected mode:

Figure 1: RRC IDLE to connected state transition

We begin by presenting the UE transition timeline from IDLE to CONNECTED mode as defined in TS 38.300, illustrated in Figure 1. Based on our understanding, during steps 2 and 2a of this timeline, the UE may be unable to transmit or receive for a certain duration. This limitation arises from the processing of the RRCSetup message, which involves configuring baseband and RF components for access to the target cell and applying the parameters contained in the setup message.

Unless a new mechanism is introduced in RAN1, this processing time results in a temporary inactivity period during which the UE cannot perform transmission or reception. Therefore, we propose that RAN4 first confirm whether the UE can transmit or receive during the RRC setup delay. If the UE cannot, RAN4 should inform RAN1 that, from the moment the UE receives the RRCSetup message, it is unable to transmit or receive on the cell until the setup processing is complete. This clarification is essential for accurately modeling UE behavior and enabling timely CSI-RS transmission from the network, particularly in scenarios where early CSI acquisition relies on aperiodic CSI-RS.

- RAN4 shall first confirm whether the UE can transmit or receive during the RRC setup delay. If the UE cannot, then RAN4 should clarify to RAN1 through LS that, as per the existing procedure, UE cannot transmit/receive from the RRCSetup reception until RRCSetup processing delay is complete. RRCSetup processing delay as per TS 38.331 is 10ms.

Inactive to connected mode:

Figure 2: RRC inactive to Connected transition

We begin by presenting the UE transition timeline from Inactive to CONNECTED mode as defined in TS 38.300, illustrated in Figure 2. Based on our understanding, during steps 4 and 5 of these timelines, the UE may be unable to transmit or receive for a certain duration. This limitation arises from the processing of the RRCResume message, which involves configuring baseband and RF components for access to the target cell and applying the parameters contained in the setup message.

Unless a new mechanism is introduced in RAN1, this processing time results in a temporary inactivity period during which the UE cannot perform transmission or reception. Therefore, we propose that RAN4 first confirm whether the UE can transmit or receive during the RRC resume delay. If the UE cannot, RAN4 should inform RAN1 that, from the moment the UE receives the RRCResume message, it is unable to transmit or receive it on the cell until the setup processing is complete. This clarification is essential for accurately modeling UE behavior and enabling timely CSI-RS transmission from the network, particularly in scenarios where early CSI acquisition relies on aperiodic CSI-RS.

Furthermore, as per our understanding, the network can begin scheduling data from step 5 onward. Therefore, if CSI acquisition is expected to occur before step 5, adjustments to the current timeline may be needed to support such early CSI reporting.

- RAN4 shall first confirm whether the UE can transmit or receive during the RRC resume delay. If the UE cannot, then RAN4 should clarify to RAN1 that, as per the existing procedure, UE cannot transmit/receive from the RRCResume reception until RRCResume processing delay is complete. RRCResume processing delay as per TS 38.331 is 6 or 10ms based on different scenarios.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- Objective 1 (enhancing UL coverage and capacity) do not have RRM requirements impact.

- RAN4 shall first confirm whether the UE can transmit or receive during the RRC setup delay. If the UE cannot, then RAN4 should clarify to RAN1 through LS that, as per the existing procedure, UE cannot transmit/receive from the RRCSetup reception until RRCSetup processing delay is complete. RRCSetup processing delay as per TS 38.331 is 10ms.

- RAN4 shall first confirm whether the UE can transmit or receive during the RRC resume delay. If the UE cannot, then RAN4 should clarify to RAN1 that, as per the existing procedure, UE cannot transmit/receive from the RRCResume reception until RRCResume processing delay is complete. RRCResume processing delay as per TS 38.331 is 6 or 10ms based on different scenarios.

# References

[1] 	RP-234007, New WID: NR MIMO Phase 5