3GPP TSG-RAN WG4 Meeting # 116-bis					R4-2514365

Prague,Czech Republic,  October 13 - 17, 2025

Source:		Ericsson

Title:			A-IoT BS conformance overview

Agenda item:		6.14.2

Document for:		Approval

# Introduction

In this paper, we present our over views on Rel-19 A-IoT conformance testing.

# Discussion

A-IoT feature is specified in new core requirement and many aspects on testing cannot reused from legacy NR 38.141-1 and therefore, these new aspects needs to be agreed before drafting the CR on new test specification.

For the test configuration, it is used to reduce the test load for different permutation of the contiguous/non-contiguous allocation, MC and multiband. As only the single band and single carrier operation is specified in 38.194 for downlink, therefore, for transmitter test, single carrier applies.  For receiver test, as FDM is specified in RAN1, it is possible to configure different frequency shift for multiple carrier and this aspect can refer to NB-IoT MC testing.

test configuration for MC operation can be specified.

### TC1: A-IoT multi-carrier operation

The purpose of the TC1 is to test A-IoT multi-carrier aspects.

#### TC1 generation

TC1 is constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum Base Station RF Bandwidth.

-	For receiver test, place a A-IoT carrier at the upper edge and a A-IoT carrier at the lower Base Station RF Bandwidth edge.

For the test signal to be used for test configuration, we propose below and choose 200kHz for band n8.

Table 1:Signal to be used to build A-IoT TCs

| Operating Band characteristics |  | Operation band specified in clause 5.2 |
| --- | --- | --- |
| TC signal | BWchannel | 200kHz |
| characteristics | Subcarrier spacing | 15 kHz |

discuss the above test signal construction for TC1 .

For the test configurations for the MC operation

| BS test case | MC capable BS |
| --- | --- |
| 6.2 Base station output power | SC |
| 6.3 Transmit ON/OFF power | SC |
| 6.4 Transmitted signal quality | SC |
| 6.6 Unwanted emissions | SC |
| 7.2 Reference sensitivity level | SC |
| 7.3 In-channel selectivity and blocking | TC1 |
| 7.4 Out-of-band blocking | TC1 |
| 7.5 Receiver spurious emissions | TC1 |
| 7.6 Receiver intermodulation | TC1 |

discuss the above applicability of test configurations.

For test model, a new one should be specified as R2D is new signal design.  The A-IoT test signal can be designed with the same OOK reference model specified at device and illustrated in Table 2.

Table 2: R2D test model configuration

| Reference channel | Configuration | Parameter value |
| --- | --- | --- |
| SIP | SCS | 15 |
|  | PRB | 1 |
|  | Bit length | 8 |
|  | M_SIP | 4 |
|  | OFDM | 2 |
| CAP | Bit length | 4 |
|  | M | 2 |
|  | OFDM | 2 |
| PRDCH | TBS | 96 |
|  | CRC | 16 |
|  | Line encoding |  |
|  | M | 2 |
| postamble | Bit length | 4 |
|  | M | 6 |
|  | chip number except for SIP,padding | 228 |
|  | Padding | 6 |

discuss the above signal configuration for test signal model.

Additionally, the MU discussion in annex C (Test tolerances and derivation of test requirements) or the new test case of timing mask should be discussed as this is a new test case compared to legacy. For the uncertainty of the timing mask, in A5.3.4.2	in TS 51.010-1 specifies both the uncertainty of power level and uncertainty of the time measurement. For power level measurement uncertainty can be referred to the other test (e.g 0.7 dB for < 3Ghz for 6.2 BS output power), for time measurement uncertainty, the 1/8 symbol time is reused and symbol refereeing to the OOK symbol (i.e one Tc chip time).

Discuss the 1/8 symbol time as the MU for timing mask requirement.

For the receiver test, a new measurement system set-up should be discussed as the CWT signal should be present for receiver test.  One possible system set-up is to place a dedicated signal generator for the CW signal generation and the phase noise profile of CW signal generation should follow the 38.194 if the BS does not declare the phase noise cancellation.

The receiver measurement system set-up should be updated considering the CWT signal injection and discuss if the dedicated CW signal can be generated with specified phase noise profile.

# Conclusions

In this contribution, we present our overview on the A-IoT conformance testing with below observation and proposals:

Proposal-1: test configuration for MC operation can be specified.

Proposal-2: discuss the above test signal construction for TC1 .

Proposal-3: discuss the above applicability of test configurations.

Proposal-4: discuss the above signal configuration for test signal model.

Proposal-5: Discuss the 1/8 symbol time as the MU for timing mask requirement.

Proposal-6: The receiver measurement system set-up should be updated considering the CWT signal injection and discuss if the dedicated CW signal can be generated with specified phase noise profile.

# References

- TS 38.194

- TS 51.010-1