3GPP TSG-RAN WG4 Meeting # 116-bis					R4-2514369

Prague,Czech Republic,  October 13 - 17, 2025

Source:		Ericsson

Title:			A-IoT coexisting simulation assumptions

Agenda item:		7.11.3

Document for:		Approval

# Introduction

In this paper, we present our views on Rel-20 A-IoT coexisting simulation assumptions.

# Discussion

In SID[1], some coexisting related objectives are quoted below:

Objectives

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]:

- Deployment scenario 4 with topology 1 with no external carrier wave.

- Traffic types DO-DTT, DT, DO-A.

- Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command).

- Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs.

- FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.

- Assumption of the same air interface for Device 2b and Device C.

  - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769

RAN1-led

- Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios

  - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios

- Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C.

- While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming

  - Maximum distance between Reader and Device of 50-500 m

  - Maximum transmit power between -3 dBm and +5 dBm for device C

    - RAN1 to recommend feasible values within this range

The scenario 4 with topology 1 in TR 38.848 is to be simulated.

A-IoT BS co-sites with NR macro BS and A-IoT device FR1 bands usage as the same as NR UE

The maximum distance between Reader and device is 500 m

Device C transmission power between -3 dBm and 5 dBm

The topology 1 in TR 38.848 is quoted below, though the A-IoT can be received at a different BS with a BS transmitting DL signal, such is not a worst case for the coexisting and therefore may not be considered in coexisting simulation.

Figure 4.2.1.1-1 [TR 38.848]: Topology 1

In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a basestation. The communication between the basestation and the ambient IoT device includes Ambient IoT data and/or signalling. This topology includes the possibility that the BS transmitting to the Ambient IoT device is a different from the BS receiving from the Ambient IoT device.

In RAN4 coexisting simulation assumptions in Rel-19 SI, the outdoor macro NR base coexisting simulation parameter is agreed. This can be modified to reflect the Rel-20 SI objectives.

Table 6.6.2.1-1. Deployment parameters for Toplogy 1

| Parameter | Topology 1 |
| --- | --- |
| Carrier frequency | 900 MHz (Band n8), 2GHz |
| BW for NR | 10MHz with 15KHz SCS |
| BW for A-IOT system | 180kHz, for 15kHz SCS |
| Waveform (R2D) | OOK waveform generated by OFDM modulator |
| A-IoT DL power control | No |
| A-IoT UL power control | Yes |
| Traffic model | Full buffer |
| Frequency reuse | 1 |
| NR BS deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 A-IOT indoor scenario per sector NR BS height: Uma, 25m |
| A-IoT reader deployment | Same with NR BS |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed 1 active A-IoT device under one reader at one drop |
| NR UE dropping | For NR UE, uniformly distributed outdoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| NR BS Inter-site distance | 750 m |
| Minimum NR BS – NR UE distance (2D) | 35 m |
| Pathloss model | PL= PLb+PLtw+PLin+N(0, σP2) - PLin = 0.5 * d2D-in where d2D-in is the distance to nearest factory/office boundary on the line between Tx and Rx point. Set maximum value of d2D-in as [25m] as optional |
|  | PLb : NLOS and LOS in TR38.901 NR BS – NR UE: Uma Outdoor NR UE – A-IoT reader/ device: Uma Device – A-IoT reader: Uma |

Table 6.6.2.2-1: NR BS RF parameters

| NR BS Parameter | Values for evaluation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:15 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |
| ACLR | Option 1: 30dBc, Option 2: 17dBc |

Table 2: A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | 43 dBm |
| A-IoT reader receiver Noise Figure (dB) | 5 |
| A-IoT reader antenna gain (dBi) | 6 |
| ACLR | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB) |
| ACS | Same as legacy NR BS |
| Antenna configuration | Same with the BS |

Table 6.6.2.3-1: NR UE RF parameters

| NR UE Parameter | Values for evaluation |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

Table 6.6.2.3-2: A-IoT device RF parameters

| A-IoT device parameters | Device 2b | Device C |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | 0 |
| A-IoT device transmission power | {-10, -20} dBm | {-3, 0, 5} dBm |
| SNR | One value should be used in coexisting | One value should be used in coexisting |
| A-IoT device noise figure (dB) | 15 | 13 |
| Antenna configuration | Omni direction antenna | Omni direction antenna |
| ACLR | {30, 25, 20} | {30, 25, 20} |
| ACS | {28, 20 } | {28,20} |

In RAN1 agreements, the different scenario is considered for system level evaluations, i.e Uma, RMa and UMi. The Uma and UMi cell ISD is less than 500m but the RMa ISD is 1732m/5000m, which exceeds the maximum Reader to device distance of 500m and should not be considered in coexisting study.

Agreement

For Rel-20 study, the following path-loss models are used in the coverage evaluation:

- UMa NLOS defined in TR38.901

- RMa NLOS defined in TR38.901

  - Note: companies can also report to use RMa LOS defined in TR38.901

- UMi NLOS defined in TR38.901

- Note: this does not have implication on the coverage target (if any)

Table 7.2-1: Evaluation parameters for UMi-street canyon and UMa scenarios

| Parameters |  | UMi - street canyon | UMa |
| --- | --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = 200 m) | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500 m, see note 1) |
| BS antenna height hBS |  | 10 m | 25 m |
| UT location | Outdoor/indoor | Outdoor and indoor | Outdoor and indoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Height hUT | Same as 3D-UMi in TR36.873 | Same as 3D-UMa in TR36.873 |
| Indoor UT ratio |  | 80% | 80% |
| UT mobility (horizontal plane only) |  | 3 km/h | 3 km/h |
| Min. BS - UT distance (2D) |  | 10 m | 35 m |
| UT distribution (horizontal) |  | Uniform | Uniform |
| NOTE 1: UMa scenarios with ISDs between 200-500m can be used for evaluations. |  |  |  |

UMa should be simulated, Umi to be discussed and RMa should be excluded in coexisting simulation

As the co-siting of A-IoT BS and NR BS may not be a worst case as the network shift between A-IoT layer and NR layer is zero.  RAN4 can discuss if there is a case where A-IoT BS and NR BS not co-site between two operator.

Discuss whether there is a case for non-zero network shift between A-IoT layer and NR layer.

In RAN1 agreement, there is also the CFO/SFO error assumptions, such RF imperfection is not considered directly in legacy coexisting simulation and we think such can be factored in the SNR consideration when RF imperfection is made.

CFO/SFO imperfection should be considered in the SNR consideration.

Agreement

For Rel-20 study, SFO and CFO in LLS assumption for evaluation purpose is from the followings:

- Device C is smaller than Device 2b

|  | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| SFO after clock sync/calibration | 10^2 (O), 10^3 (M) ppm | [10] ppm |
| Initial CFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| CFO after clock sync/calibration | 100 (M), 200 (O), 50 (O) ppm | [10] ppm |

In Rel-19 coexisting study, the SNR degradation is used due to no waveform design available. Considering the D2R and R2D waveform available from the Rel-19, RAN4 can use the SNR instead of SNR degradation and use the detection performance instead of the throughput in Rel-20 coexisting.

Use the detection performance as coexisting metric instead of the SNR degradation in Rel-20.

The SNR for R2D and D2R link in coexisting needs to be simulated and if possible, align the SNR numbers to be used in coexisting simulation to reach certain performance.

For R2D, the simulation assumption for LLS simulation is listed in Table 1 below. The CFO and SFO impact is not small when CFO is 10ppm for R2D link and this is observed in Figure 1.

Table 1: Simulation assumption for R2D

| Parameters​ | Assumptions​ |
| --- | --- |
| Carrier frequency​ | 900 MHz​ |
| SCS​ | 15 KHz​ |
| Channel-model​ | TDLC300​ |
| Device velocity​ | 3 km/h​ |
| Number of Tx/Rx for device​ | 1​ |
| Number of Tx/Rx for BS​ | 2​ |
| Transmission bandwidth​ | 180 KHz​ |
| Adjacent carrier interference​ | No​ |
| ED bandwidth ​ | With considering CFO​ |
| Line coding​ | Manchester​ |
| ADC bits​ | 4​ |
| Device 2b/c​ | ZIF​ |
| Sampling rate​ | 1.92 Msps​ |
| CFO​ | After calibration(ppm): 100[2b], 10[c]​ |
| SFO​ | After calibration(ppm): 1000[2b], 10[c]​ |
| R2D Message size ​ | 20, 96, 400​ |
| Repetition​ | 1, 2 ,4​ |
| FEC​ | No, 1/3 TBCC​ |

Additional simulation parameter additional selected for evaluation in listed in Table 2 for R2D:

Table 2: selected parameter for evaluation

| Parameters​ | Assumptions​ |  |  |
| --- | --- | --- | --- |
| Msg Info​ | 96 bits​ |  |  |
| Device​ | Ideal​ | 2b​ | c​ |
|  | CFO: 0 ppm​ SFO: 0 ppm​ | CFO: 100 ppm​ SFO : 1000 ppm​ | CFO: 10 ppm​ SFO: 10 ppm​ |
| M​ | [1,2,6,12,24,32]​ |  |  |
| Number of PRBs​ | According to M in Table 3 |  |  |
| Sampling rate​ | 1.92 Msamples/scond​ |  |  |
| Device architecture​ | ZIF​ |  |  |

Table 3: M value and # of PRB relation

| M value​ | Minimum number of PRBs​ |
| --- | --- |
| 1​ | 1​ |
| 2​ | 1​ |
| 6​ | 1​ |
| 12​ | 2​ |
| 24​ | 4​ |
| 32​ | 6​ |

For device b, frequency selective fading channel TDLC300 the SNR is list in

Table 4: SNR for device b with different M value

| ​ | M=1​ | M=2​ | M=6​ | M=12​ |
| --- | --- | --- | --- | --- |
| BW = 1 PRB​ | 11.76​ | 12.46​ | 20.88​ | -​ |
| BW = 2 PRB​ | 8.47​ | 9.07​ | 15.92​ | NaN​ |
| BW = 3 PRB​ | 6.77​ | 7.09​ | 13.15​ | NaN​ |
| BW = 4 PRB​ | 5.4​ | 5.67​ | 10.54​ | NaN​ |
| BW = 5 PRB​ | 4.38​ | 4.47​ | 8.54​ | NaN​ |
| BW = 6 PRB​ | 3.47​ | 3.38​ | 7.21​ | 27.65​ |

For device C,  frequency selective fading channel TDLC300 the SNR is list in

Table 5: SNR for device c with different M value

| ​ | M=1​ | M=2​ | M=6​ | M=12​ |
| --- | --- | --- | --- | --- |
| BW = 1 PRB​ | 10.3​ | 10.54​ | 18.69​ | -​ |
| BW = 2 PRB​ | 7.77​ | 7.46​ | 13.56​ | NaN​ |
| BW = 3 PRB​ | 6.1​ | 5.62​ | 10.27​ | 20.84​ |
| BW = 4 PRB​ | 5.11​ | 4.31​ | 8.06​ | 14.1​ |
| BW = 5 PRB​ | 4.05​ | 3.15​ | 6.67​ | 11.3​ |
| BW = 6 PRB​ | 3.18​ | 2.42​ | 5.66​ | 9.49​ |

Figure 1: CFO/SFO impact on SNR of R2D

D2R LLS simulation assumption is listed in Table 6.

Table 6: D2R simulation assumption

| Parameters | Value |
| --- | --- |
| Carrier frequency | 900MHz |
| subcarrier spacing (SCS) | 15kHz |
| Channel model | AWGN, TDL_A, TDL_D |
| Delay spread | 30ns, 150ns |
| Device velocity | 3km/h |
| Message size | 16 bits, 96 bits, 400 bits |
| BLER target | 1%, 10% |
| Message size | 16 bits, 96 bits, 400 bits |
| Waveform and modulation | OOK/FSK/BPSK |
| Number of antenns in UE | 1 |
| Number if antennas in BS | 2 or 4 |
| FEC | No FEC, CC |
| Line code | Manchester, no line coding |
| D2R receiver | Coherent receiver, Non-coherent receiver |
| Number of slots in each simulation | 10000 |
| Simulation times | 10 |
| Slots time | 1ms |
| Transmission bandwidth | 180kHz |

The simulation result is listed in Table 7.

Table 7: SNR for D2R LLS simulation

| Device type | Data rate | FEC [N0, ½,1/4] | Rep. [No, 2, 4] | BS TxRU [2, 4] | Msg size [20,96,400] | 10% BLER | 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Device 2b | 7 Kbps | No | No | 2 | 20 | 10.03 | 17.24 |
| Device 2b | 7 Kbps | No | No | 2 | 96 | 11.55 | 20.46 |
| Device 2b | 7 Kbps | No | No | 2 | 400 | NaN | NaN |
| Device C | 7 Kbps | No | No | 2 | 20 | 8.69 | 15.92 |
| Device C | 7 Kbps | No | No | 2 | 96 | 10.22 | 19.13 |
| Device C | 7 Kbps | No | No | 2 | 400 | 13.64 | NaN |
| Device 2b | 7 Kbps | Yes (1/3) | No | 2 | 20 | 5.52 | 11.84 |
| Device 2b | 7 Kbps | Yes (1/3) | No | 2 | 96 | NaN | NaN |
| Device 2b | 7 Kbps | Yes (1/3) | No | 2 | 400 | NaN | NaN |
| Device C | 7 Kbps | Yes (1/3) | No | 2 | 20 | 4.23 | 11.48 |
| Device C | 7 Kbps | Yes (1/3) | No | 2 | 96 | 6.57 | 13.52 |
| Device C | 7 Kbps | Yes (1/3) | No | 2 | 400 | NaN | NaN |

Discuss the simulation assumption of LLS for R2D and D2R to align the SNR value used in coexisting study.

# Conclusions

In this contribution, we present our overview on the A-IoT coexisting simulation assumptions with below observation and proposals:

Observation 1 The scenario 4 with topology 1 in TR 38.848 is to be simulated.

Observation 2 A-IoT BS co-sites with NR macro BS and A-IoT device FR1 bands usage as the same as NR UE

Observation 3 The maximum distance between Reader and device is 500 m

Observation 4 Device C transmission power between -3 dBm and 5 dBm

Proposal-1: UMa should be simulated, Umi to be discussed and RMa should be excluded in coexisting simulation

Proposal-2:Discuss whether there is a case for non-zero network shift between A-IoT layer and NR layer.

Proposal-3: CFO/SFO imperfection should be considered in the SNR consideration.

Proposal-4: Use the detection performance as coexisting metric instead of the SNR degradation in Rel-20.

Proposal-5: Discuss the simulation assumption of LLS for R2D and D2R to align the SNR value used in coexisting study.

# References

- RP-251884, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices