3GPP TSG-RAN WG4 Meeting #116bis	R4-2514383

# Prague, Czech Republic, 13th – 17th October 2025

Agenda item:		7.6.3

Source: 		Qualcomm Inc.

Title:	 	NB-IoT NTN PC1.5 requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements for NR NTN and IoT NTN phase 2 has been approved [1]. One of the objectives is PC1.5 requirements for NB-IoT. In this contribution related UE RF requirements are discussed.

## 2	Discussion

2.1 Component power handling capability

Ideally, High power UE requirements should maximize the possibility for high power UE transmissions. However, also UE RF component feasibility needs to be considered. In real UE implementations, it is highly possible that UE could support e.g. NR NTN and IoT NTN and use the same signal path for both. Given the differences in NR and NB-NTN waveforms, it may be possible that Tx chain is used to support e.g. PC2 NR and PC1.5 NB-IoT. However, components may not otherwise be able to tolerate higher average power. This can create instances where UE needs to protect it’s components.

Even though NB-IoT is HD-FDD, RAN1 specifications allow up to 256ms contiguous UL transmission, after which UE has to stop to receive DL

Observation 1: It is possible to use the same signal path to support e.g. NR NTN PC2 and NB-IoT NTN PC1.5.

Observation 2: Up to 256ms contiguous UL transmission is possible for NB-IoT.

To enable re-use of same technologies and components, it would be beneficial not to mandate UE to follow higher than 25% duty cycle for PC1.5. Unlike the TN solution, this should be optional: UEs with higher power handling capability can transmit longer times.

Proposal 1: Allow but do not mandate PC1.5 NB-IoT NTN UE to reduce transmit power to PC3 power level when 25% Tx duty cycle in a radio frame is exceeded.

2.2 Tx requirements

When PC2 and PC1 requirements were defined, it was assumed that PC2 NB-IoT could be supported with a PC3 NR PA and PC1 NB-IoT could be supported with PC2 NR PA. In these discussions low post-PA loss even down to 2 dB were discussed, as captured e.g. in [2]. In real UE implementations, it is highly possible that UE could support e.g. NR NTN and IoT NTN and use the same signal path for both, meaning the signal path can include e.g. duplexer. Therefore, post PA loss may be higher. It is suggested to consider 4 to 5 dB post-PA loss in MPR evaluations.

Proposal 3: Consider 4 to 5 dB post-PA loss in MPR evaluations

Proposal 4: Consider NR PC2 PA as reference for defining NB-IoT PC1.5 requirements.

Co-existence simulations done during rel-19 covered power classes up to PC1. As a result, both PC2 and PC1 have the same UTRA ACLR of 37 dBc defined. It would be logical to apply the same ACLR requirement also for PC1.5.

Proposal 5: Define 37 dBc UTRA ACLR for NB-IoT PC1.5, aligned with PC2 and PC1 requirements.

2.3 RSD

The work item [1] has an objective to specify RSD requirements. As NB-IoT is half-duplex system, no RSD requirements need to be specified.

Proposal 6: There is no need for RSD requirements for NB-IoT due to the HD-FDD operation.

2.4 RAN2 impact

Currently, there is no signalling in LTE to indicate support for PC1.5. This needs to be added in this WI.

Preferably support for this new power class is early implementable, while details on the exact signalling method are up to RAN2.

Proposal 7: Signalling to indicate support for PC1.5 needs to be added to LTE RRC. Signalling should be early implementable.

## 3	Conclusion

In this contribution UE RF requirements for NB-IoT NTN for PC1.5 were discussed. Following observations and proposals were made.

2.1 Component power handling capability

Observation 1: It is possible to use the same signal path to support e.g. NR NTN PC2 and NB-IoT NTN PC1.5.

Observation 2: Up to 256ms contiguous UL transmission is possible for NB-IoT.

Proposal 1: Allow but do not mandate PC1.5 NB-IoT NTN UE to reduce transmit power to PC3 power level when 25% Tx duty cycle in a radio frame is exceeded.

2.2 Other Tx requirements

Proposal 3: Consider 4 to 5 dB post-PA loss in MPR evaluations

Proposal 4: Consider NR PC2 PA as reference for defining NB-IoT PC1.5 requirements.

Proposal 5: Define 37 dBc UTRA ACLR for NB-IoT PC1.5, aligned with PC2 and PC1 requirements.

2.3 RSD

Proposal 6: There is no need for RSD requirements for NB-IoT due to the HD-FDD operation.

2.4 RAN2 impact

Proposal 7: Signalling to indicate support for PC1.5 needs to be added to LTE RRC. Signalling should be early implementable.

## 4	References

[1] 	RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

[2]	R4-2505114, WF on HPUE RF requirements for IoT NTN, Vivo