3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2514408

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Proposal on 1UL band 2CC and 2UL band 3CC HPUE MSD

Source: 	Skyworks Solutions, Inc.

Agenda Item:	5.2.6	RAN4 PRD

Document for:	Approval

# 1	Background

In RAN#116, we submitted a contribution [1] that proposed HPUE MSDs for 1UL band 2CC and 2UL band 3CC configurations. Since this could not be discussed in detail, we re-submit this contribution for discussion under the RAN4 PRD topic. We also further elaborated on the 1UL band 2CC MSD for the even IMDs.

# Discussion

## 2.1 Proposal for one UL band 2CC case

The MSD for the one UL band 2CC cases (UL CA_nXX class B/C or 2A) is related to:

- The odd order IMDs of the 1RB+1RB allocation in each CC for victim bands in the proximity of the UL band with:

  - For TDD bands: IMD3 to IMD9 for class B or C and IMD3 to IMD7 for class 2A considering MPR

  - For FDD bands, higher order odd IMDs may be considered, especially when the two bands are in the same band group.

- The IMD4 and IMD6 for victim bands in proximity of the 2nd harmonic ranges of the UL band

- The IMD5 for victim bands in proximity of the 3rd harmonic ranges of the UL band

- IMD4 of class 2A falling into low-band DL for bands with >300MHz BW (NR bands n77/n78/n79)

Since the IMD3 and IMD5 depend on applying MPR to meet SEM mask for TDD bands, their level is independent of the power class. This is also the case for any IMDs with an order higher than 3 since they decay in a similar or faster way compared to PC3.

For TDD bands it is reasonable to assume that HPUE MSDs caused by odd-orders IMDs from a single UL band with 2CC have the same value as PC3.

For even orders however, their power is proportional to the transmitted power, but accounting for MPR needed for IMD3 and/or IMD5 to meet -30dBm/MHz. For the 1RB+1RB outer2 allocations used for the single-band 2CC test points, compared to PC3:

- PC2 1Tx sees a 0.5dB higher MPR and PC2 2Tx sees a 1.5dB higher MPR

- PC1.5 2Tx sees a 3.5dB higher MPR.

- To simplify, we can assume that PC2 and PC1.5, 2CC band power increases by 2dB and 3dB versus PC3 respectively:

- IMD4 increases by 8dB for PC2 and 12dB for PC1.5. These can be mapped to ΔMSDmax= 9dB and 12dB from the 2UL MSD LUT, respectively

- IMD6 increases by 12dB for PC2 and 18dB for PC1.5. These can be mapped to ΔMSDmax= 12dB and 18dB from the from the 2UL MSD LUT, respectively.

- Note that for non-contiguous intra-band ULCA case, the MPR is larger than what is considered above for contiguous intra-band ULCA which is the only case where the BB IMD4 overlaps with low-bands.

Proposal on HPUE one UL band 2CC MSD:

- For TDD UL bands:

- For odd IMDs, the PC2 and PC1.5 HPUE MSD is equal to PC3 MSD: ΔMSD(dB)=0

- For even IMDs:

  - IMD4 MSD is mapped to ΔMSDmax= 9dB and 12dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

  - IMD6 MSD is mapped to ΔMSDmax= 12dB and 18dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

- For FDD UL bands, the HPUE MSD is studied case-by-case.

- A CR will be proposed at the next meeting to correct legacy related HPUE MSDs accordingly

- Companies are encouraged to use this guideline for their HPUE TPs/CRs.

- RAN4 discusses whether, if agreed, such rules should only be captured to the PRD or added to the MSD simplification in the TS.

## 2.2 Proposal for two UL band 3CC case

The MSD for the two UL band 3CC cases (UL CA_nXXA-nYYB/C) is related to the triple beat (IMD3) between the 1CC band and the 2CC band allocations.

For PC2 inter-band HPUE the configuration goes from 20dBm+20dBm for PC3 to 23dBm+23dBm. Thus, one could assume that the interference follows the IMD3 9dB ΔMSDmax from the 2UL MSD simplification LUT. However, to meet emissions for IMD3 and IMD5, MPR is allowed, thus reducing the power for the band with 2 UL CCs. The same happens for the PC1.5 26+26dBm case.

It is unlikely that HPUE intra-band contiguous UL CA, which is the only type considered in triple beat cases, will meet SEM without some backoff applied (but not necessarily the full amount allowed by the specification), the amount of backoff increase between PC3 and PC2 or PC3 and PC1.5 is likely to follow what is defined for MPR.

For the 1RB+1RB outer2 allocations used for the triple beat test points, compared to PC3:

- PC2 1Tx sees a 0.5dB higher MPR and PC2 2Tx sees a 1.5dB higher MPR

- PC1.5 2Tx sees a 3.5dB higher MPR.

To target the same simplification for the 2Tx and 3Tx cases as for the two UL band inter-band 2CC case, we propose to use 1dB MPR for PC2 cases and 3dB for the PC1.5 case, meaning that:

- For PC2:

  - 2CC band power increases by 2dB versus PC3

  - 1CC band power increases by 3dB versus PC3

  - Thus, triple beat interference power increases by 2*3+1*2 =8dB => PC2 triple beat ΔMSDmax is 8dB and can be simplified to 9dB to map on existing LUT.

- For PC1.5:

  - 2CC band power increases by 3dB versus PC3

  - 1CC band power increases by 6dB versus PC3

  - Thus, triple beat interference power increases by 2*6+1*3 =15dB => PC2 triple beat ΔMSDmax is 15dB.

Proposal on HPUE two UL band 3CC MSD:

- PC2 2UL band 3CC triple beat (IMD3) is mapped to ΔMSDmax= 9dB 2UL ΔMSD LUT

- PC1.5 2UL band 3CC triple beat (IMD3) is mapped to ΔMSDmax= 15dB 2UL ΔMSD LUT

- A CR will be proposed at the next meeting to correct legacy related HPUE MSDs accordingly

- Companies are encouraged to use these guidelines for their HPUE TPs/CRs.

- RAN4 discusses whether, if agreed, such rules should only be captured to the PRD or added to the MSD simplification in the TS.

# Conclusion

In this contribution, we analysed the HPUE delta MSDs for the one UL band 2CC IMDs and two UL band 3CC triple beat PC2 and PC1.5 cases and make the following proposals.

Proposal on HPUE one UL band 2CC MSD:

- For TDD UL bands:

- For odd IMDs, the HPUE MSD is equal to PC3: PC2 and PC1.5 ΔMSD(dB)=0

- For even IMDs:

  - IMD4 MSD is mapped to ΔMSDmax= 9dB and 12dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

  - IMD6 MSD is mapped to ΔMSDmax= 12dB and 18dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

- For FDD UL bands, the HPUE MSD is studied case-by-case.

- A CR will be proposed at the next meeting to correct legacy related HPUE MSDs accordingly

- Companies are encouraged to use this guideline for their HPUE TPs/CRs.

- RAN4 discusses whether, if agreed, such rules should only be captured to the PRD or added to the MSD simplification in the TS.

Proposal on HPUE two UL band 3CC MSD:

- PC2 2UL band 3CC triple beat (IMD3) is mapped to ΔMSDmax= 9dB 2UL ΔMSD LUT

- PC1.5 2UL band 3CC triple beat (IMD3) is mapped to ΔMSDmax= 15dB 2UL ΔMSD LUT

- A CR will be proposed at the next meeting to correct legacy related HPUE MSDs accordingly

- Companies are encouraged to use these guidelines for their HPUE TPs/CRs.

- RAN4 discusses whether, if agreed, such rules should only be captured to the PRD or added to the MSD simplification in the TS.

# References

[1] R4-2509759 Proposal on 1UL band 2CC and 2UL band 3CC HPUE MSD, Skyworks Solutions Inc., RAN4#116