3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514415

Prague, CZ, October 13th – 17th 2025

Agenda Item:	4.19.2

Source:	Ericsson

Title:	Discussion on ICASL and FR2-1 in-band blocking

Document for:	Approval

# 1	Introduction

In this contribution, we present our view on the necessity of the additional 3dB relaxation of in-channel adjacent subband leakage for band n104, and the derivation of FR2-1 in-band blocking requirement for SBFD-capable BS.

# 2	In-channel adjacent sub-band leakage

##### Issue 4-2-2: In-channel adjacent subband leakage for FR1 WA/MR BS, for DU/UD case Agreement: - Absolute limit for in-channel adjacent subband leakage for DU/UD case - Reuse legacy ACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1) for bands: Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit BS category / BS class; ACLR absolute basic limit. Category A Wide Area BS; -13 dBm/MHz. Category B Wide Area BS; -15 dBm/MHz. Medium Range BS; -25 dBm/MHz. Local Area BS; -32 dBm/MHz. - The necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104 shall be studied in maintenance of Rel-19 SBFD WI

In RAN4#116, the core part of Rel-19 SBFD BS RF work was concluded. However, there are still some remaining issues to be discussed during the maintenance phase, one of which is to study the necessity of the additional 3dB relaxation of in-channel adjacent subband leakage for band n104.

It is worth to mention that, the derivation of the in-channel adjacent subband blocking requirement is based on an 80 dB isolation between sectors. For in-channel adjacent subband leakage, considering the reference sensitivity at the UL subband receiver as -115 dBm/MHz, and assuming 80 dB isolation between sectors, the required emission limit at the DL subband transmitter should be -35 dBm/MHz. The agreed emission limit is already significantly less stringent than the required level and should not be further relaxed.

Furthermore, the 9 dB relaxation for legacy TDD BS was justified that the requirements for AAS should not be more stringent than those for non-AAS. However, for the in-channel adjacent subband leakage requirement, this implies that the SBFD BS could handle 9 dB more interference between sectors. Unfortunately, how this same 9 dB relaxation can be applied to the new requirement for SBFD BS was not thoroughly discussed before the agreement. Therefore, it is not reasonable to have 3 dB more relaxation on top of that.

- Considering the reference sensitivity at the UL subband receiver as -115 dBm/MHz, and assuming 80 dB isolation between sectors, the required emission limit at the DL subband transmitter should be -35 dBm/MHz.

- It was not thoroughly discussed and is a bit unclear if the same 9 dB relaxation as for legacy ACLR absolute limit can be reused for in-channel adjacent subband leakage.

RAN4 do not introduce 3 dB more relaxation of in-channel adjacent subband leakage for band n104.

# 3	In-band blocking for FR2-1 WA SBFD BS

### Issue 3: In-band blocking value for FR2-1 WA BS - Agreement - Adopt -44 dBm as a provisional requirement. The value and the methodology to derive it shall be revisited in maintenance.

In RAN4#116, the core part of Rel-19 SBFD BS RF work was concluded. However, there are still some remaining issues to be discussed during the maintenance phase, one of which is to revisit the requirement derivation of FR2-1 in-band blocking requirement for SBFD-capable BS.

It is worth mentioning that, depending on the hypothesis on the beamforming architecture, whether analog or digital/hybrid (no subarray is considered in simulation assumptions from SI Rel-18), the assumption of whether the Rx beamforming gain should be included or not in the in-band blocking interference received at the BS may change.

In addition, during previous meetings, two approaches were proposed by companies: the absolute value-based approach and the ratio-based approach. Regarding the ratio-based approach adopted for FR2 OTA in-band blocking requirements in TS 38.104, it is derived based on a ratio of the in-band blocking power to the declared EISrefsens and considering 6 dB relaxation. An approach based on simulations to derive the in-band blocking power/wanted signal, where the wanted signal is derived based on simulations, would lack of a fixed declared reference value, which is instead required by the specification. More background information can be found in TR 38.817-02 [1]. The blocking results are derived from co-existence simulations where UEs are the interfering source, and the blocking offset of 33 dB considers interference originating from UEs, rather than from neighbouring BSs, and is therefore not applicable to BS-to-BS blocking scenarios.

10.5.3.3 FR2 BS type 2-O has a number of differences when considering the OTA blocking levels. - There are no conducted requirements, so simulation of conducted interferer power levels do not give a final OTA power level. - Beam forming is necessary in order to overcome the path loss. - A wide range of implementations with varying antenna maximum beam forming gain are envisaged. - Different beam forming architectures result in different statistical spread of interferer power at the active Rx input (i.e. the LNA). Traditionally the in-band blocking level has been analysis on a statistical basis based on the 99.99% probability of an interferer being possible. This has been used since UTRA where WCDMA modulation was susceptible to blocking and the entire system would be blocked if such an event occurred. The NR OFDMA scheme does not suffer so greatly due to a blocking event hence such a high probability is not required, probabilities between 99% and 99.9% have also been considered. In the past the wanted signal has not been considered when studying the interferer level, however with an OTA requirement and a beam forming system when considering a statistical worst case, it is important to consider both the wanted and the interfering signal. This is due to the fact that blocking arises when a low power wanted signal and a high-power blocking signal occur simultaneously (the likelihood of this occurring depends on instantaneous power control, scheduling and beam directions) Hence the difference between the wanted signal and the interferer is also important. Initially the probability of the interferer alone was simulated looking at the same scenarios identified in the co-existence simulation in 3GPP TR 38.803 [24]. The probability of the blocking signal level and also the probability of simultaneous low wanted signal and high blocking signal were both considered. Simulations of both of these aspects suggested that the blocking signal is typically 33dB above the reference sensitivity level. This result was consistent when considering different architectures. It was agreed that the specification for the interferer power level will be 33dB higher than the OTA REFSENS power level. As for FR2 there is only a single sensitivity requirement, a 6dB offset from OTA reference sensitivity is used for the wanted signal and a 33dB offset from reference sensitivity is used for the interferer.

The methodology to derive the in-band blocking requirements should be clearly described, including the details about where the received in-band blocking power is measured and if the Rx beamforming gain is included or not in that measurement.

In our view, the methodology considers the absolute value of the total wideband power received in the UL of the SBFD BS, from the adjacent channel, should be adopted to derive the in-band blocking power requirement. In our simulation, we consider a coexistence scenario where FR2-1 urban macro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 10% and 5%.

Two different values of BS TRP are considered 1) 30 dBm, which is the default value in the assumptions of the SI; 2) 40 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 1.

Figure 1: FR2-1 UMa: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 1 it can be observed that the in-band blocking requirement is higher as the TRP increases and the grid-shift decreases. It can be observed that when considering TRP 40 dBm and grid-shift 5%, we are around 35 dB above the currently specified in-band blocking requirement (-58 dBm) for FR2-1 Wide Area BS. The results in Figure 1 are obtained without including the Rx beamforming gain in the interference received from the adjacent channel.

- The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

- The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

- For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Adopt the absolute value-based approach for requirement derivation for FR2-1 in-band blocking. Define in-band blocking interferer signal power to -23 dBm for FR2-1 Wide Area BSs in SBFD networks.

# Conclusion

In this contribution, overview of the necessity of the additional 3dB relaxation of in-channel adjacent subband leakage for band n104, and the requirement derivation for FR2-1 in-band blocking for SBFD-capable BS is presented with the following observations and proposals.

Observation 1	Considering the reference sensitivity at the UL subband receiver as -115 dBm/MHz, and assuming 80 dB isolation between sectors, the required emission limit at the DL subband transmitter should be -35 dBm/MHz.

Observation 2	It was not thoroughly discussed and is a bit unclear if the same 9 dB relaxation as for legacy ACLR absolute limit can be reused for in-channel adjacent subband leakage.

Observation 3	The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

Observation 4	The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

Observation 5	For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 do not introduce 3 dB more relaxation of in-channel adjacent subband leakage for band n104.

Proposal 2	The methodology to derive the in-band blocking requirements should be clearly described, including the details about where the received in-band blocking power is measured and if the Rx beamforming gain is included or not in that measurement.

Proposal 3	Adopt the absolute value-based approach for requirement derivation for FR2-1 in-band blocking. Define in-band blocking interferer signal power to -23 dBm for FR2-1 Wide Area BSs in SBFD networks.

# References

TR 38.817-02, General aspects for Base Station (BS) Radio Frequency (RF) for NR