3GPP TSG-RAN WG4 Meeting #116bis	R4-2514796

# Prague, Czech Republic, Oct. 13-17, 2025

Title: 	WF on NR_ATG_enh_demod

Agenda Item:	6.7.1

Source: 	CMCC

Document for:	Approval

<Topic 1> DL CA

Issue 1-1: TDD pattern

Agreement:

- Introduce PDSCH test cases of TDD pattern 30D4S6U configuration with following restrictions:

  - Only test the default HARQ mode, which is applicable for UE not supporting outOfOrderOperationDL-r16.

  - Introduce the applicability rule that if UE pass the new TDD pattern test, the legacy TDD pattern test can be skipped.

Issue 1-2: HARQ configuration for TDD pattern 30D4S6U in CA

Agreement:

- Following the number of HARQ process and K1 values

Test parameters for number of HARQ processes for ATG CA requirements

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 13 |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 13 | N/A |

Test parameters for K1 values for ATG CA requirements

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | 2 if mod(i,40) = {1,4,5,9,10,14,15,19,20,24,25,28,29} |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 33 if mod(i,40) = 1 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 | N/A |

Issue 1-3: CA configuration selection rules

Agreement:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Issue 1-4: Test applicability rules

Agreement:

- Agree on Proposal 1 for legacy TDD pattern tests and Proposal 2 for 30D4S6U TDD pattern tests

- Merge the two proposals when drafting the CR.

  - Proposal 1: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

  - Proposal 2: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

<Topic 2> DL MIMO

Issue 2-1: TDD pattern

Agreement:

- Introduce PDSCH test cases of TDD pattern 30D4S6U configuration with following restrictions:

  - Only test the default HARQ mode, which can be applied for all ATG UE.

  - Introduce the applicability rule that if UE pass the new TDD pattern test, the legacy TDD pattern test can be skipped.

Issue 2-2: MCS

Agreement:

- 64QAM MCS 22

- 256QAM Table 2 MCS 24

Annex

- Simulation assumption for DL CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD TDD: 7D1S2U and 30D4S6U Note: The same simulation results can be assumed for two TDD patterns |
| Subcarrier spacing |  |  | FDD: 15kHz TDD: 30kHz |
| CHBW |  |  | FDD:5,10,15,20,25,30,35,40,45,50MHz TDD: 5,10,15,20,25,30,40,50,60,80,90,100MHz |
| MCS |  |  | 64QAM MCS22 |
| Antenna configuration |  |  | 2T2R and 2T4R |
| Propagation condition |  |  | FDD: AWGN+220Hz TDD: AWGN+500Hz |
| Rank |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Receiver type |  |  | MMSE-IRC |

- Simulation assumption for DL MIMO

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD TDD: 7D1S2U and 30D4S6U Note: The same simulation results can be assumed for two TDD patterns |
| Subcarrier spacing |  |  | FDD: 15kHz TDD: 30kHz |
| CHBW |  |  | FDD:10MHz TDD:40MHz |
| MCS |  |  | 64QAM MCS 22 256QAM Table 2 MCS 24 |
| Antenna configuration |  |  | 2T2R and 2T4R with XP-low |
| Propagation condition |  |  | FDD: AWGN+220Hz TDD: AWGN+500Hz |
| Rank |  |  | 2 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | DMRS port |  | {1000},{1001} |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Receiver type |  |  | MMSE-IRC |

- Simulation assumption for UL MIMO

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15kHz SCS: N/A 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| CHBW |  | FDD: 5MHz TDD:10MHz |
| MCS |  | 64QAM Table 1 MCS 20 |
| Antenna configuration |  | 2T2R with X-pol low correlation |
| Propagation conditions |  | FDD 15kHz SCS: H= 1j1−j+200Hz TDD 30kHz SCS: H= 1j1−j+500Hz |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0},{1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL pattern, e.g., 30D4S6U, S=14G for 30kHz SCS. |  |  |

Annex.B

| Spec | Clauses and Title | Notes | Company |
| --- | --- | --- | --- |
| 38.101-4 | 5.1.1.7.8 (new) Applicability and test rules for ATG | For CA feature, including Applicability and test rules Selection of CA configurations Applicability between different TDD pattern if introduced | CMCC |
|  | 5.1.1.12 Applicability of requirements for ATG | For DL-MIMO feature UE with Cross Polarized Antennas HARQ related if 30D introduced | CMCC |
|  | 5.2.2.1.22, 5.2.2.2.23, 5.2.3.1.21, 5.2.3.2.22 Minimum requirements for ATG | For DL-MIMO feature, including the test requirements | Ericsson |
|  | 5.2A.2.7, 5.2A.3.6 (new) Minimum requirements for ATG CA | For CA feature, at least including: Test purpose Test parameters HARQ and K1 related configuration if TDD pattern 30D is introduced Test requirements | ZTE |
|  | A.3.2.1.1, A.3.2.2.2 Reference measurement channels | For CA feature and DL-MIMO feature | CMCC |
| 38.104 | 8.2.14 Requirements for PUSCH for ATG |  | Samsung |
|  | A.5 Fixed Reference Channels for performance requirements (64QAM, R=567/1024) |  | Samsung |
|  | G.1 Static propagation condition | The static propagation condition model from TS38.101-4 can be taken as the reference Capture the static channel matrix in spec | ZTE |
|  | G.2.3.2.3 MIMO Correlation Matrices using cross polarized antennas |  | Nokia |
| 38.141-1 | 8.1.2.11 Applicability of PUSCH performance requirements for ATG scenario |  | CMCC |
|  | 8.2.14 Performance requirements for PUSCH for ATG |  | Ericsson |
|  | A.5 Fixed Reference Channels for performance requirements (64QAM, R=567/1024) |  | Ericsson |
|  | G.1 Static propagation condition |  | ZTE |
|  | G.2.3.2.3 MIMO correlation matrices using cross polarized antennas |  | Nokia |
| 38.141-2 | 8.1.2.11 Applicability of ATG PUSCH performance requirements |  | ZTE |
|  | 8.2.14 Performance requirements for PUSCH for ATG |  | Ericsson |
|  | A.5 Fixed Reference Channels for performance requirements (64QAM, R=567/1024) |  | Ericsson |
|  | J.1 Static propagation condition |  | CMCC |
|  | J.2.3.2.3 MIMO correlation matrices using cross polarized antennas |  | Nokia |