3GPP TSG-RAN WG4 Meeting #116bis                                                                       R4-2514618

Prague, Czech Republic, Oct 13 – 17, 2025

Agenda item:			8.1

Source:	Feature lead (Huawei, HiSilicon)

Title:	AGREEMENT for 6GR system parameter

Document for:	Approval

# Topic #1: Waveform

## Framework study for waveform

- Agreement:

  - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1

  - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects

    - Tx assumption including PA model

    - Related RF requirements which should be taken into consideration

    - Both UL and DL are considered

    - Take the candidate waveforms identified/discussed in RAN1 as the baseline

    - Identify the evaluation metric, e.g. net gain

    - Implementation constraints

  - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs

    - Evaluation cases could be selected step-by-step, based on RAN1 progress

    - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

## PA model

- Agreement

  - Consider PA modelling at least for the following aspects for RAN4 discussion

    - Memory effects for UE supporting larger CBW

    - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3)

    - Calibration conditions

    - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc.

    - PA models for different device types, handheld UE is prioritized

    - Applicable requirements

      - 5G-A requirements as starting point

    - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

# Topic #2: Modulation

- Agreement

  - RAN4 evaluation work focus on the feasibility study and RF requirements impact

  - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1

    - Identify the main affected requirements for modulation evaluations

      - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions.

    - Align on the evaluation assumptions

    - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation

    - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed.

    - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1.

  - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations

    - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with  RAN1 studies

    - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

# Topic #3: Channel bandwidth

### Sub-topic 3-1: Max Channel Bandwidth

- AGREEMENT

In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths.

  - RF performance evaluation with proposed max CBW

    - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation

    - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements

    - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers

    - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum

    - [Study the need and impact of specifying large CBW, e.g. from operator spectrum perspective]

    - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum.

    - Assess the implementation feasibility and complexity and power consumption

    - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1.

  - Study the feasibility to support different NW and UE max CBW.

  - Collaborate with RAN1 on feasibility findings

    - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations.

### Sub-topic 3-2: Min Channel Bandwidth

- AGREEMENT

  - Compare options of defining min CBW, considering pros and cons

  - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress

    - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands

    - SCS-dependent framework

    - Other aspects are not precluded

  - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

### Sub-topic 3-3: FFT size

- AGREEMENT

  - Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges

  - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

### Sub-topic 3-4: Numerology

- AGREEMENT

  - Evaluate the following proposals regarding numerology from RAN4 perspective

    - "Single numerology" proposal

    - Frequency sub-range/Band specific SCS values proposal

      - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band

      - Study numerology for SSB of initial cell search from RAN4 perspective

    - Other proposals not presented in this meeting are not precluded

  - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

### Sub-topic 3-5: Spectrum utilization

- AGREEMENT

  - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM).

    - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G

  - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled

    - Considering trade-offs between SU, RF performance, and UE/BS complexity

    - Channel bandwidth and SCS with smaller SU should be prioritized

    - SU for larger channel bandwidth shall be evaluated based on progress on CBW

    - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

### Sub-topic 3-6: Asymmetric channel bandwidths

- AGREEMENT

  - Study the need and feasibility to enable asymmetric CBW

# Topic #4: Channel arrangement

### Sub-topic 4-1: Channel raster

- AGREEMENT

  - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands

    - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster

  - Investigate the interaction between the channel raster and the synchronization raster

  - Investigate the necessity of channel raster or alternative ways for the channel configuration

    - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster

    - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported

  - Study the listed main proposals especially for the migration and co-existence approaches

    - Other options not presented in this meeting are not precluded

  - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

### Sub-topic 4-2: Sync raster

- AGREEMENT

  - Evaluation on sync raster from RAN4 perspective:

    - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4

    - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving

  - Collaboration and planning:

    - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset.

    - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

### Sub-topic 4-3: Channel spacing

- AGREEMENT

  - Further study in RAN4 regarding channel spacing

# Topic #5: Irregular channel bandwidth

- AGREEMENT

  - 5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G

    - The listed main proposals and identified issues should be taken into account for the following study of irregular/flexible/scalable channel bandwidths

    - Other options not presented in this meeting are not precluded

# Topic #6: Number of Tx and Rx

- AGREEMENT

  - Study the framework of spec impact with different number of Tx/Rx

  - Evaluate number of Tx/Rx from both a performance and implementation complexity perspective.

    - E.g. device size (e.g. foldable smartphone)/form factors constraints, antenna design

  - Co-ordinate with RAN and RAN1 for number of Tx/Rx

# Topic #7: Device types

- AGREEMENT

  - The following aspects could be further studied

    - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range.

      - Study on the NW impact should also be considered

    - Study how to better support variety devices from RAN4 perspective

      - E.g. differentiate devices, use cases, features

    - Other aspects are not precluded

  - Collaborate on the framework with other WGs:

    - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues