3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2520095

Dallas, US, 17th –21th Nov, 2025

Source: 	CATT

Title: 	Discussion on TN BS RF

Agenda item:	8.4.1

Document for:	Discussion

- Introduction

In RAN4#116bis meeting, 6GR BS RF and co-existence began to be discussed and the WF is captured in [1]. In this contribution, we focus on TN BS RF issues for 6GR.

- Discussion

Below is the TN BS RF requirements related content in [1].

- BS RF requirements - Issue 1-1-1: Starting point Agreement: 5G BS RF requirements should be considered as starting point when defining the 6G BS RF requirements. - Issue 1-1-2: Requirements to be re-assessed Way Forward: For next meeting, companies are encouraged to evaluate how 5G BS RF requirements should be considered for 6G and provide their view using the table in Annex A as example. Note: The goal is to identify: - 5G requirements that might also be applicable to 6G. - 5G requirements which will need to be re-evaluated based on 6G system parameters but would not need a further study. - 5G requirements which should be further studied in the scope of the 6G SI, a prioritization might be made in next meeting. - Requirements for cmWave bands - Issue 1-3-3: Rx Requirements Agreement: For above 7 GHz, further study how BS Rx requirements should be specified, using FR2 approach or not, including if blocking should be specified based on FR2 methodology.

Below table captures our discussions and views on each BS RF requirements, our analysis is based on around 7GHz, for other frequency range there may be some changes.

Table 1 BS RF requirements for 7GHz

| 5G BS RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G (SU, channel BW, …), following TR 38.817-02 principles/formula | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Conducted requirements |  |  |  |
| Base station output power | X |  |  |
| Output power dynamics |  |  |  |
| RE power control dynamic range |  |  | X, based on EVM and ACLR/UEM requirements |
| Total power dynamic range |  | X, based on channel bandwidth and SU |  |
| Transmit ON/OFF |  |  | X, may have new transit period in new frequency range |
| Transmitted signal quality |  |  |  |
| Frequency error | X |  |  |
| Modulation quality |  | X, for potential new modulation order |  |
| Time alignment error | X |  |  |
| Unwanted emissions |  |  |  |
| Occupied bandwidth | X, may not needed unless regulator requires |  |  |
| ACLR |  |  | X, based on co-existence study an new channel bandwidth |
| Operating Band Unwanted Emissions |  |  | X, based on ACLR change |
| Transmitter spurious emission |  |  |  |
| General requirement | X |  |  |
| Protection of BS receiver of own or different BS | X |  |  |
| Additional spurious (coex) |  |  | X, based on R19/20 output |
| Co-location |  |  | X, based on R19/20 output |
| Transmitter intermodulation |  |  | X, based on R19/20 output |
| Receiver sensitivity level |  | X, based on reqired SNR |  |
| Dynamic range |  |  | X, based on system simulation results |
| In-band selectivity and blocking |  |  |  |
| ACS |  |  | X, based on co-existence study and maximum channel bandwidth |
| In-band blocking |  |  | X, based on system simulation results |
| Out of band blocking | X |  |  |
| Receiver spurious emissions | X |  |  |
| Receiver intermodulation |  |  | X, based on system simulation results |
| In channel selectivity |  |  | X, based on system simulation results |
| Radiated requirements |  |  |  |
| Radiated transmit power | X |  |  |
| OTA base station output power | X |  |  |
| OTA output power dynamics |  |  |  |
| OTA RE power control dynamic range |  |  | X, based on EVM and ACLR/UEM requirements |
| OTA Total power dynamic range |  | X, based on channel bandwidth and SU |  |
| OTA transmit ON/OFF |  |  | X, may have new transit period in new frequency range |
| OTA transmitted signal quality |  |  |  |
| OTA Frequency error | X |  |  |
| OTA Modulation quality |  | X, for potential new modulation order |  |
| OTA Time alignment error | X |  |  |
| OTA Unwanted emissions |  |  |  |
| OTA Occupied bandwidth | X, may not needed unless regulator requires |  |  |
| OTA ACLR |  |  | X, based on co-existence study an new channel bandwidth |
| OTA Operating Band Unwanted Emissions |  |  | X, based on ACLR change |
| OTA Transmitter spurious emission |  |  |  |
| OTA General requirement | X |  |  |
| OTA Protection of BS receiver of own or different BS | X |  |  |
| OTA Additional spurious (coex) |  |  | X, based on R19/20 output |
| OTA Co-location |  |  | X, based on R19/20 output |
| OTA transmitter intermodulation |  |  | X, based on R19/20 output |
| OTA sensitivity |  |  |  |
| OTA receiver sensitivity level |  | X, based on reqired SNR |  |
| OTA dynamic range |  |  | X, based on system simulation results |
| OTA in-band selectivity and blocking |  |  |  |
| OTA ACS |  |  | X, based on co-existence study and maximum channel bandwidth |
| OTA In-band blocking |  |  | X, based on system simulation results |
| OTA out of band blocking | X |  |  |
| OTA receiver spurious emissions | X |  |  |
| OTA receiver intermodulation |  |  | X, based on system simulation results |
| OTA in channel selectivity |  |  | X, based on system simulation results |

- Conclusion

This contribution provides TN BS RF requirements analysis in table 1.

- References

- R4-2514638 WF on 6G BS RF and co-existence BS RF requirements