3GPP TSG-RAN WG4 Meeting #117	R4-2520114

# Dallas, US, 17th – 21st Nov, 2025

Title: 	Discussion on performance requirements for RedCap UEs with NTN

Source: 	CATT

Agenda item:	6.19.3.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the perf RRM requirements and work split for perf has been done [1]. In this contribution, we continue to discuss and provide our views on the performance requirements for RedCap UEs with NTN.

- Discussion

  - Test configuration and parameters

In the last meeting, RAN4 discussed test configuration and parameters for RedCap UEs with NTN, and whether to follow the test principle in NOTE2 case by case for different core requirements need to be further discussed in RAN4. The agreement is copied as follow [1]:

Online agreement < Agreement >: - Both FDD and HD-FDD duplex mode should be considered in test configuration for 1Rx and 2Rx (e)RedCap UE with FR1-NTN. - 15 kHz SCS and 10MHz bandwidth can be configured for 1Rx and 2Rx (e)RedCap UE with FR1-NTN in test cases. - Both GSO and NGSO are configured in test cases for 1Rx and 2Rx RedCap UE with FR1-NTN. - If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. According to the above, the following table is just a reference for 1Rx and 2Rx RedCap UE with FR1-NTN for cell reselection requirements. Table 1: Supported test configurations Configuration; Description. 1; GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW. 2; NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW. 3; GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW. 4; NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW. NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both.. - RAN4 need to further discuss whether to follow the test principle in NOTE2 case by case for different core requirements. - For the agreed test configuration, the existing RMC related parameters, Io values, SSB configuration and SMTC configurations defined for NTN can be reused for RedCap UE with FR1-NTN. - RAN4 NOT to consider NCD-SSB related configuration and RedCap specific UL/DL BWP configuration in test cases for (e)RedCap UE with NTN.

In core requirements, the following requirements were defined for HD-FDD  (e)RedCap UE with NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Random access

- SA: RRC Connection Release with Redirection

- Requirements for CSI-RS based RLM/BFD/CBD

- Minimum requirement for L1 indication for RLM and BFD

- Scheduling availability of UE during RLM/BFD/CBD

- Scheduling availability of UE performing intra/inter measurements

- CSI-RS based L1-RSRP Reporting

- Scheduling availability of UE during L1-RSRP measurement

- UE Rx-Tx time difference measurements

Among the above core requirements, the HD-FDD related requirements applicability rules for yellow highlighted part are related the new collision handling approach defined in R19 RAN1, so the related requirements for HD-FDD (e)RedCap UE with NTN need to tested. Therefore, we propose the test principle in Note 2 at least can apply for all of test cases except for the requirements highlighted.

Observation 1: Among the above core requirements, the HD-FDD related requirements applicability rules for yellow highlighted part are related the new collision handling approach defined in R19 RAN1.

Proposal 1: The test principle in Note 2 at least can apply for all of test cases except for the following requirements:

- Requirements for CSI-RS based RLM/BFD/CBD

- Minimum requirement for L1 indication for RLM and BFD

- Scheduling availability of UE during RLM/BFD/CBD

- CSI-RS based L1-RSRP Reporting

- Scheduling availability of UE during L1-RSRP measurement

- UE Rx-Tx time difference measurements

In addition, for other configurations, the existing configurations defined in 1Rx RedCap test cases can be reused for (e)RedCap UE with NTN including:

- Dedicated CORESET Reference Channel for FDD

- Aggregation level(CCE) for RLM and BFD

- Correlation Matrix and Antenna Configuration

Proposal 2: The existing configurations defined in 1Rx RedCap test cases can be reused for (e)RedCap UE with NTN including:

- Dedicated CORESET Reference Channel for FDD

- Aggregation level(CCE) for RLM and BFD

- Correlation Matrix and Antenna Configuration

  - Test scope

In the last meeting, RAN4 agreed the test case list for (e)RedCap UE with NTN, there were still FFS for some test cases and test configurations, and the list was captured in [1]. The FFS parts are listed as below:

4.2E Cell Re-selection for NR RedCap UE with Satellite Access - FFS: Location-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for 1Rx/2Rx RedCap UE with NTN - FFS: Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements - FFS: Time-based measurement initiation to FR1 inter-frequency cell reselection for 1Rx/2Rx RedCap UE with NTN 6.1F.2 NR SAN Conditional Handover - FFS: Inter-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN 8.1E Radio Link Monitoring for RedCap UE with Satellite Access - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode for 1Rx/2Rx RedCap UE with NTN

In our view, since the agreed test cases have been included a variety of configurations, for reducing the test efforts, we support the above FFS test cases can be removed for RedCap UE with NTN.

For the following test cases agreed in the last meeting [1], for reducing the test efforts, we think that the followings can also be removed.

9.3E NR inter-frequency measurements for Redcap UEs with satellite access - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for 1Rx/2Rx RedCap UE with NTN 9.5E L1-RSRP measurements for Reporting for RedCap UE with satellite access - SSB based L1-RSRP measurement for satellite access when DRX is used for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement for satellite access when DRX is not used for 1Rx/2Rx RedCap UE with NTN

Proposal 3: In the test cases list, the above FFS test cases can be removed for RedCap UE with NTN.

Proposal 4: In the test cases list, the following test cases agreed in the last meeting can also be removed to reduce the test efforts:

- SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for 1Rx/2Rx RedCap UE with NTN

- SSB based L1-RSRP measurement for satellite access when DRX is used for 1Rx/2Rx RedCap UE with NTN

- CSI-RS based L1-RSRP measurement for satellite access when DRX is not used for 1Rx/2Rx RedCap UE with NTN

In addition, another issue is about whether to differentiate 1Rx and 2Rx RedCap UE in the following specific test cases:

- UE transmit timing

- SA FR1 timing advance adjustment accuracy

- DCI-based and Timer-based Active BWP Switch

- RRC-based Active BWP Switch

- UE specific CBW change on PCell in FR1 in non-DRX

- MAC-CE based pathloss reference signal switch delay

In core part for RedCap UE with NTN, the above requirements are the same for 1Rx and 2Rx RedCap UE and the legacy requirements in FR1-NTN are reused. In our view, for reducing the test efforts, the related test cases need to be defined without differentiating 1Rx and 2Rx RedCap UE.

Observation 2: In legacy RedCap TN, even though core requirements are same for 1Rx and 2Rx RedCap UE, there were separate test cases defined for 1Rx and 2Rx RedCap UE.

Observation 3: In core part for RedCap UE with NTN, the above requirements are the same for 1Rx and 2Rx RedCap UE and the legacy requirements in FR1-NTN are reused.

Proposal 5: For reducing the test efforts, the test cases for the following requirements need to be defined without differentiating 1Rx and 2Rx RedCap UE.

- UE transmit timing

- SA FR1 timing advance adjustment accuracy

- DCI-based and Timer-based Active BWP Switch

- RRC-based Active BWP Switch

- UE specific CBW change on PCell in FR1 in non-DRX

- MAC-CE based pathloss reference signal switch delay

- Conclusion

This document discussed performance requirements for RedCap UEs with NTN and presented following observations and proposals:

Observation 1: Among the above core requirements, the HD-FDD related requirements applicability rules for yellow highlighted part are related the new collision handling approach defined in R19 RAN1.

Proposal 1: The test principle in Note 2 at least can apply for all of test cases except for the following requirements:

- Requirements for CSI-RS based RLM/BFD/CBD

- Minimum requirement for L1 indication for RLM and BFD

- Scheduling availability of UE during RLM/BFD/CBD

- CSI-RS based L1-RSRP Reporting

- Scheduling availability of UE during L1-RSRP measurement

- UE Rx-Tx time difference measurements

Proposal 2: In the test cases list, the above FFS test cases can be removed for RedCap UE with NTN.

Proposal 3: The existing configurations defined in 1Rx RedCap test cases can be reused for (e)RedCap UE with NTN including:

- Dedicated CORESET Reference Channel for FDD

- Aggregation level(CCE) for RLM and BFD

- Correlation Matrix and Antenna Configuration

Proposal 4: In the test cases list, the following test cases agreed in the last meeting can also be removed to reduce the test efforts:

- SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for 1Rx/2Rx RedCap UE with NTN

- SSB based L1-RSRP measurement for satellite access when DRX is used for 1Rx/2Rx RedCap UE with NTN

- CSI-RS based L1-RSRP measurement for satellite access when DRX is not used for 1Rx/2Rx RedCap UE with NTN

Observation 2: In legacy RedCap TN, even though core requirements are same for 1Rx and 2Rx RedCap UE, there were separate test cases defined for 1Rx and 2Rx RedCap UE.

Observation 3: In core part for RedCap UE with NTN, the above requirements are the same for 1Rx and 2Rx RedCap UE and the legacy requirements in FR1-NTN are reused.

Proposal 5: For reducing the test efforts, the test cases for the following requirements need to be defined without differentiating 1Rx and 2Rx RedCap UE.

- UE transmit timing

- SA FR1 timing advance adjustment accuracy

- DCI-based and Timer-based Active BWP Switch

- RRC-based Active BWP Switch

- UE specific CBW change on PCell in FR1 in non-DRX

- MAC-CE based pathloss reference signal switch delay

- Reference

[1]	R4-2515056, Way Forward for [116bis][327] Rel-19 RRM performance_Part3, Moderator (CATT)