3GPP TSG RAN WG4 Meeting #117		R4-2520179

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Further discussion on channel arrangement for 6GR

Agenda Item:	8.2.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

At RAN4#116-bis, the first meeting where RAN4 initiated its 6GR studies, extensive discussions were held on system parameters, including channel arrangement for 6GR, leading to the following agreements [1]:

Channel raster - Agreement - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster - Investigate the interaction between the channel raster and the synchronization raster - Investigate the necessity of channel raster or alternative ways for the channel configuration - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported - Study the listed main proposals especially for the migration and co-existence approaches - Other options not presented in this meeting are not precluded - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

Sync raster - Agreement - Evaluation on sync raster from RAN4 perspective: - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4 - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving - Collaboration and planning: - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset. - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

Channel spacing - Agreement - Further study in RAN4 regarding channel spacing

In this contribution, we present our views on channel arrangement for 6GR from RAN4 perspective.

# 2 Discussion

Channel raster

Channel raster is one of the fundamental elements in defining system parameters. As in previous generations of mobile communication systems—from 3G and 4G to 5G—two categories can be distinguished:

- channel raster definitions for legacy refarming bands, and

- channel raster definitions for new non-refarming bands.

For the latter category, there is full flexibility in defining the raster without constraints from legacy systems. For example, in 5G, an SCS-based channel raster was introduced for newly defined frequency bands to better support multi-numerology operation and wider bandwidths.

For the former category, a fine 100kHz channel raster is usually defined. This can be dated back to GSM where the carrier bandwidth is 200 kHz, and in WCDMA, the 100 kHz raster was inherited partly from GSM, which used 200 kHz channel spacing with the following considerations:

- Defining a raster at half of that value (100 kHz) ensures alignment with GSM and WCDMA carrier grids, making spectrum re-farming and coexistence easier.

- This allowed regulators and operators to reuse existing frequency plans without redefining band edges or centre frequencies.

The same design principle was applied in 5G, where an SCS-based channel raster was introduced in addition to the legacy 100 kHz raster. This approach ensured numerology alignment and flexible multi-carrier coexistence, while maintaining compatibility with regulatory frequency grids.

For 6G, a similar distinction between the legacy refarming bands and new non-refarming bands should likewise be made when defining the channel raster.

Proposal 1: RAN4 to consider two categories for channel raster design in 6G − one for legacy refarming bands and another for new non-refarming bands respectively.

During the early stage of 5G development, the principle of “one band, one channel raster” was agreed upon. However, this principle was later compromised when both the 100 kHz and SCS-based channel rasters were introduced within the same band (e.g., band n90), leading to increased specification complexity.

With “single numerology” being one of the key targets for 6G, it is recommended that this principle be reinstated, ensuring that only one channel raster is specified per band to maintain clarity and consistency in the specifications.

Proposal 2: RAN4 to reinstate “one-band-one-channel-raster” in 6G channel raster design to maintain clarity and consistency in the specifications.

Sync raster

In 5G NR, the synchronization raster was introduced because the center frequency of the SSB is no longer necessarily aligned with the center frequency of the carrier. Unlike in LTE—where the synchronization signals were fixed at the carrier center—NR allows the SSB to be transmitted at frequency positions offset from the carrier center, for example to support beam-based synchronization, wideband operation, or multi-numerology configurations. As a result, a separate synchronization raster was defined to indicate the possible frequency locations of SSBs, enabling UEs to efficiently search for synchronization signals without assuming carrier-centered alignment.

This design provides flexibility in placing the SSB within the carrier, enabling efficient support for wideband operation. However, this flexibility comes at a cost: since the SSB center frequency is no longer fixed to the carrier center, the UE must search across all possible synchronization raster positions to detect and decode the SSB for a given band. Such a procedure increases the initial cell search complexity and power consumption. Hence, while the decoupled design improves deployment flexibility, it also introduces additional UE energy consumption and acquisition latency considerations.

One possible approach to balance flexibility and energy efficiency is to introduce a two-level synchronization raster: a coarser primary sync raster and a finer secondary sync raster. For a given carrier, the SSB should preferably be placed on a valid primary sync raster point; if such alignment is not feasible, a secondary sync raster can be used instead. This hierarchical design would allow UEs to prioritize scanning the primary raster, thereby reducing energy consumption during initial cell search, while still retaining a degree of flexibility in SSB placement for network deployment.

Proposal 3: RAN4 to consider a hierarchical sync raster in 6G consisting of a coarser primary sync raster and a finer secondary sync raster to reduce energy consumption during initial cell search while still retaining a degree of flexibility in SSB placement for network deployment.

Channel spacing

In 5G NR, two distinct channel spacing definitions are used depending on the operational scenario:

- Channel spacing between adjacent NR carriers (single-carrier operation): This definition applies when two independent NR carriers are deployed adjacently within the same band or in neighbouring bands, but not aggregated. The channel spacing in this case refers to the frequency separation between the centre frequencies of two adjacent NR carriers.

- Channel spacing for component-carrier contiguity (multi-carrier / CA operation): This definition applies when multiple component carriers are aggregated to form a multi-carrier transmission Here, channel spacing determines the frequency relationship between the centres of aggregated CCs transmitted by the same gNB or UE. For multi-carrier operation, there is a certain degree of flexibility in the placement of component carriers, since each component carrier already includes a built-in guard band. As a result, the spacing between component carriers in carrier aggregation can be optimized for implementation feasibility and spectral efficiency.

For 6GR, both channel-spacing definitions remain relevant but will need re-examination due to much wider channel bandwidths (e.g., 200–400 MHz), new operating frequencies (e.g., around 7 GHz and above), and shared spectrum scenarios.

Proposal 4: RAN4 to re-examine channel-spacing definitions for both single- and multiple-carrier operations by considering wider channel bandwidth, new operating frequencies and shared spectrum scenarios.

# 3 Conclusion

In this contribution, we have the following proposals for channel arrangement for 6GR:

Proposal 1: RAN4 to consider two categories for channel raster design in 6G − one for legacy refarming bands and another for new non-refarming bands respectively.

Proposal 2: RAN4 to reinstate “one-band-one-channel-raster” in 6G channel raster design to maintain clarity and consistency in the specifications.

Proposal 3: RAN4 to consider a hierarchical sync raster in 6G consisting of a coarser primary sync raster and a finer secondary sync raster to reduce energy consumption during initial cell search while still retaining a degree of flexibility in SSB placement for network deployment.

Proposal 4: RAN4 to re-examine channel-spacing definitions for both single- and multiple-carrier operations by considering wider channel bandwidth, new operating frequencies and shared spectrum scenarios.

# Reference

[1] R4-2514641, “WF for 6GR system parameter”, Huawei, HiSilicon

[2] R1-25xxxxx, “Minutes_report_RAN1#122bis_v104”, RAN1