3GPP TSG-RAN WG4 Meeting #117	R4-2520228

# Dallas, USA, Nov. 17-21, 2025

Source:	LG Electronics

Title:	Discussion on inter-vendor training collaboration and interoperability for two-sided models

Agenda item:	7.9.3

Document for:	Discussion

Introduction

In WID [1], objectives for CSI compression using two-sided model are captured as follows:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we provide our views on inter-vendor training collaboration and interoperability for two-sided models.

Discussion

According to WF [2], following issues are supposed to be discussed in RAN4#117.

The following are not agreements, but are recommendations for contributions and discussions at RAN4#117: - Proposals for assumptions for the dataset including how to cover scalability, channel models, number of samples - Proposals for how to evaluate the dataset including potential metrics to compare different dataset contributions - Proposals on model hyperparameters, considering in particular complexity, considering also quantization size - Proposals and reasoning for scalability solutions - Proposals and reasoning for training parameters - Proposals for performance metrics for evaluating the different potential model structures - Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

In this section, we provide our views on above open issues.

Assumptions for the dataset including how to cover scalability, channel models, number of samples

For the model scalability of CSI compression, number of Tx port (e.g., 16, 32), number of sub-bands, CSI feedback payload (e.g., low, mid, high) can be considered as a factor of model scalability. In Rel-19, the mixed dataset with fixed configuration is used. For example, 32 Tx port, 13 SBs and 64 bits of feedback payload was considered. In order to resolve scalability issue, various configuration should be considered in Rel-20. For potential candidate parameters, following can be considered.

- Candidate number of Tx ports: 16, 32

- Candidate number of sub-bands: 13 (for 10MHz), 26 (for 20MHz)

- Candidate payload size: X≤100bits, 100bits<Y<200bits, 200bits≤Z, e.g. X=64, Y=128, Z=256

Regarding channel model, in our view, TDL channel model which is the same assumptions of Rel-19 CSI prediction as agreed in RAN4#113 can be re-used.

Agreement For channel model for CSI prediction tests, TDL channel models will be used.

For the datasets for reference model training, if only TDL or CDL model is used, the expected outcome tends to be very high which may not be practical in some scenarios. So, for dataset for reference model training, mixed dataset from LLS and SLS can be further considered.

Proposal 1: For dataset assumption, following is considered.

- Candidate number of Tx ports: 16, 32

- Candidate number of sub-bands: 13 (for 10MHz), 26 (for 20MHz)

- Candidate payload size: X≤100bits, 100bits<Y<200bits, 200bits≤Z, e.g. X=64, Y=128, Z=256

- For channel model for CSI compression tests, TDL channel models will be used.

Assumptions for the test decoder and reference encoder structures

During previous meetings, following agreement regarding criteria for selecting the reference encoder/decoder models were made as captured below.

Criteria for selecting the encoder/decoder model: Agreements: - Achievable performance, complexity and robustness in different conditions need to be taken into account in selecting the model. - Whether a single or multiple test decoders is needed will need more elaboration during the WI phase - For option 3 the reference encoder should have been joint trained with the test decoder. Test decoder and reference encoder backbone The test decoder and the reference encoder in Option 3 will have the same backbone. - It is FFS if a single test decoder will be specified. - The test decoder and the reference encoder can have different structures, e.g. # of layers

Regarding on the number of decoders, in our view, a single decoder is preferred. The generalization performance of a single decoder can be achieved by training the test decoder with mixed dataset obtained from various deployment scenarios, frequency bands, antenna configurations, etc. However. according to the Rel-19 study, there is no evaluation whether a single decoder can be generalized across all considered scenarios. If the generalization cannot be achieved with a single decoder, multiple decoder can be considered, and this may lead to a burden on the storage in 3GPP. Therefore, if multiple decoder is indeed needed, the max number of test decoders should be limited, e.g., 2.

For the reference encoder, if the low-complexity encoder (e.g., transformer based) can achieve similar performance compared to high complexity encoder, there is no need to consider high complexity encoder.

Proposal 2: RAN4 to consider a single test decoder trained from sufficient amount of mixed dataset. If multiple decoder is considered, the max number should be limited, e.g., 2.

Performance metrics for evaluating the different potential model structures

In RAN4#116bis, following is agreed.

Model backbone Transformer is assumed as the baseline Only Quantized parameters are specified

For the selection of model structure, SGCS can be considered as performance metric. For SGCS, average SGCS and SGCS values for X percentiles can be considered. Since throughput evaluation is more practical evaluation methodology compared to using intermediate KPI (e.g., SGCS), throughput gain can be alternatively considered for performance metric. This is because that 10% SGCS gain does not imply 10% throughput gain in general.

Proposal 3: Consider at least following performance metrics for evaluating the difference model structure

- Average SGCS and/or SGCS values for X-percentiles

- Throughput gain

Preliminary evaluation

In this section, we provide our initial evaluation for various model hyperparameters. For the evaluation, following structure of transformer agreed in RAN1 is used.

Figure 1. The structure of Transformer-based CSI compression model

For the model hyper parameters, following parameter configurations are considered.

|  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| NTF | 6 | 2 | 4 | 4 |
| dmodel | 64 | 128 | 128 | 256 |
| Nhead | 8 | 8 | 8 | 32 |
| dhead | 8 | 16 | 16 | 8 |
| dFF | 256 | 512 | 512 | 1024 |

Also, we create dataset based on SLS where UMa at 2GHz, 32 port Tx ports, and 20MHz BW are considered.

Table 1. Performance comparison according to various hyper parameter and payload size

|  | X=70 bits |  |  | Y=130 bitss |  |  | Z=250 bits |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Absolute SGCS | FLOPs [M] | # param [M] | Absolute SGCS | FLOPs [M] | # param [M] | Absolute SGCS | FLOPs [M] | # param [M] |
| Case 1 | 0.7637 | 2.5 | 0.6 | 0.8105 | 2.8 | 0.7 | 0.8499 | 3.8 | 1.1 |
| Case 2 | 0.7843 (+2.70%) | 3.6 (+44%) | 0.8 (+33%) | 0.8198 (+1.15%) | 4.0 (+42%) | 0.9 (+28%) | 0.8566 (+0.79%) | 5.2 (+36%) | 1.3 (+18%) |
| Case 3 | 0.79 (+3.44%) | 6.2 (+148%) | 1.6 (+166%) | 0.8229 (+1.53%) | 6.7 (+139%) | 1.7 (+142%) | 0.8617 (+1.39%) | 7.8 (+105%) | 2.1 (+90%) |
| Case 4 | 0.7903 (+3.48%) | 22.9 (+816%) | 6.4 (+966%) | 0.8237 (+1.63%) | 23.5 (+739%) | 6.5 (+828%) | 0.8779 (+3.29%) | 25.1 (+560%) | 6.9 (+527%) |

According to above simulation results, following observation can be made.

Observation 1: Increasing model complexity does not lead to proportional improvements in SGCS gain. As the model becomes more complex, SGCS gain improvement rate becomes saturated.

Proposal 4: Model hyper parameter selection should consider complexity and performance trade-off.

Summary

In this contribution, we provide our views on inter-vendor training collaboration and interoperability for two-sided models. Based on above, observation and proposals are proposed as below.

Observation 1: Increasing model complexity does not lead to proportional improvements in SGCS gain. As the model becomes more complex, SGCS gain improvement rate becomes saturated.

Proposal 1: For dataset assumption, following is considered.

- Candidate number of Tx ports: 16, 32

- Candidate number of sub-bands: 13 (for 10MHz), 26 (for 20MHz)

- Candidate payload size: X≤100bits, 100bits<Y<200bits, 200bits≤Z, e.g. X=64, Y=128, Z=256

- For channel model for CSI compression tests, TDL channel models will be used.

Proposal 2: RAN4 to consider a single test decoder trained from sufficient amount of mixed dataset. If multiple decoder is considered, the max number should be limited, e.g., 2.

Proposal 3: Consider at least following performance metrics for evaluating the difference model structure

- Average SGCS and/or SGCS values for X-percentiles

- Throughput gain

Proposal 4: Model hyper parameter selection should consider complexity and performance trade-off.

Reference

[1] RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm

[2] R4-2514614, “WF on R20 NR_AIML_air_Ph2”, Ericsson