3GPP TSG-RAN WG4 Meeting #117	R4-2520248

Dallas, United States, 17th – 21st November 2025

Source:	Nokia

Title:	Simulation results for UE demodulation performance requirements for LPWUS

Agenda item:	6.16.3

Document for:	Information

Introduction

This contribution contains Nokia’s simulation results related to defining demodulation requirements for LPWUS

Simulation results

Agreed simulation configuration

Table 1 contains the agreed configurations from RAN4#116 and RAN4#116bis.

Table 1: Agreed configuration for simulation alignment

| Parameter | Value | Comments |
| --- | --- | --- |
| Waveform FR1 | OOK-4 OFDM |  |
| Waveform FR2 | [OOK-1] OFDM |  |
| Duplex modes | FR1: FDD FR1+FR2: TDD |  |
| Number of RBs | 11 |  |
| CBW/SCS | FR1: 10MHz/15kHz for FDD 40MHz/30kHz for TDD FR2: 100MHz/120kHz for TDD |  |
| Carrier frequency | FR1 FDD: Fc = 900MHz FR2 TDD: Fc = 2.6GHz FR2 TDD: Fc = 28GHz |  |
| Antenna configuration | FR1: 1T1R FR2: 1T1R |  |
| Channel model | FR1: TDLA30-10 FR2: TDLA30-75 |  |
| Time offset | FR1: - 5us for OOK1and OOK4 with M=1 - 1us for OOK4 with M=4 FR2: - 0.5us for OOK1 |  |
| Frequency offset | FR1: - OFDM: 5 ppm - OOK: 10 ppm FR2: - OFDM: 0.5 ppm - OOK: 1 ppm |  |
| Coding/decoding algorithms | Small block coding for 2 info bits RM coding for 5 info bits |  |
| Number of info bits | 2 and 5 |  |
| Number of coded bits after rate-matching | 12 for 2 info bits 32 for 5 info bits |  |
| M | For OFDM receiver: - FR1: M=4 only - FR2: M=1 only For OOK receiver: - FR1: M=4 only - FR2: M=1 only |  |
| Number of overlaid sequence (L1) | FR1: 4 FR2: 2 |  |
| Root sequence | q=1, root1=1 |  |

FR1 Results for Missed Detection Rate (MDR)

FDD

Table 2 provides an overview of simulation results for FR1 FDD using OOK and OFDM receiver.

Table 2 MDR for OOK/OFDM receivers (FR1 - FDD). Results are provided in dB@1% MDR

| Frequency | 900MHz |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | OOK4 |  | OFDM |  |
| SCS (kHz) | 15 |  |  |  |
| Infobits | 2 | 5 | 2 | 5 |
| symbols | 12 | 32 | 12 | 32 |
| M | 4 |  | 1 |  |
| Time offset (us) | 1 |  | 0 |  |
| Freq. offset (ppm) | 10 |  | 5 |  |
| Results (dB@1% MDR) | 3.9 | 5.1 | 1.6 | 2.4 |

TDD

Table 3 provides an overview of simulation results for FR1 TDD using OOK and OFDM receiver.

Table 3 MDR for OOK/OFDM receivers (FR1 - TDD). Results are provided in dB@1% MDR

| Frequency | 2.6GHz |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | OOK4 |  | OFDM |  |
| SCS (kHz) | 30 |  |  |  |
| Infobits | 2 | 5 | 2 | 5 |
| symbols | 12 | 32 | 12 | 32 |
| M | 4 |  | 1 |  |
| Time offset (us) | 1 |  | 0 |  |
| Freq. offset (ppm) | 10 |  | 5 |  |
| Results (dB@1% MDR) | 2.7 | 3.6 | 1.9 | 2.1 |

FR2 Results for Missed Detection Rate (MDR)

Table 4 provides an overview of simulation results for FR2 using OOK and OFDM receiver.

Table 4 MDR for OOK/OFDM receivers (FR2). Results are provided in dB@1% MDR

| Frequency | 28GHz |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | OOK1 |  | OFDM |  |
| SCS (kHz) | 120 |  | 120 |  |
| Infobits | 2 | 5 | 2 | 5 |
| symbols | 12 | 32 | 12 | 32 |
| M | 1 |  |  |  |
| Time offset (us) | 0.5 |  | 0 |  |
| Freq. offset (ppm) | 1 |  | 0.5 |  |
| Results (dB@1% MDR) | 3.7 | 6.8 | 0.4 | 0.7 |

Conclusion

This paper provides Nokia’s simulation results for LP-WUS MDR.

- References

- R4-2514813 - WF for [116bis][220] NR_LPWUS_Demod