3GPP TSG-RAN4 Meeting #117	R4-2520251

# Dallas, United States, 17th November 2025 - 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | DraftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_FR1_7MHz_BW-Perf) DraftCR on 38.101-4 for adding UE performance requirements for 7 MHz CBW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_7MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of CA performance requirements for 7MHz CBW |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce 7MHz CBW requirements for FDD 15kHz SCS. Set Number of PRBs in CORESET to 30. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No CA requirements introduced for 7MHz CBW |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.2A.2, 5.2A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Re-submission of R4-2512618 to match latest specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

## 5.2	PDSCH demodulation requirements

The parameters specified in Table 5.2-1 are valid for all PDSCH tests unless otherwise stated.

Table 5.2-1: Common test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  | Symbols | 0, 1 |
|  | Number of PRBs in CORESET |  |  | Table 5.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI state |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I; Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12] For number of Tx>2, set “codebookMode” to 1 as defined in section 5.2.2.2.1 of TS 38.214 [12] |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 3 for 2 CSI-RS ports, 5 for 4 CSI-RS ports and 6 for 8 CSI-RS ports |
|  | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'No CDM' for 1 transmit antenna 'FD-CDM2' for 2, 4 and 8 transmit antenna |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests {1000-1007} for Rank 8 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3,Rank 4 and Rank 8 tests |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| PT-RS configuration |  |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PUCCH HARQ ACK spaitial bundling |  |  |  | Not configured |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1:Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. For number of Tx>2 and Rank=1 or 2, Set “codebookMode” to 1 as defined in section 5.2.2.2.1 of TS 38.214 [12] |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. Note 3: Refer to Table 7.4.1.5.3-1 in [9] |  |  |  |  |

Table 5.2-2: Number of PRBs in CORESET

| SCS (kHz) | 5 MHz | 7 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 24 | 30 | 48 | 78 | 102 | 132 | 156 | 186 | 216 | 240 | 270 | N/A | N/A | N/A |
| 30 | 6 | N/A | 24 | 36 | 48 | 60 | 78 | 90 | 102 | 114 | 132 | 162 | 216 | 270 |

# <End of change 1>

# <Start of change 2>

### 5.2A.2	2RX requirements

#### 5.2A.2.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.2.1-4 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.2.1-1 ~ Table 5.2A.2.1-3, with the parameters in Table 5.2A-1 ~ Table 5.2A-3 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.2.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 7 | R.PDSCH.1-10.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 15 | R.PDSCH.1-9.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 20 | R.PDSCH.1-9.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 25 | R.PDSCH.1-9.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.0 |
| 30 | R.PDSCH.1-9.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 35 | R.PDSCH.1-10.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.9 |
| 40 | R.PDSCH.1-10.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.0 |
| 45 | R.PDSCH.1-10.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.5 |
| 50 | R.PDSCH.1-10.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.4 |

# <End of change 2>

# <Start of change 3>

### 5.2A.3	4RX requirements

#### 5.2A.3.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.3.1-4 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.3.1-1 ~ Table 5.2A.3.1-3, with the parameters in Table 5.2A-1 ~ Table 5.2A-3 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.3.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 7 | R.PDSCH.1-10.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 15 | R.PDSCH.1-9.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 20 | R.PDSCH.1-9.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 25 | R.PDSCH.1-9.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 30 | R.PDSCH.1-9.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 40 | R.PDSCH.1-10.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 50 | R.PDSCH.1-10.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.9 |

# <End of change 3>