3GPP TSG-RAN WG4 Meeting #117						         	       		R4-2520290

Dallas, USA, Nov. 17-21, 2025

Title: 	Influence of antenna coupling and ACLR on 4x26dBm and 2x26dBm HPUE back-off

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.4	HPUE requirements	[NR_RF_Ph5-Core]

Document for:	Approval

# 1	Background

In RAN#116bis, we submitted a contribution [2] on release 20 HPUE based on 4x26dBm and 2x26dBm PA configurations partially reusing the work we did in R19 on NTN HPUE [3]. A few agreements were captured in way forward [1] based on the preliminary discussion. In this contribution, based on further analysis of the measurement data collected for [3], we further elaborate on our proposals for 4x26dBm power class and MPR and on PC1.5 FDD sensitivity to antenna isolation. Additionally, we further detail our proposal on a method to automatically calculate the HPUE RSD from the PC3 REFSENS.

# Discussion

## 2.1 Way forward input

In RAN4#116bis, a way forward [1] captured several agreements copied below in italics.

4Tx with up to 32 dBm MOP MPR/A-MPR assumptions:

Agreement: Evaluate assuming following antenna isolation assumptions

•	For CPE, assuming both 10dB and 20dB,

•	For FWA, assuming 20dB.

Come back once MPR results are available, to evaluate if it is possible to select a single CPE antenna isolation assumption based on the MPR results.

Agreement: Evaluate A-MPR for NS_04 and NS_47 for n41.

PC1.5 with 2Tx for FDD bands

Agreement: 29 dBm is the maximum output power. 31 dB is the agreed ACLR value.

Agreement:

- Agree to evaluate PC1.5 A-MPR for NS_03, NS_05, NS_48, and NS_49.

- Encourage companies to check if NS_03U, NS_05U, and NS_100 are needed.

  - If needed, how to define UTRA ACLR for PC1.5

Agreement on REFSENS degradation: Encourage companies to provide the general methodology for deriving the REFSENS degradation (RSD) in next meeting.

During the discussion, it was also discussed what antenna isolation assumptions should be taken for the 4x26dBmPA case and also for PC1.5 FDD with 2x26dBmPA at lower frequencies.

In this contribution we reused the measurement data we took for the NTN PC1 4Tx work and re-evaluated the 4x26dBm configurations versus antenna isolation and, for the 4x26dBm case, two ACLR targets of 31dB (same as PC1.5) and 34dB (same as NTN PC1) to get an estimate of the achievable output power. Since this measurement is based on a 2Tx measurement with 10*Log(3) (~ 5dB for simplicity) increased coupling between the two PAs to mimic the equivalent coupling of 3PAs into1PA. doing so, we also get the 2x26dBm PC1.5 backoff versus antenna isolation at 31dB ACLR.

## 2.2 Required backoff for different antenna coupling and ACLR

The complete data is provided in annex, but since we are in the early phase where we check for the maximum achievable power, we focus our observations on the maximum LCRB inner cases.

The complete list of measurements if provided in annex (chapter 5), for this chapter, the following cases are presented for analysis

  - PA calibrated for 31dB ACLR at 25dBm with 20MHz QPSK DFT-s-OFDM 100RB0

  - Measurement for 5/10/20/50MHz CBW at 15kHz SCS + 100MHz at 30kHz SCS

  - Both DFT-s-OFDM and CP-OFDM QPSK waveforms

  - Maximum LCRB inner

- 2x26dBm PA measurements in Table 1

  - SEM and 31dB ACLR are measured with a 0.5dB step power sweep, only the three relevant steps are presented in terms of margin to the requirement.

  - Measurements are done for 5/10/15/20dB antenna isolation

- 4x26dBm PA measurements in Table 2

  - SEM and 31dB or 34dB ACLR are measured with a 0.5dB step power sweep, only the three relevant steps are presented in terms of margin to the requirement.

  - Measurements are done for 10/15/20/25dB antenna isolation (5dB offset from the 2x26dBm case, the exact offset should be 4.77dB, but have been using 5dB as a simplification while measurement accuracy is limited to 0.1/0.2dB)

Table 1: SEM and ACLR versus power and antenna isolation for 2x26dBm configuration

Observations:

- The specified PC1.5 MPR is met for both the typical 10dB antenna isolation case and the FWA 20dB antenna isolation case (although marginally for the 20dB isolation case at 20MHz)

- The 0.5dB additional MPR for 10dB versus 20dB antenna isolation is justified

- At full power, the limitation comes from SEM, not ACLR

- At 5dB antenna isolation, the margin drops between 0.5 to 1 dB compared to 10dB antenna isolation potentially requiring another 0.3 to 0.5dB additional back-off

- There are slightly higher margins for CP-OFDM compared to DFT-s-OFDM.

Table 2: SEM and ACLR versus power and antenna isolation for 2x26dBm configuration

Observations for DFT-s-OFDM:

- At 32dBm and 31.5dBm, except for 100MHz CBW with 34dB ACLR, the limiting requirement is SEM not ACLR irrespective of the antenna isolation value:

  - Thus, 31dB or 34dB ACLR is not relevant and 32dBm cannot be reached with the largest inner allocation. However, 32dBm may be reachable for smaller LCRB inner and especially inner third allocations, as we discussed for NR NTN.

- For 20dB antenna isolation SEM is passed only at 31dBm with some margin. Note that the 15dB antenna isolation assumption used for NR NTN 4Tx PC1 also passes, though marginally

- For 10dB antenna isolation SEM is still not passing at 31dBm, so an additional 0.5dB MPR might be needed

- In case 37dB ACLR would be required, the maximum reachable power would likely have to drop to 30dBm as there is only a 0.3dB margin for 34dB ACLR at 31dBm.

Observations for CP-OFDM:

- At 30dBm, except for 100MHz CBW with 34dB ACLR, the limiting requirement is SEM not ACLR irrespective of the antenna isolation value. Thus, 31dB or 34dB ACLR is not relevant

- At 29.5dBm, irrespective of the antenna isolation value, SEM and 34dB ACLR are passed

- In case 37dB ACLR would be required, it would still not pass at 29dBm and an additional 0.5dB MPR might be needed, reaching 28.5dBm.

## 2.3 4Tx based on 26dBm PAs

Based on the observations made on the measurements results in the previous chapter, we can come to the following conclusions:

- Overall, the reachable output power limitation is related to SEM with ACLR target up to 34dB.

  - 32dBm is not reached for inner while at 31dBm, 34dB ACLR and SEM are met for antenna isolation ≥15dB

- To avoid having two sets of requirements, using 15dB antenna isolation would be a good compromise

- With the above, there is an option to merge the TN and NTN PC1 4Tx requirement.

- With 37dB ACLR, there would not be any 0dB MPR waveform as the power would likely have to drop down to 30dBm for largest inner allocation.

Proposal on 4x26dBm PA power class:

- Assuming ≥15dB antenna isolation and up to a 34dB ACLR target, the 31dBm PC1 power class can be reused with a single MPR table.

  - PC1 4Tx can be merged for TN and NTN

  - To account for smaller inner that may reach 32dBm but avoid specifying a new power class only for a limited number of allocations:

    - Either a 1dB boosting or 1dB extra margin at the top of the power class range can be added.

    - To avoid significant work in defining the allocations that can be boosted, the latter is preferred

  - Inner waveforms are 0dB MPR for DFT-s-OFDM and 1.5dB MPR for CP-OFDM

  - Based on our measurements for full allocation waveforms, with the above assumptions, outer waveforms need 2.5dB MPR for DFT-s-OFDM and 4.5dB MPR for CP-OFDM

  - Edge allocations are dominated by the digital filtering/windowing, thus, the edge allocation definition and value from 1Tx PC1 can be reused.

- If 37dB ACLR is required, all MPR should be increased by 1dB.

## 2.4 2Tx PC1.5 for FDD

Since the assumption is to reuse 31dB ACLR for FDD 2Tx PC1.5, the only question is whether the 10dB isolation assumption is valid in low bands. In our analysis in chapter 2.2, we showed that a 5dB antenna isolation would require an additional 0.2 to 0.3dB of back-off. However, a 5dB antenna isolation is not practical even for the Rx diversity gain and, at worst, one may consider 8dB antenna isolation which likely will not need any additional back-off. As such we suggest that current PC1.5 2Tx MPR developed for TDD bands is still valid for FDD bands.

Proposal for 2Tx PC1.5 for FDD MPR:

- The current 2Tx TDD PC1.5 MPR tables are reused for FDD bands where the UL is >1.6GHz

- Unless antenna isolation <8dB or ACLR >31dB should be assumed for FDD band <1.6GHz, the current 2Tx TDD PC1.5 MPR tables are reused for all FDD bands.

Separately from the reuse of the PC1.5 TDD bands for MPR and ACLR, the unique aspect related to PC1.5 FDD is the need to evaluate the RSD as was done for PC2. However, we suggest that, similar to what was done for inter-band MSD for HPUE, there are means to simplify the RSD calculations by using a fixed set of parameters like interference increase from PC3 to PC1.5 and a fixed main path to diversity path imbalance for the PC3 reference. Also, only 2Tx RSD is needed.

Proposal for 2Tx PC1.5 FDD RSD:

- Only 2Tx RSD is specified for PC1.5

- RSD simplification is investigated in a similar way than for MSD simplification based on:

  - For PC3 reference, a 6dB main to diversity imbalance is assumed and based on smallest CBW ideal REFSENS scaling, from which the equivalent de-sense and UL interference is calculated

  - The UL interference is increased by 3dB compared to PC3 1Tx and applied on main and diversity branches for PC1.5 2Tx

    - i.e. , being a 2Tx case, the REFSENS is recalculated with a main to diversity imbalance of 0dB

  - As de-sense is indirect, there is no possibility for an LUT from PC3 REFSENS to HPUE RSD but de-sense can be calculated from the above assumptions and an PC1.5 LUT applied to derive the RSD.

    - FFS if a similar method should be applied to derive PC2 1Tx and 2Tx RSDs.

  - Another option is to provide an LUT for “additional RSD” from 2Tx PC2 to 2Tx PC1.5

- Given the large duplex distance, PC1.5 RSD should be minimal for Band n1 but important for Band n25 that has a significant de-sense for CBW>25MHz (both from REFSENS and PC2 2Tx RSD).

# Conclusion

In this contribution, we provide measurements to assess the required back-off depending on ACLR target and antenna isolation for 2x26dBm 2Tx PC1.5 and 4x26dBm configurations and make the following proposals.

Proposal on 4x26dBm PA power class:

- Assuming ≥15dB antenna isolation and up to a 34dB ACLR target, the 31dBm PC1 power class can be reused with a single MPR table.

  - PC1 4Tx can be merged for TN and NTN

  - To account for smaller inner that may reach 32dBm but avoid specifying a new power class only for a limited number of allocations:

    - Either a 1dB boosting or 1dB extra margin at the top of the power class range can be added.

    - To avoid significant work in defining the allocations that can be boosted, the latter is preferred

  - Inner waveforms are 0dB MPR for DFT-s-OFDM and 1.5dB MPR for CP-OFDM

  - Based on our measurements for full allocation waveforms, with the above assumptions, outer waveforms need 2.5dB MPR for DFT-s-OFDM and 4.5dB MPR for CP-OFDM

  - Edge allocations are dominated by the digital filtering/windowing, thus, the edge allocation definition and value from 1Tx PC1 can be reused.

- If 37dB ACLR is required, all MPR should be increased by 1dB.

Proposal for 2Tx PC1.5 for FDD MPR:

- The current 2Tx TDD PC1.5 MPR tables are reused for FDD bands where the UL is >1.6GHz

- Unless antenna isolation <8dB or ACLR >31dB should be assumed for FDD band <1.6GHz, the current 2Tx TDD PC1.5 MPR tables are reused for all FDD bands.

Additionally, we have proposals for a method to calculate HPUE RSD from the PC3 REFSENS.

Proposal for 2Tx PC1.5 FDD RSD:

- Only 2Tx RSD is specified for PC1.5

- RSD simplification is investigated in a similar way than for MSD simplification based on:

  - For PC3 reference, a 6dB main to diversity imbalance is assumed and based on smallest CBW ideal REFSENS scaling, from which the equivalent de-sense and UL interference is calculated

  - The UL interference is increased by 3dB compared to PC3 1Tx and applied on main and diversity branches for PC1.5 2Tx

    - i.e. , being a 2Tx case, the REFSENS is recalculated with a main to diversity imbalance of 0dB

  - As de-sense is indirect, there is no possibility for an LUT from PC3 REFSENS to HPUE RSD but de-sense can be calculated from the above assumptions and an PC1.5 LUT applied to derive the RSD.

    - FFS if a similar method should be applied to derive PC2 1Tx and 2Tx RSDs.

  - Another option is to provide an LUT for “additional RSD” from 2Tx PC2 to 2Tx PC1.5

- Given the large duplex distance, PC1.5 RSD should be minimal for Band n1 but important for Band n25 that has a significant de-sense for CBW>25MHz (both from REFSENS and PC2 2Tx RSD).

# References

[1] R4-2515087 Way Forward for [116bis][316] NR_UE_RF_Ph5, Nokia, Murata, RAN4#116b

[2] R4-2513781 Inputs to 4Tx based on 26dBm PAs and PC1.5 for FDD	Skyworks Solutions Inc., RAN4#116b

[3] R4-2511320 MPR and antenna isolation evaluation for 4Tx PC1, Skyworks Solutions Inc., RAN4#116

# Annexes

The complete list of measurements if provided in this annex:

- 2x26dBm PA measurements in chapter 5.1

  - PA calibrated for 31dB ACLR at 25dBm with 20MHz QPSK DFT-s-OFDM 100RB0

  - Measurement for 5/10/20/50MHz CBW at 15kHz SCS + 100MHz at 30kHz SCS

  - Both DFT-s-OFDM and CP-OFDM QPSK waveforms

  - Maximum LCRB inner and maximum LCRB outer in Table 5.1-1

  - 4RB outer in Table and 1RB edge in Table 5.1-2

  - Measurements are done for 5/10/15/20dB antenna isolation

  - SEM and 31dB ACLR are measured with a 0.5dB step power sweep, only the three relevant steps are presented in terms of margin to the requirement.

- 4x26dBm PA measurements in chapter 5.2

  - PA calibrated for 31dB ACLR at 25dBm with 20MHz QPSK DFT-s-OFDM 100RB0

  - Measurement for 5/10/20/50MHz CBW at 15kHz SCS + 100MHz at 30kHz SCS

  - Both DFT-s-OFDM and CP-OFDM QPSK waveforms

  - Maximum LCRB inner and maximum LCRB outer in Table 5.2-1

  - 4RB outer in Table and 1RB edge in Table 5.2-2

  - Measurements are done for 10/15/20/25dB antenna isolation (5dB offset from 2x26dBm case, for which the exact offset should be 4.77dB but using 5dB as a simplification since measurement accuracy is limited to 0.1/0.2dB)

  - SEM and 31dB or 34dB ACLR are measured with a 0.5dB step power sweep, only the three relevant steps are presented in terms of margin to the requirement.

## 5.1 2x26dBm PA measurements

Table 5.1-1: 2x26dBm Maximum LCRB inner and Maximum LCRB outer ACLR and SEM versus power and antenna isolation

Table 5.1-2: 2x26dBm 4RB outer in Table and 1RB edge ACLR and SEM versus power and antenna isolation

## 5.2 4x26dBm PA measurements

Table 5.2-1: 4x26dBm Maximum LCRB inner and Maximum LCRB outer ACLR and SEM versus power and antenna isolation

Table 5.2-2: 4x26dBm 4RB outer in Table and 1RB edge ACLR and SEM versus power and antenna isolation