3GPP TSG-RAN WG4 Meeting #117	R4-2520304

Dallas, USA, Nov. 17-21, 2025

Source: 	KDDI Corporation

Title: 	Discussion on 6G general UE RF requirement

Agenda Item:	8.2.3

Document for:	Discussion

- 1. Introduction

In RAN4#116-bis, RAN4 began the discussion on 6G system parameter and agreed the WF on the numerology as follows[1].

- Sub-topic 3-4: Numerology - Agreement - Evaluate the following proposals regarding numerology from RAN4 perspective - "Single numerology" proposal - Frequency sub-range/Band specific SCS values proposal - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band - Study numerology for SSB of initial cell search from RAN4 perspective - Other proposals not presented in this meeting are not precluded - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

In this contribution, we focus on single carrier output power requirements and propose some issues.

- 2. Discussion

Lessons and learn from 5G

Considering 5G deployment, we think that while 5G NR specified multiple SCS options per band and allowed different SCS for SSB and data, these features are not typically used in commercial deployments. This complexity has yielded limited practical benefit.

Re-farming FR1 FDD band to 6G

For re-farmed FR1 FDD bands, using the same SCS as 5G is considered beneficial for MRSS between 5G and 6G and smoother migration from 5G to 6G.

Proposal 1: The numerology discussion for 6GR in legacy band has impact on MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD.

We prefer to adopt a single SCS per operating band or frequency sub-range for 6G. This aims to drastically simplify specification, implementation, and network management.

Proposal 2: Specify single numerology per operating band.

- 3. Conclusion

Proposal 1: The numerology discussion for 6GR in legacy band has impact on MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD.

Proposal 2: Specify single numerology per operating band.

- 4. Reference

[1] R4-2514641, WF for 6GR system parameter, Huawei, RAN4#116bis