| 3GPP TR 38.719-00-00 V0.4.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Rel-19 SUL configurations and CA band combinations with SUL(s); (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Background	8

4.1	TR Maintenance	9

4.2	Notation for CA with SUL(s)	9

5	SA NR SUL band combination: Specific Band Combination Part	9

5.1	CA_n1-n3_n78-n80	9

5.1.1	Operating bands	9

5.1.2	Configuration	10

5.1.3	Maximum output power	10

5.1.4	UE co-existence studies for 1 band UL	10

5.1.5	REFSENS requirements	11

5.1.6	∆TIB and ∆RIB values	11

5.1.7	Out-of-band blocking exception	12

5.2	CA_n1-n3_n78-n84	12

5.2.1	Operating bands	12

5.2.2	Configuration	12

5.2.3	Maximum output power	12

5.2.4	UE co-existence studies for 1 band UL	12

5.2.5	REFSENS requirements	14

5.2.6	∆TIB and ∆RIB values	14

5.2.7	Out-of-band blocking exception	14

5.3	SUL_n80-n104	14

5.3.1	Operating bands	14

5.3.2	Configuration	14

5.3.3	Maximum output power	14

5.3.4	UE co-existence studies for 1 band UL	15

5.3.5	REFSENS requirements	16

5.3.6	∆TIB and ∆RIB values	16

5.3.7	Out-of-band blocking exception	17

5.4	SUL_n81-n104	17

5.4.1	Operating bands	17

5.4.2	Configuration	17

5.4.3	Maximum output power	17

5.4.4	UE co-existence studies for 1 band UL	17

5.4.5	∆TIB and ∆RIB values	18

5.4.6	Out-of-band blocking exception	19

5.5	CA_n5_n78-n89	19

5.5.1	Operating bands	19

5.5.2	Configuration	19

5.5.3	Maximum output power	20

5.5.4	UE co-existence studies for 1 band UL	20

5.5.5	REFSENS requirements	21

5.5.6	∆TIB and ∆RIB values	21

5.5.7	Out-of-band blocking exception	21

5.6	SUL_n83-n104	22

5.6.1	Operating bands	22

5.6.2	Configuration	22

5.6.3	Maximum output power	22

5.6.4	UE co-existence studies for 1 band UL	22

5.6.5	REFSENS requirements	23

5.6.6	∆TIB and ∆RIB values	23

5.6.7	Out-of-band blocking exception	24

5.7	SUL_n84-n104	24

5.7.1	Operating bands	24

5.7.2	Configuration	24

5.7.3	Maximum output power	24

5.7.4	UE co-existence studies for 1 band UL	24

5.7.5	REFSENS requirements	25

5.7.6	∆TIB and ∆RIB values	26

5.7.7	Out-of-band blocking exception	26

5.8	SUL_n89-n104	26

5.8.1	Operating bands	26

5.8.2	Configuration	26

5.8.3	Maximum output power	26

5.8.4	UE co-existence studies for 1 band UL	27

5.8.5	REFSENS requirements	28

5.8.6	∆TIB and ∆RIB values	28

5.8.7	Out-of-band blocking exception	29

5.Y	SUL_nX-nY/ CA_nX_nY-nZ/ CA_nX-nW_nY-nZ	29

5.Y.1	Operating bands	29

5.Y.2	Configuration	29

5.Y.3	Maximum output power	29

5.Y.4	UE co-existence studies for 1 band UL	29

5.Y.5	REFSENS requirements	31

5.Y.6	∆TIB and ∆RIB values	31

5.Y.7	Out-of-band blocking exception	31

Annex <A> (informative): Change history	31

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for SUL configurations and CA band combinations with SUL(s) under Rel-19 time frame. The purpose is to gather the relevant background information and studies in order to address NR SUL for the Rel-19 bands/band combinations in Table 1-1 to Table 1-2.

Table 1-1: Release 19 SUL bands

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |

Table 1-2: Release 19 SA SUL band combinations

| SA SUL band combination | REL independent from |
| --- | --- |
| CA_n1-n3_n78-n80 | Rel-15 |
| CA_n1-n3_n78-n84 | Rel-15 |
| SUL_n80-n104 | Rel-15 |
| SUL_n81-n104 | Rel-15 |
| CA_n5_n78-n89 | Rel-15 |
| SUL_n83-n104 | Rel-15 |
| SUL_n84-n104 | Rel-15 |
| SUL_n89-n104 | Rel-15 |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-241674, New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, RAN#104

[3]	R4-2306546, "WF on the notation for single SUL CA combinations", Apple

[4]	R4-2309086, "CR for 38.101-1: Single SUL CA combination notation modifications", Apple

[5]	R4-2303459, "WF on the notation for band combinations with two SUL", CMCC

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA and/or SUL operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA and/or SUL operation, for serving cell c.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

BS				Base Station

BCS				Bandwidth Combination Set

CA				Carrier Aggregation

CC				Component Carriers

DC				Dual Connectivity

DL				Downlink

E-UTRA 				Evolved UMTS Terrestrial Radio Access

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

NR				New Radio

NSA				Non-Stand Alone

REFSENS				Reference Sensitivity power level

SA				Stand Alone

SCS				Subcarrier spacing

SUL				Supplementary uplink

TDD				Time Division Duplex

UE				User Equipment

UL				Uplink

ULSUP				UL sharing from the UE perspective

4	Background

The present document is a technical report for SA NR SUL configurations and CA band combinations with SUL cell(s) under Rel-19 time frame. It covers both the UE and BS side. New configurations still emerge from existing bands and whenever new band is specified, it will create a potential for several new SUL configurations. There is no restriction on supporting UL MIMO in SUL band and/or NR band for this basket WI. SUL configurations will be introduced in a release independent manner based on TS38.307, which will be updated depending on newly introduced SUL configurations.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

4.2	Notation for CA with SUL(s)

Referring to the WFs [3][5] and Rel-18 CR [4], the notations for single SUL CA combinations have been revised based on new rules in current specification. Three exemplary band combinations are provided below.

| CA configuration | Old notation | New notation | Note |
| --- | --- | --- | --- |
| Intra-band contiguous | SUL_n41C-n80A | CA_n41C-n80A | For the two NUL carriers within CA_n41C-n80A, the SUL band n80 is corresponding to one of them in one cell. |
| Intra-band non-contiguous | SUL_n41(2A)-n99A | CA_n41(2A)-n99A | For the two NUL carriers within CA_n41(2A)-n99A, the SUL band n99 is corresponding to one of them in one cell. |
| Inter-band | CA_n1A_SUL_n78A-n80A | CA_n1A_n78A-n80A | For the two NUL carriers within CA_n1A_n78A-n80A, the SUL band n80 is corresponding to n78 in one cell. |
| Intra-band with two SUL cells |  | CA_n78C_n81A-n84A |  |
| Inter-band with two SUL cells |  | CA_n41A-n95A_n79A-n98A |  |

5	SA NR SUL band combination: Specific Band Combination Part

5.1	CA_n1-n3_n78-n80

5.1.1	Operating bands

Table 5.1.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |
| CA_n1-n3_n78-n80 | n1, n3, n78, n80 |

5.1.2	Configuration

Table 5.1.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A_n78A-n80A | SUL_n78A-n80A | n1 | See n1 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 for each carrier |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 for each carrier |  |
|  |  | n80 | See n80 channel bandwidths in Table 5.3.5-1 for each carrier |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from TS 38.101-1. |  |  |  |  |

5.1.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.1.4	UE co-existence studies for 1 band UL

Table 5.1.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1-n3_n78-n80.

Table 5.1.4-1: Harmonic/Harmonic mixing analysis

| UL/DL harmonics |  | n80 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| n78 | fLow | fHigh | 1785 | 3570 | 5355 | 7140 | 8925 |  |
| DL1 | 3300 | 3800 | N/A | D | No | No | No | UL harmonic |
| DL23 | 6600 | 7600 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 11400 | No | No | N/A | No | N/A |  |
| DL4 | 13200 | 15200 | No | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 19000 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | The 2nd harmonic interference from band n80 may fall into band n78. |  |  |  |  |  |
| UL/DL harmonics |  | n80 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| n1 | fLow | fHigh | 1785 | 3570 | 5355 | 7140 | 8925 |  |
| DL1 | 2110 | 2170 | N/A | No | No | No | No | UL harmonic |
| DL23 | 4220 | 4340 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 6330 | 6510 | No | No | N/A | No | N/A |  |
| DL4 | 8440 | 8680 | No | N/A | N/A | N/A | N/A |  |
| DL54 | 10550 | 10850 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | There is no interference issues between UL band n80 and DL band n1. |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.1.4-2 summarizes frequency ranges where cross band isolation may occur for CA_n1-n3_n78-n80.

Table 5.1.4-2: Cross-band isolation analysis

| Bands3 | n78 |  | n80 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 3300 | 3800 | 1710 | 1785 |
| fDL (MHz) | 3300 | 3800 | NA | NA |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 10 | 100 | 5 | 50 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 3200 | 3900 | 1660 | 1835 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 3100 | 4000 | 1610 | 1885 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 3000 | 4100 | 1560 | 1935 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 2900 | 4200 | 1510 | 1985 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 2800 | 4300 | 1460 | 2035 |
| Analysis | There is no cross band isolation issue for this combo when UL band is band n78. |  | There is no cross band isolation issue for this combo when UL band is band n80. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.1.5	REFSENS requirements

The 2nd harmonic MSD requirements for DL band n78 affected by UL band n80 have been specified in the spec.

5.1.6	∆TIB and ∆RIB values

Table 5.1.6-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |  |
| CA_n1-n3_n78-n80 | 0.6 | 0.6 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n81 the order of band is n41 and n81. |  |  |  |  |

Table 5.1.6-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |  |
| CA_n1-n3_n78-n80 | 0.2 | 0.2 | 0.5 | N/A |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |  |  |

5.1.7	Out-of-band blocking exception

There is no Out-of-band blocking exception for this band combination.

5.2	CA_n1-n3_n78-n84

5.2.1	Operating bands

Table 5.2.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |
| CA_n1-n3_n78-n84 | n1, n3, n78, n84 |

5.2.2	Configuration

Table 5.2.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A_n78A-n84A | SUL_n78A-n84A | n1 | See n1 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 for each carrier |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 for each carrier |  |
|  |  | n84 | See n84 channel bandwidths in Table 5.3.5-1 for each carrier |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from TS 38.101-1. |  |  |  |  |

5.2.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.2.4	UE co-existence studies for 1 band UL

Table 5.2.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1-n3_n78-n84.

Table 5.2.4-1: Harmonic/Harmonic mixing analysis

| UL/DL harmonics |  | n84 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| n78 | fLow | fHigh | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 3300 | 3800 | N/A | No | No | No | No | UL harmonic |
| DL23 | 6600 | 7600 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 11400 | No | No | N/A | No | N/A |  |
| DL4 | 13200 | 15200 | No | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 19000 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | There is no interference issues between UL band n84 and DL band n78. |  |  |  |  |  |
| UL/DL harmonics |  | n84 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| n1 | fLow | fHigh | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 1805 | 1880 | N/A | No | No | No | No | UL harmonic |
| DL23 | 3610 | 3760 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 5415 | 5640 | No | No | N/A | No | N/A |  |
| DL4 | 7220 | 7520 | No | N/A | N/A | N/A | N/A |  |
| DL54 | 9025 | 9400 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | There is no interference issues between UL band n84 and DL band n3. |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.2.4-2 summarizes frequency ranges where cross band isolation may occur for CA_n1-n3_n78-n84.

Table 5.2.4-2: Cross-band isolation analysis

| Bands3 | n78 |  | n84 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 3300 | 3800 | 1920 | 1980 |
| fDL (MHz) | 3300 | 3800 | NA | NA |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 10 | 100 | 5 | 50 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 3200 | 3900 | 1870 | 2030 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 3100 | 4000 | 1820 | 2080 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 3000 | 4100 | 1770 | 2130 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 2900 | 4200 | 1720 | 2180 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 2800 | 4300 | 1670 | 2230 |
| Analysis | There is no cross band isolation issue for this combo when UL band is band n78. |  | The UL 1st and 2nd adjacent channels from UL band n84 may fall into the DL band n3. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.2.5	REFSENS requirements

The cross band isolation MSD requirements for DL band n3 affected by UL band n84 have been specified in the spec.

5.2.6	∆TIB and ∆RIB values

Table 5.2.6-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |  |
| CA_n1-n3_n78-n84 | 0.6 | 0.6 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n81 the order of band is n41 and n81. |  |  |  |  |

Table 5.2.6-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |  |
| CA_n1-n3_n78-n84 | 0.2 | 0.2 | 0.5 | N/A |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n84 the order of band is n41 and n84. |  |  |  |  |

5.2.7	Out-of-band blocking exception

There is no Out-of-band blocking exception for this band combination.

5.3	SUL_n80-n104

5.3.1	Operating bands

Table 5.3.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |
| SUL_n80A-n104A | n104, n80 |

5.3.2	Configuration

Table 5.3.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n80A-n104A | n80 | See n80 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  | n104 | See n104 channel bandwidths in Table 5.3.5-1 for each carrier |  |

5.3.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.3.4	UE co-existence studies for 1 band UL

Table 5.3.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n80-n104.

Table 5.3.4-1: Harmonic/Harmonic mixing analysis

| UL/DL harmonics |  | n80 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| n104 | fLow | fHigh | 1785 | 3570 | 5355 | 7140 | 8925 |  |
| DL1 | 6425 | 7125 | N/A | - | - | D | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Direct hit n80 UL4 / n104 DL1, and no near miss n80 ULx / n104 DLy (5MHz) |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.3.4-2 summarizes frequency ranges where cross band isolation may occur for SUL_n80-n104.

Table 5.3.4-2: Cross-band isolation analysis

| Bands3 | n80 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1710 | 1785 | 6425 | 7125 |
| fDL (MHz) | NA | NA | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1660 | 1835 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1610 | 1885 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1560 | 1935 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1510 | 1985 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1460 | 2035 | 5925 | 7625 |
| Analysis | There are no cross-band isolation problem for n80 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n80 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There is no cross band isolation issue for this combo when UL band is band n104. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.3.5	REFSENS requirements

The 4th harmonic MSD requirements for DL band n104 affected by UL band n80 can be referred to that for CA_n3-n104 as follows.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n80 | n104 | 5 | 15 | 6 | 20 | 11.7 | NOTE X | UL4/DL1 direct-hit |
| n80 | n104 | 5 | 15 | 6 | 100 | 6.1 | NOTE X | UL4/DL1 direct-hit |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

5.3.6	∆TIB and ∆RIB values

Table 5.3.6-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n80-n104 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

Table 5.3.6-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n80-n104 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

5.3.7	Out-of-band blocking exception

There is no Out-of-band blocking exception for SUL_n80-n104.

5.4	SUL_n81-n104

5.4.1	Operating bands

Table 5.4.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |
| SUL_n81A-n104A | n104, n81 |

5.4.2	Configuration

Table 5.4.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n81A-n104A | n81 | See n81 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  | n104 | See n104 channel bandwidths in Table 5.3.5-1 for each carrier |  |

5.4.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.4.4	UE co-existence studies for 1 band UL

Table 5.4.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n81-n104.

Table 5.4.4-1: Harmonic/Harmonic mixing analysis

| UL/DL harmonics |  | n81 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 880 | 1760 | 2640 | 3520 | 4400 |  |
| n104 | fLow | fHigh | 915 | 1830 | 2745 | 3660 | 4575 |  |
| DL1 | 6425 | 7125 | N/A | - | - | - | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit nor near miss n81 ULx / n104 DLy (5MHz) |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.4.4-2 summarizes frequency ranges where cross band isolation may occur for SUL_n80-n104.

Table 5.4.4-2: Cross-band isolation analysis

| Bands3 | n81 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 880 | 915 | 6425 | 7125 |
| fDL (MHz) | NA | NA | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 35 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 845 | 950 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 810 | 985 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 775 | 1020 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 740 | 1055 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 705 | 1090 | 5925 | 7625 |
| Analysis | There are no cross-band isolation problem for n81 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n81 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There is no cross band isolation issue for this combo when UL band is band n104. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.4.5	∆TIB and ∆RIB values

Table 5.4.5-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n81-n104 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

Table 5.4.5-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n81-n104 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

5.4.6	Out-of-band blocking exception

Since band n81 is a low band and n104 is a high and wide band, the OOBB exception is needed.

Table 5.4.6-1: SUL operating band combination with exceptions allowed

CA band combination

SUL_n81-n104

5.5	CA_n5_n78-n89

5.5.1	Operating bands

Table 5.5.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |
| CA_n5_n78-n89 | n5, n78, n89 |

5.5.2	Configuration

Table 5.5.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A_n78A-n89A | SUL_n78A-n89A | n5 | See n5 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 for each carrier |  |
|  |  | n89 | See n89 channel bandwidths in Table 5.3.5-1 for each carrier |  |
| CA_n5A_n78C-n89A | SUL_n78A-n89A CA_n78C-n89A | n5 | See n5 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 for each carrier |  |
|  |  | n89 | See n89 channel bandwidths in Table 5.3.5-1 for each carrier |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from TS 38.101-1. |  |  |  |  |

5.5.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.5.4	UE co-existence studies for 1 band UL

Table 5.5.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5_n78-n89.

Table 5.5.4-1: Harmonic/Harmonic mixing analysis

| UL/DL harmonics |  | n89 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 824 | 1648 | 2472 | 3296 | 4120 |  |
| n78 | fLow | fHigh | 849 | 1698 | 2547 | 3396 | 4245 |  |
| DL1 | 3300 | 3800 | N/A | - | - | D | - | UL harmonic |
| DL23 | 6600 | 7600 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 11400 | - | - | N/A | - | N/A |  |
| DL4 | 13200 | 15200 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 19000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Direct hit n89 UL4 / n78 DL1, and no near miss n89 ULx / n78 DLy (5MHz) |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.5.4-2 summarizes frequency ranges where cross band isolation may occur for CA_n5_n78-n89.

Table 5.5.4-2: Cross-band isolation analysis

| Bands3 | n78 |  | n89 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 3300 | 3800 | 824 | 849 |
| fDL (MHz) | 3300 | 3800 | NA | NA |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 10 | 100 | 5 | 20 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 3200 | 3900 | 804 | 869 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 3100 | 4000 | 784 | 889 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 3000 | 4100 | 764 | 909 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 2900 | 4200 | 744 | 929 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 2800 | 4300 | 724 | 949 |
| Analysis | There is no cross band isolation issue for this combo when UL band is band n78. |  | There is no cross band isolation issue for this combo when UL band is band n89. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.5.5	REFSENS requirements

The 4th harmonic MSD requirements for DL band n78 affected by UL band n89 has been specified for SUL_n78-n89.

5.5.6	∆TIB and ∆RIB values

Table 5.5.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n5_n78-n89 | 0.6 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 5.5.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n5_n78-n89 | 0.2 | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

5.5.7	Out-of-band blocking exception

There is no Out-of-band blocking exception for CA_n5_n78-n89.

5.6	SUL_n83-n104

5.6.1	Operating bands

Table 5.6.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |
| SUL_n83A-n104A | n104, n83 |

5.6.2	Configuration

Table 5.6.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n83A-n104A | n83 | See n83 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  | n104 | See n104 channel bandwidths in Table 5.3.5-1 for each carrier |  |

5.6.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.6.4	UE co-existence studies for 1 band UL

Table 5.6.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n83-n104.

Table 5.6.4-1: Harmonic/Harmonic mixing analysis

| UL/DL harmonics |  | n83 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 703 | 1406 | 2109 | 2812 | 3515 |  |
| n104 | fLow | fHigh | 748 | 1496 | 2244 | 2992 | 3740 |  |
| DL1 | 6425 | 7125 | N/A | - | - | - | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit nor near miss n83 ULx / n104 DLy (5MHz) |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.6.4-2 summarizes frequency ranges where cross band isolation may occur for SUL_n83-n104.

Table 5.6.4-2: Cross-band isolation analysis

| Bands3 | n83 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 703 | 748 | 6425 | 7125 |
| fDL (MHz) | NA | NA | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 653 | 798 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 603 | 848 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 553 | 898 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 503 | 948 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 453 | 998 | 5925 | 7625 |
| Analysis | There is no cross-band isolation issue for n83 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n83 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There is no cross-band isolation issue for this combo when UL band is band n104. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.6.5	REFSENS requirements

Based on the analysis in the co-existence study, no additional MSD requirements are expected to be specified.

5.6.6	∆TIB and ∆RIB values

Referring to the CA_n28-n79, the ΔTIB,c and ΔRIB,c values for SUL_n83-n104 can be specified below.

Table 5.6.6-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n83-n104 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

Table 5.6.6-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n83-n104 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

5.6.7	Out-of-band blocking exception

There is no Out-of-band blocking exception for SUL_n83-n104.

5.7	SUL_n84-n104

5.7.1	Operating bands

Table 5.7.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |
| SUL_n84A-n104A | n104, n84 |

5.7.2	Configuration

Table 5.7.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n84A-n104A | n84 | See n84 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  | n104 | See n104 channel bandwidths in Table 5.3.5-1 for each carrier |  |

5.7.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.7.4	UE co-existence studies for 1 band UL

Table 5.7.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n84-n104.

Table 5.7.4-1: Harmonic/Harmonic mixing analysis

| UL/DL harmonics |  | n84 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| n104 | fLow | fHigh | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 6425 | 7125 | N/A | - | - | - | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit nor near miss n84 ULx / n104 DLy |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.7.4-2 summarizes frequency ranges where cross band isolation may occur for SUL_n84-n104.

Table 5.7.4-2: Cross-band isolation analysis

| Bands3 | n84 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1920 | 1980 | 6425 | 7125 |
| fDL (MHz) | NA | NA | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1870 | 2030 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1820 | 2080 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1770 | 2130 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1720 | 2180 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1670 | 2230 | 5925 | 7625 |
| Analysis | There is no cross-band isolation issue for n84 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n84 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There is no cross-band isolation issue for this combo when UL band is band n104. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.7.5	REFSENS requirements

Based on the analysis in the co-existence study, no additional MSD requirements are expected to be specified.

5.7.6	∆TIB and ∆RIB values

Referring to the CA_n1-n102, the ΔTIB,c and ΔRIB,c values for SUL_n84-n104 can be specified below.

Table 5.7.6-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n84-n104 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

Table 5.7.6-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n84-n104 | N/A | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

5.7.7	Out-of-band blocking exception

There is no Out-of-band blocking exception for SUL_n84-n104.

5.8	SUL_n89-n104

5.8.1	Operating bands

Table 5.8.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |
| SUL_n89A-n104A | n104, n89 |

5.8.2	Configuration

Table 5.8.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n89A-n104A | n89 | See n89 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  | n104 | See n104 channel bandwidths in Table 5.3.5-1 for each carrier |  |

5.8.3	Maximum output power

There is only single UL in uplink, so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.8.4	UE co-existence studies for 1 band UL

Table 5.8.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n89-n104.

Table 5.8.4-1: Harmonic/Harmonic mixing analysis

| UL/DL harmonics |  | n89 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 824 | 1648 | 2472 | 3296 | 4120 |  |
| n104 | fLow | fHigh | 849 | 1698 | 2547 | 3396 | 4245 |  |
| DL1 | 6425 | 7125 | N/A | - | - | - | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit nor near miss n89 ULx / n104 DLy |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.8.4-2 summarizes frequency ranges where cross band isolation may occur for SUL_n89-n104.

Table 5.8.4-2: Cross-band isolation analysis

| Bands3 | n89 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 824 | 849 | 6425 | 7125 |
| fDL (MHz) | NA | NA | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 20 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 804 | 869 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 784 | 889 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 764 | 909 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 744 | 929 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 724 | 949 | 5925 | 7625 |
| Analysis | There is no cross-band isolation issue for n89 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n89 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There is no cross-band isolation issue for this combo when UL band is band n104. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.8.5	REFSENS requirements

Based on the analysis in the co-existence study, no additional MSD requirements are expected to be specified.

5.8.6	∆TIB and ∆RIB values

Referring to the CA_n5-n78, the ΔTIB,c and ΔRIB,c values for SUL_n89-n104 can be specified below.

Table 5.8.6-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n89-n104 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

Table 5.8.6-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_n89-n104 | N/A | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

5.8.7	Out-of-band blocking exception

For operation band combination SUL_n89-n104, exceptions to the requirement specified in Table 7.6C.3-2 in TS38.101-1 are allowed when the second order intermodulation product of the SUL carrier and the CW interfering signal fully or partially overlaps with the DL carrier.

Table 7.6C.3-2 （in TS38.101-1）: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when ¦fInterferer± fSUL- fDL¦≤(BWSUL+ BWDL)/2, where BWSUL and BWDL are the channel bandwidths configured for SUL and DL (victim) bands in MHz, respectively. |  |  |

5.Y	SUL_nX-nY/ CA_nX_nY-nZ/ CA_nX-nW_nY-nZ

5.Y.1	Operating bands

Table 5.Y.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |

5.Y.2	Configuration

Table 5.Y.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
|  |  |  |  | 0 |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from TS 38.101-1. |  |  |  |  |

5.Y.3	Maximum output power

<Editor’s note: This requirement is only applicable when there is simultaneous transmission in the band combination.>

5.Y.4	UE co-existence studies for 1 band UL

<Editor’s Note: The tables in this section are provided to identify potential issues to be analysed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.>

Table 5.Y.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_ nX-nY.

| UL/DL harmonics |  | nX | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow |  |  |  |  |  |  |
| nY | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow |  |  |  |  |  |  |
| nX | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.Y.4-2 summarizes frequency ranges where cross band isolation may occur for SUL_nX-nY.

Table 5.Y.4-2: Cross-band isolation analysis

| Bands3 | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) |  |  |  |  |
| fDL (MHz) |  |  |  |  |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) |  |  |  |  |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) |  |  |  |  |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) |  |  |  |  |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) |  |  |  |  |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) |  |  |  |  |
| Analysis |  |  |  |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.Y.5	REFSENS requirements

<Editor's note 1: Text may be added if harmonics/harmonic mixing/cross band isolation issues are identified in subclause 5.Y.3.

Editor's note 2: Proponent are encouraged to provide the detailed technical analysis of the MSD requirements. For example: RF architecrture, RFcomponents parameter, and etc. In the case where the proponent is missing some elements to calculate the REFSENS exception cases, the TP can be submitted to the “not for block approval” AI for expert support.

Editor's note 3: The table format shall be the same with the corresponding tables in TS38.101-1, i.e.: Table 7.3C.2-1, Table 7.3C.2-2, Table 7.3C.2-4 for harmonics, harmonic mixing and cross band isolation, respectively.>

5.Y.6	∆TIB and ∆RIB values

Table 5.Y.6-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_nX-nY |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n81 the order of band is n41 and n81. |  |  |

Table 5.Y.6-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_nX-nY |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

5.Y.7	Out-of-band blocking exception

<Editor’s note: This requirement is only applicable when there is Out-of-band blocking exception in the band combination.>

Table 5.Y.7-1: SUL band combination with exceptions allowed

SUL band combination

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | 3GPP RAN4#112 | R4-2412929 |  |  |  | Initial TR skeleton | 0.0.1 |
| 2024-08 | 3GPP RAN4#112 | R4-2412930 |  |  |  | TR for 38.719-00-00 v0.1.0 R4-2412936 TP for TR 38.719-00-00 on introduction of CA_n1A-n3A_n78A-n80A Huawei, HiSilicon R4-2412937 TP for TR 38.719-00-00 on introduction of CA_n1A-n3A_n78A-n84A Huawei, HiSilicon | 0.1.0 |
| 2024-10 | 3GPP RAN4#112bis | R4-2416028 |  |  |  | withdrawn | 0.2.0 |
| 2025-10 | 3GPP RAN4#116bis | R4-2509895 R4-2513394 |  |  |  | TR for 38.719-00-00 v0.3.0 R4-2510472 TP for TR 38.719-00-00 SUL_n80-n104, Huawei, HiSilicon R4-2510473 TP for TR 38.719-00-00 SUL_n81-n104, Huawei, HiSilicon R4-2510474 TP for TR 38.719-00-00 CA_n5_n78-n89, Huawei, HiSilicon R4-2513392 Draft TP to TR 38.719-00-00 include SUL_n83A-n104A, Huawei, HiSilicon | 0.3.0 |
| 2025-11 | 3GPP RAN4#117 | R4-2520424 |  |  |  | TR for 38.719-00-00 v0.4.0 R4-2522348 Draft TP to TR 38.719-00-00 include SUL_n84A-n104A, Huawei, HiSilicon R4-2522349 Draft TP to TR 38.719-00-00 include SUL_n89A-n104A, Huawei, HiSilicon | 0.4.0 |