# 3GPP TSG-RAN WG4 Meeting #117	                                                         R4-2520625

# Dallas, USA, Nov. 17-21, 2025

Source:	Apple

Title:	Discussion on 6G AI/ML Framework and General Issues

Agenda item:	8.8.1

Document for:	Discussion

Introduction

Following the agreements reached at RAN4#116bis on the 6G AI/ML study scope and timeline [R4-2514637], we provide our inputs under Topic #2 RAN4 AI/ML framework.

The topics of discussion are:

- Post-deployment enhancement

- Generalization issues

Discussion On Post-deployment LCM enhancement

## Current Post-Deployment Framework

The existing post-deployment framework for AI/ML models in 3GPP remains fundamentally reactive, relying on two validation approaches. Option 1 (pre-activation testing) validates updated models in controlled lab environments or on-device test modes before activation, establishing KPI envelopes through standardized procedures similar to RAN5 conformance testing.

Option 2 (post-activation monitoring) uses LCM procedures to collect live performance metrics and detect degradation by comparing them against predefined thresholds.

A hybrid Option 1 + Option 2 approach is sometimes proposed: major updates (architecture change, new RF front-end, dataset shift) require pre-activation validation, while all updates are subject to continuous post-activation monitoring. However, the framework is still reactive: degradation is only detected after KPI violations occur, meaning users experience reduced performance before the system takes action.

The only corrective action is to revert to a legacy non-AI baseline. There is no standardized way to detect drift early, no proactive mitigation, no model-switching logic, and no mechanism for in-field adaptation. The framework detects failure,  it does not prevent it.

Enhanced Framework: with Proactive Tier-Based Model Adaptation

it is desirable to have proactive model degradation management to replace the reactive failure detection. Instead of waiting for KPI failure, the UE continuously monitors internal early-warning signals such as confidence decay, input distribution drift, rising uncertainty, and temporal instability.

Presumably, there are four tiers of model degradations defined

- Tier 0 — Normal: model healthy, monitoring only

- Tier 1 — Early Alert: drift detected, prepare alternatives (shadow validation)

- Tier 2 — Proactive Intervention: switch models/adapters or apply online learning before KPI drop

- Tier 3 — Reactive Recovery: only if KPI violation occurs → mandatory fallback to validated baseline

depending on LCM, UE may enter a higher “tier” from a lower tiers and begins corrective action. This can preserve reliability since most degradations are fixed in Tier 1 or Tier 2 before user impact, while Tier 3 provides a guaranteed safety net.

The tier-based state machine enables proactive model management because it gives the UE a graded response system instead of the current all-or-nothing “works until it fails” approach. In today’s reactive framework, the UE only takes action after KPI failure is detected meaning the model is already performing poorly and users are already impacted.

Figure 2: Proactive Tier-Based AI/ML model LCM

- RAN4 to introduce a proactive post-deployment framework for AI/ML model management.

Discussion On Generalization of AI/ML models

Current AI/ML models used for mobility, RSRP prediction, or beam/event forecasting are tightly coupled to the physical radio configuration of the cell on which they were trained. Since RSRP patterns, beam shapes, and propagation behavior are shaped by vendor-specific parameters such as antenna geometry, beamwidth, codebook design, tilt, transmit power, panel arrangement, polarization schemes, and frequency-specific beam steering characteristics, a model trained on one vendor's deployment cannot reliably generalize to another. This forces each operator or vendor to maintain separate models per site, per configuration, or even per frequency layer, causing model fragmentation, exponential retraining costs as network complexity grows, and failure of cross-deployment inference. Additionally, this fragmentation prevents effective federated learning or collaborative model improvement across operators, since models remain locked to specific infrastructure profiles and cannot benefit from aggregated multi-network training data.

What breaks generalization is not the AI technique itself, but the fact that the model is learning implicit, hidden assumptions about the infrastructure: wide versus narrow beams, analog versus digital beamforming, 8-beam versus 32-beam codebooks, macro versus small-cell geometry, multi-TRP versus single-TRP configurations, or dynamic network energy saving (NES) mode reconfigurations that alter beam patterns and transmission power dynamically.

Intuitively, if a semantic abstraction can encode deployment characteristics such as BeamWidth, TX_Codebook etc. in a vendor-agnostic format, there is a chance that a single AI/ML model conditions its predictions by using the semantic abstraction as a structured side-input describing why the radio behaves as observed. Vendors can map their proprietary internal configurations to these standardized classes, preserving confidentiality while enabling one shared model to adapt across heterogeneous cells, sites, and equipment. The semantic abstraction actually serves as a "configuration fingerprint" that makes infrastructure characteristics observable to the AI model without exposing vendor-proprietary implementation details such as specific antenna element spacing, phase shifter designs, or beamforming algorithms.

- Study on how semantic abstract of the deployment characteristics can help to address the model generalization issues.

Conclusions

In this paper we made the following proposals:

- RAN4 to introduce a proactive post-deployment framework for AI/ML model management.

- Study on how semantic abstract of the deployment characteristics can help to address the model generalization issues.

References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR, Draft Technical Report, RAN #109.

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Technical Report