3GPP TSG-RAN WG4 Meeting #117	R4-2520706

Dallas, USA, November 17th –  21st, 2025

Source:	Nokia

Title:	RRM core requirements for AI/ML Based Beam Management

Agenda item:	6.11.4

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#116bis in the scope of Rel-19 WI NR_AIML_air ‎[1]. Agreements achieved during RAN4#116bis meeting are summarized in the WF ‎[2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion. In this paper, we provide some additional views on issues related to RRM core requirements and testability aspects, as listed below:

- Prediction reporting delay

- Testing framework for AI/ML based beam management

- Simplification of CDL model

Discussion

Prediction reporting delay

Reporting delay

Agreement:

Reuse d’ as to be agreed by RAN1

d’ to be added to the measurement period to reflect the extra time needed by the UE to perform inference

Depending on the RAN1 specifications, d’ shall not be double counted in the overall delay for periodic, aperiodic or semi-persistent reporting

Final specifications to be agreed after RAN1 includes d’ in the specs

Measurement period for inference – case2

Agreement:

K(number of samples)*[M]*N*P

K as defined by RAN1

The following agreement was reached in RAN4#116bis meeting:

Since the formula for prediction reporting delay was agreed in RAN4#116bis meeting, we have provided the relevant specifications change for both SSB based and CSI-RS based beam prediction in our accompanying CR on RRM requirements for beam management use case with NR AI/ML Air Interface [3] based on the endorsed draft CR [4] in RAN4#116bis meeting.

RAN4 to consider corrections for prediction reporting delay requirements for both SSB based and CSI-RS based beam prediction captured in CR [R4-2521469]

Testing framework for AI/ML based Beam Management

In RAN4#116bis meeting, test system setup for the conformance testing of AI/ML based beam management (BM) use case was finalized and the corresponding agreements are provided below for reference.

Agreements on Test System Setup: [RAN4#114bis] Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified. [RAN4#116] Consider multi AoA based RRM testing setup as baseline (enhanced IFF system with 4 probes used for FR2 RRM conformance testing) - Single AoA tests can also be performed in the multi AoA based RRM test setup Further analyze what spatial channel models can be emulated with this test setup - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup - Single AoA tests are not precluded [RAN4#116bis] A multi-AoA IFF based test setup with 4 active cross-polarized probes placed in the single plane (xz plane). The relative angular separation between neighbouring probes is 30º, 60º, and 60º, respectively.

However, the channel model to be used for the conformance test is still not clear and needs further discussion. The RAN4 agreements related to channel model are provided below.

Agreements on Channel Model: [RAN4#114] Use CDL-based channel model as starting point [RAN4#116] Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber. - Take CDL-C Umi as one example - Other channel models can be further discussed [RAN4#116] RAN4 will further discuss the simplified Uma and UMi channel model by taking the following aspects into consideration - Commercial test system limitation, including the number of probes and the separation - Model simplification should not result in inference performance degradation beyond the margin when the training is done based on the original models. - The margin can be discussed and decided further. - Other aspects are not precluded.

In the following, we provide Nokia’s further views on the topic and open issues after RAN4#116bis meeting.

The test system agreed by RAN4 for the conformance testing of AI/ML BM use case is a multi-AoA enhanced IFF (eIFF) test system equipped with 4 cross-polarized test probes which are placed in the single xz plane and are relatively separated from the adjacent test probe, respectively, by 30º, 60º, and 60º. This test system was earlier selected for the FR2 RRM conformance testing. However, the types of spatial channel models that can be supported by eIFF test system is still unclear and needs further investigation. This becomes even more important since RAN4 has already agreed to use CDL based channel model (full/simplified CDL) for the conformance testing of this use case.

For this, FR2 CDL-C Umi channel models (at 28 GHz) is reproduced below.

[TS 38.151 Annexure D.1] # D.1 FR2 Channel models The following channel model is required for FR2 MIMO OTA measurement. The generic model is Table D.1-1 FR2 UMi CDL-C, which does not include base station antenna filtering. Therefore, in addition, the BS beam filtering effect defined in Annex D.2 also apply when emulating the channel models. Table D.1-1: Channel model parameters for UMi CDL-C at 28 GHz Cluster #; Absolute Delay [ns]; Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -4.4215; -30.4353; -134.4434; 98.9242; 83.3318. 2; 12.594; -1.25; -20.9269; 129.1633; 99.1915; 72.5229. 3; 13.314; -3.4684; -20.9269; 129.1633; 99.1915; 72.5229. 4; 13.974; -5.2294; -20.9269; 129.1633; 99.1915; 72.5229. 5; 13.056; -2.5215; -28.0782; -152.8206; 99.5732; 71.1282. 6; 38.196; 0; -11.6982; 164.1145; 99.306; 74.7544. 7; 38.688; -2.2185; -11.6982; 164.1145; 99.306; 74.7544. 8; 39.36; -3.9794; -11.6982; 164.1145; 99.306; 74.7544. 9; 39.504; -7.4215; 17.3861; 84.3647; 100.4513; 69.2454. 10; 47.61; -7.1215; -37.5865; 92.0623; 98.5616; 66.7349. 11; 49.278; -10.7215; 20.2226; -97.7585; 100.6231; 72.0348. 12; 56.016; -11.1215; -50.6106; 78.4702; 98.218; 64.4337. 13; 73.71; -5.1215; -33.911; 93.1719; 100.165; 85.4238. 14; 78.498; -6.8215; -37.5066; -112.0441; 100.2604; 64.1548. 15; 130.224; -8.7215; -43.1797; 102.4645; 98.1225; 64.7824. 16; 162.63; -13.2215; 29.2116; 67.2359; 100.2604; 92.467. 17; 255.534; -13.9215; 27.8133; 34.5731; 98.4852; 65.6889. 18; 276.018; -13.9215; 23.6584; 48.5813; 98.1416; 68.7572. 19; 329.412; -15.8215; -52.5282; 36.4455; 97.9698; 59.1339. 20; 336.462; -17.1215; 25.0168; 52.6729; 100.7376; 65.3402. 21; 378.39; -16.0215; 25.4562; 49.8296; 98.1225; 58.4365. 22; 398.244; -15.7215; 30.7697; 46.4316; 98.1034; 65.2705. 23; 422.562; -21.6215; 35.9234; 30.759; 100.4513; 62.6903. 24; 519.138; -22.8215; -61.2775; 69.2469; 100.9476; 61.993. Per-Cluster Parameters. Parameter; CASD in [°]; CASA in [°]; CZSD in [°]; CZSA in [°]; XPR in [dB]. Value; 0.799; 10.4021; 0.5726; 4.8814; 7.

- To emulate UMi CDL-C channel model during the test, BS beam filtering effect defined in Annex D.2 of TS 38.151 is required, which is reproduced below.

[TS 38.151, Annexure D.2] # D.2 FR2 Base Station beam configuration The emulated BS beam configuration to be used for emulation of channel model defined in Annex D.1 is specified in this clause. The Base Station beam configuration includes basic antenna parameters and beamforming characteristic. The basic BS antenna parameters is defined in Table D.2-1. Table D.2-1: FR2 BS Antenna Parameters Parameter description; Symbol; Parameter value. FR2. Antenna panels in vertical dimension; Mg; 1. Antenna panels in horizontal dimension; Ng; 1. Elements per panel in vertical dimension; Me; 8. Elements per panel in horizontal dimension; Ne; 16. Number of polarizations per panel; P; 2. Element spacing in horizontal dimension (); dH; 0.5. Element spacing in vertical dimension (); dV; 0.5. Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array are as in the example pattern in Table 7.3-1 of TR 38.901. The antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi. The beamforming characteristic of the FR2 BS pattern is defined as follow: - A code book of 128 fixed beams is constructed to a grid of eight elevation angles from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size； - 1 strongest transmitting beam is generated from BS, the direction of this beam towards the strongest cluster of the FR2 channel model. Specifically, the direction in UMi CDL-C model is: AoD: -12o, ZoD: 100.71

- If the FR2 CDL-C Umi channel defined in Annexure D.1 of TS 38.151 is used for the conformance testing of AI/ML BM use case, the BS beam configuration defined in Annexure D.2 of TS 38.151 needs to be emulated. For this, a codebook of 128 fixed beams, based on a grid of eight elevation angles ranging from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size, is defined.

Furthermore, in RAN4#114 meeting, Set A = 32 beams and Set B = 8 beams was agreed as a starting point.

Select at least 32 Set A beams, to be emulated during the conformance testing of AI/ML BM use case, from the existing codebook of 128 fixed beams (constructed from a grid of eight elevation angles from –25o to +25 o with ~7.1 o step size and 16 azimuth angles from –60 o to +60 o with 8 o step size) along with the other parameters as defined in Table D.2-1 of TS 38.151.

Selection of at least 32 Set A beams from the codebook of 128 beams can be done is several possible ways. For example,

- By restricting the range of azimuth or elevation or both (azimuth and elevation) angles.

- By increasing the step size in azimuth or elevation or both (azimuth and elevation) directions.

- Test equipment dynamically selects 32 beams out of 128 in each test iteration. FFS if some information about this needs to be shared with the DUT during the test or not.

RAN4 to study the impact of selecting different combinations of at least 32 Set A beams from the 128-beam codebook on the performance with CDL based channel model.  How to select the at least 32 Set A beams from 128 beam codebook is FFS.

Moreover, how to emulate 32 Set A beams by using a limited number of test probes during the AI/ML BM conformance testing is a well acknowledged issue in RAN4. We next provide two candidate solutions along with the corresponding test procedures for the same.

      - Emulation of Set A and/or Set B beams without using UE rotation

- This solution provides a method, applicable only for the multiple AoAs based test systems, to emulate the Set A and/or Set B beams without using the UE rotation. Considering a test chamber equipped with ‘M’ test probe antennas where each test probe antenna is contributing to form a definite Tx antenna pattern and/or Tx beam(s) which is also affected by the amount of Tx power corresponding to the signals applied to each test probe antenna. A particular Tx antenna pattern (containing at least a Tx beam) can be formed by selecting and activating any of the ‘N’ test probes out of the available ‘M’ probes, where 1≤N≤M, i.e.,

- MC1 Tx antenna patterns can be formed if only 1 probe is selected/activated.

- …

- MCM Tx antenna patterns can be formed if ‘M’ probes are selected/activated.

- Hence, a test system with ‘M’ test probe antennas in the test chamber can emulate (MC1 + … +MCM) Tx antenna radiation patterns (or beams). For example, for a test system having 6 test probes, simultaneously activating at least 3 test probe antennas will result in the creation of (20+15+6+1) = 42 Tx antenna patterns which is sufficient to emulate 32 Set A beams during the conformance testing.

RAN4 to discuss and explore the feasibility of generating Set A and Set B beams during the conformance testing of the AI/ML-BM use case by selecting unique combinations of available test probes and decide the transmit power of each selected probe. Tx power of each selected probe is FFS.

This solution may need a special beam codebook for the generation of Set A and Set B beams using unique combinations of test probes during the conformance testing. To ensure that the UE can perform correct inferences using the measurement results of Set B beams generated using this method, a pre-alignment check may be needed before the exact start of the conformance test. The objective of the pre-alignment check is to ensure that the TE provided parameters (e.g., parameters corresponding to the beam codebook) and the DUT’s model/functionality/configuration’s current/preferred parameter (e.g., the parameters using which the training was performed) agree with each other. This is needed to ensure that the test does not fail because of the (AI/ML) parameter(s) mismatch between the DUT (pre)-training and conformance testing.

RAN4 to discuss and finalize the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

- We next provide a test procedure which utilizes this concept and will call it as “Test Procedure A”.

- Test Procedure A:

- Place the DUT inside the testing chamber in the pre-selected or standardized orientation.

- Perform the pre-alignment check between the TE and the DUT. If the pre-alignment check is successful, TE parameters and DUT training parameters are not mismatched. FFS the content of pre-alignment check. Example: parameters related to beam codebook for the testing.

- If the pre-alignment check is successful, go to Step 4 else end the test and mark it as not feasible/applicable.

- TE selects a unique combination of three or more test probes from the available test probes antennas and decides the Tx power of the corresponding signals applied to the selected test probes to form one Tx antenna pattern comprising one Set A (or Set B) beam.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- TE selects another unique combination of three or more test probes and decides the Tx power of the corresponding signals applied to the selected test probes to form another Tx pattern containing a different Set A (or Set B) beam.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- Repeat till all Set A (or Set B) beams are transmitted, and DUT has measured the RSRP of all transmitted beams.

- If Set A beams are transmitted first, DUT reports the ground truth RSRP of the measured Set A beams to the TE.

- TE stores the UE reported Set A measurement results for deriving the test verdict.

- TE transmits each Set B beams to the DUT by forming a unique Tx antenna pattern via selecting a unique combination of three or more test probes selected from the combinations used for transmitting Set A beams.

- DUT measures the RSRP of each Set B beam and perform the AI/ML based inference at the UE side.

- DUT reports the inference results back to the TE (e.g., reporting top-K beams along with their predicted RSRPs).

- TE derives the test verdict based on the UE reported prediction results and UE reported measurement results.

      - Emulation of Set A and/or Set B beams with UE rotation

- This solution provides a method, which is applicable for both single AoA and multiple AoAs based test systems, to emulate the Set A and/or Set B beams by using the concept of UE rotation. It is expected during the test that different Set A/B beams should be generated in a way that these will arrive at the UE antenna array from different directions as similar scenario will be observed in the real deployments. Hence, different UE rotations can be used during the test to ensure that different beams arrive the DUT from different directions. One benefit of using the UE rotation is that it enables the test system to emulate the AoA angles which are outside the angular range of test probes (from UE’s point of view) and hence the test system can generate any possible AoA beam angles.

The solution proposes the DUT to perform beam sweeping in multiple iterations and to measure the RSRP of the corresponding beams in Set A and/or Set B during the conformance test for which a new test command is to be defined and to be sent to the DUT. After the DUT receives this test command from the TE and configures itself to perform beam sweeping in multiple iterations, one of the possible UE orientations is selected and TE selects the power level (typically different) for each of the available test probes to form different beams towards the UE. The above solution is valid for emulating all Set A and/or Set B beams in an iterative manner to perform beam sweeping during the conformance testing.

Figure 2: An example of the proposed test system with UE rotation.

Given the Set A and/or Set B beams can be generated iteratively during the conformance test as proposed above, DUT can measure the L1-RSRP of each beam in Set B and provides it as input to its AI/ML model for Set A beam prediction.

RAN4 to discuss the feasibility of performing beam sweeping using UE rotation in multiple iterations during the conformance testing of AI/ML based BM use case.

- We next provide a test procedure which utilizes this concept and will call it as “Test Procedure B”.

- Test Procedure B:

- Place the DUT inside the testing chamber in the pre-selected or standardized orientation.

- TE sends the test command to the DUT to configure itself and perform beam sweeping in multiple iterations.

- DUT configures itself to perform beam sweeping in multiple iterations and sends an acknowledgement to the TE for the same.

- TE transmits one of the Set A (or Set B) beams to the DUT using the available test probes in the testing chamber. This means that for single AoA based test systems, the only available test probe will be used to transmit the required beam towards the DUT whereas for multi-AoA based test systems, either one or more available test probes will be selected based on the agreement on channel model to be used during the conformance test.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- Rotate the UE to a different orientation. Allow the UE to settle down.

- Transmit next Set A (or Set B) beams to the DUT.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- Repeat Steps 6-8 till all Set A (or Set B) beams are transmitted, and DUT has measured the RSRP of all transmitted beams.

- If Set A beams are transmitted first, DUT reports the ground truth RSRP of the measured Set A beams to the TE.

- TE stores the UE reported Set A measurement results for deriving the test verdict.

- TE transmits each Set B beams to the DUT using a subset of the UE orientations previously used to transmit Set A beams.

- DUT measures the RSRP of each Set B beam and performs the AI/ML based inference at the UE side.

- DUT reports the inference results back to the TE (e.g., reporting top-K beams along with their predicted RSRPs).

- TE derives the test verdict based on the UE reported prediction results and UE reported measurement results.

Simplification of CDL model

Spatial filter applied to a normal CDL model

The CDL model UMi CDL-C is an omnidirectional channel model, where clusters are distributed in all directions of the channel scenario, as illustrated in Figure 1. However, due to the limited beamwidth of the SSB beam, it can only interact with a subset of clusters in such a scenario. To address this, a spatial filter needs to be applied according to the SSB beamwidth. This procedure is described in 3GPP TR 38.901, Chapter 7.7.4 “Spatial filter for generating TDL channel model”. The extension of using the spatial filter to generate a CDL model has been developed in SCM WI in Rel-19, R4-2512646.

By applying the spatial filter based on the SSB beamwidth and the UE Rx beam’s beamwidth, only the clusters that interact with the corresponding SSB and UE Rx beam are retained, which resulting in significantly fewer than the 24 clusters defined in the UMi CDL-C model.

Based on the reflection and diffusion of the SSB from the gNB to the UE and its reception by the Rx beam, the power angular spectrum (PAS) represents the “FR2 channel” that needs to be emulated in the anechoic chamber using probes. Since the PAS depends on the number of clusters, methods for generating it with a limited number of probes in the chamber should be further discussed.

Figure 1 Directional channel based on a CDL model

A spatial filter should be applied to the UMi CDL-C channel model based on the SSB and UE Rx beam, so that only the clusters interacting with them are retained. In this case, the number of clusters will be significantly fewer than the 24 clusters defined in the UMi CDL-C model.

Based on the reflection and diffusion of the SSB from gNB to UE and its reception by the Rx beam, the power angular spectrum (PAS) represents the “FR2 channel” that needs to be emulated in the anechoic chamber using probes.

Spatial filter applied to a reduced-clusters CDL model

Regarding the simplification of the CDL model discussed in previous meetings, it is straightforward to create a similar cluster map while reducing the number of clusters to six, as illustrated in Figure 2. The same spatial filtering procedure should then be applied to this reduced-cluster CDL model.

Figure 2 Directional channel based on a reduced-cluster CDL model

Each CDL channel model is created based on a specific channel scenario, with defined cluster locations, angles, delays, and powers that preserve the agreed channel characteristics, for example, the agreed UMi CDL-C model reflects the properties of the UMi scenario. Reducing the number of clusters in a CDL model can significantly alter these properties.  potentially shifting the UMi characteristics toward those of UMa, or even resulting in undefined channel properties.

Spatial filter should also be applied to the reduced-cluster channel model according to the beamwidth of SSB and UE Rx beam, so that the interacting clusters can be identified, and the rest of clusters can be ignored.

Reducing the number of clusters in the CDL model carries a high risk of significantly altering the channel properties, potentially shifting the UMi characteristics to those of UMa or to an undefined channel property.

It is not recommended to reduce the number of clusters in the UMi CDL-C model for simplification. There is high risk of significantly altering the channel properties, potentially shifting the UMi characteristics to those of UMa or to an undefined channel property.

    - Alternative channel model “rCDL-C1” with 12 clusters, 38.753

The rCDL-C1 channel model parameters are derived based on 38.901 CDL-C, Uma scenario parameters in 38.827, 3.5 GHz frequency with the desired delay and angle spread applied and truncated to 12 significant clusters.

The rCDL-C1 model’s full clusters can be found as in Table 5.1.3.3.1-1: Channel model parameters for rCDL-C1 from 38.753. This can also possible to be considered as channel scenario for BM test case.

However, due to this model is only used for single-user MIMO (SU-MIMO) cases, and some of the channel characteristics have been removed from the original CDL model in 38.901. Therefore, it is not recommended to use it with large bandwidth test cases.

rCDL-C1 model is only used for single-user MIMO (SU-MIMO) cases, and some of the channel characteristics have been removed from the original CDL model in 38.901. Therefore, it is not recommended to use it with large bandwidth test cases.

Table 5.1.3.3.1-1: Channel model parameters for rCDL-C1 (38.753)

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

The rCDL-C1 channel model in 38.753 with only “12” clusters in total can be considered to generate BM test case scenarios. However, it is only for SU-MIMO cases. It is not suggested to use it for BM testing case with large bandwidth.

Alternative way of generating cluster map by using real measurement date or ray tracing scenario

- Based on the offline discussion, we may also considering to simply generate cluster map based on the a channel scenario through a real channel measurement data or ray tracing model. The purpose of doing so means to reduce the total clusters that need to be represented in the TE chambers.

- Considering the FR2 base station coverage can be relatively small, such as a few hundred meters or less than 100 meters. So, the objects in such coverage can be much less.

- In the real channel measurement: due to the small FR2 cell, the channel scenario is relatively simple, and the clusters will be fewer than 24 clusters.

- Based on ray tracing simulation, we can find the scenarios that with less clusters than 24.

- If using those realistic channel measurement or ray tracing model, the scenarios that contain less objects/clusters can be considered as test environment.  So, we don’t need to start with 24 clusters CDL-A channel model.

The real FR2 channel measurements and the ray tracing model can be considered to create the cluster-map with less clusters other than CDL-A model with 24 clusters.

Test case creation

- For creating the test cases based on CDL-A channel model (rCDL-C1 model) or based on ray tracing model, real measurements, we can consider to place the gNB in the center, and place the UEs around the gNB as below picture. The 24 clusters are around gNB, and there are some clusters surrounding each UE locations,

Figure 4 test cases example

So, based on the delay described in 38.901 table 7.7.3-1, the clusters distance from gNB and UEs can be scaled. In the case that the clusters are 100-200 meters away, then, only the clusters around UE will be illuminated due to the high attenuation of FR2 frequency. So, based on the figure here, we can create 2 test cases:

- UE at the left-hand side of the gNB, and the PAS1 needs to be presented in the TE chamber, or present those illuminated clusters instead of PAS1.

- UE at the right-hand side of the gNB, and the PAS2 needs to be presented in the TE chamber, or present those illuminated clusters instead of PAS2.

Test case creation can be based on the illuminated clusters according to the UE location and gNB location. The clusters far away (a few hundred meters away, based on the scaling of delay in 38.901 table 7.7.3.-1) from the UE will not be illuminated.

Setup difference of TE probes weights for different test cases:

For those above-mentioned a few test cases, the TE chamber and the probes are the same; the only setup that needs to be changed is the weights of the probes to present the PAS or clusters based on the UE locations.

Using the same TE chamber and the same probes to present the illuminated clusters. The setting of the weights for the probes need to be changed based on the illuminated clusters in each test case.

Use the same probes of a TE chamber with different setting for weights of the probes to generate different test cases according to the illuminated clusters.

Procedure of representing clusters interact with clusters in an anechoic chamber (based on the CDL-A model)

Based on the previous analysis, the spatial filter will inevitably be applied to either the UMi CDL-C model or its reduced version. Therefore, it is recommended to follow the procedure based on the original UMi CDL-C model to avoid the risk of altering the UMi channel properties into those of UMa or an undefined channel property.

1. The CDL-C UMi models, as agreed in the WF, are omnidirectional, meaning the 24 clusters are distributed across the entire channel environment around the gNB and UE. Since we are using beams, the SSB/CSI-RS from the gNB will only interact with clusters in certain directions. Therefore, we propose to retain all 24 clusters and not remove any of them.

2. Next, we can generate or simulate the SSB beams, starting with 32 beams as agreed. Each SSB beam has a beamwidth of only 11.25°, meaning it will interact with only a small subset of the clusters in the CDL-C UMi model, which is significantly fewer than all 24 clusters. The SSB then propagates to the UE and is received according to the UE’s power angular spectrum (Rx-PAS). Based on this, we can determine which clusters contribute for a given SSB or test case in the CDL-C model, using those clusters while ignoring the others.

3. At this step, we can also check whether the power of the clusters contributing to the UE’s Rx-PAS is too weak. If so, these clusters can be ignored as well.

4. When creating the Rx-PAS in the anechoic chamber, we can simulate or emulate only the clusters that interact with the SSB beam, while also applying the condition that the cluster power is not too weak. Consequently, the number of clusters that need to be generated by the probes in the chamber for each test case is much fewer than 24.

Throughout the entire procedure, we do not remove any clusters from the CDL model directly; instead, we simply ignore the clusters that are not used. This ensures that the underlying channel properties remain unchanged.

The above procedure can be applied to any test case. If a test case requires generating a large number of SSBs simultaneously, we can consider developing methods to generate more than 4–8 clusters, which is more than the limited number of probes available in the chamber.

A spatial filter should be applied to the CDL-C model first, which can significantly reduce the number of clusters. Next, clusters with insufficient power can be ignored, further reducing the number of clusters. Only the remaining clusters need to be represented in the chamber using probes.

Endorse the above-mentioned 4-step procedure for representing the clusters in the chamber using probes.

The threshold of weak clusters that can be ignored

There are reference sensitivity power levels defined in 3GPP TS 38.101-2, Chapter 7.3.2 (“Reference Sensitivity Power Level”). According to the standard:

“The reference sensitivity power level REFSENS is defined as the EIS level at the centre of the quiet zone in the RX beam peak direction, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.”

This means that REFSENS represents a lower bound for the power level at which the UE should be able to successfully receive the signal from the gNB.

Here is an example of REFSENS for PC3 in 3GPP TS 38.101-2, for different signal bandwidths. We can map these reference sensitivity levels to channel scenarios (e.g., UMi CDL-C) when SSB or CSI-RS signals are transmitted from the gNB. In the anechoic chamber, any cluster with a power level below the corresponding REFSENS can be safely ignored.

The reference sensitivity power levels can be reused to determine whether the power of a cluster is too weak and can therefore be ignored.

Conclusion

In this paper we share our views on RAN4 impacts from issues related to RRM core requirements and testability aspects for AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

- Prediction reporting delay

- Testing framework for AI/ML based beam management

- Simplification of CDL model

In the paper, the following Observations and Proposals were made:

Proposal 1: RAN4 to consider corrections for prediction reporting delay requirements for both SSB based and CSI-RS based beam prediction captured in CR [R4-2521469]

Proposal 2: Select at least 32 Set A beams, to be emulated during the conformance testing of AI/ML BM use case, from the existing codebook of 128 fixed beams (constructed from a grid of eight elevation angles from –25o to +25 o with ~7.1 o step size and 16 azimuth angles from –60 o to +60 o with 8 o step size) along with the other parameters as defined in Table D.2-1 of TS 38.151.

Proposal 3: RAN4 to study the impact of selecting different combinations of at least 32 Set A beams from the 128-beam codebook on the performance with CDL based channel model.  How to select the at least 32 Set A beams from 128 beam codebook is FFS.

Proposal 4: RAN4 to discuss and explore the feasibility of generating Set A and Set B beams during the conformance testing of the AI/ML-BM use case by selecting unique combinations of available test probes and decide the transmit power of each selected probe. Tx power of each selected probe is FFS.

Proposal 5: RAN4 to discuss and finalize the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

Proposal 6: RAN4 to discuss the feasibility of performing beam sweeping using UE rotation in multiple iterations during the conformance testing of AI/ML based BM use case.

Observation 1: A spatial filter should be applied to the UMi CDL-C channel model based on the SSB and UE Rx beam, so that only the clusters interacting with them are retained. In this case, the number of clusters will be significantly fewer than the 24 clusters defined in the UMi CDL-C model.

Observation 2: Based on the reflection and diffusion of the SSB from gNB to UE and its reception by the Rx beam, the power angular spectrum (PAS) represents the “FR2 channel” that needs to be emulated in the anechoic chamber using probes.

Observation 3: Spatial filter should also be applied to the reduced-cluster channel model according to the beamwidth of SSB and UE Rx beam, so that the interacting clusters can be identified, and the rest of clusters can be ignored.

Observation 4: Reducing the number of clusters in the CDL model carries a high risk of significantly altering the channel properties, potentially shifting the UMi characteristics to those of UMa or to an undefined channel property.

Proposal 7: It is not recommended to reduce the number of clusters in the UMi CDL-C model for simplification. There is high risk of significantly altering the channel properties, potentially shifting the UMi characteristics to those of UMa or to an undefined channel property.

Observation 5: rCDL-C1 model is only used for single-user MIMO (SU-MIMO) cases, and some of the channel characteristics have been removed from the original CDL model in 38.901. Therefore, it is not recommended to use it with large bandwidth test cases.

Proposal 8: The rCDL-C1 channel model in 38.753 with only “12” clusters in total can be considered to generate BM test case scenarios. However, it is only for SU-MIMO cases. It is not suggested to use it for BM testing case with large bandwidth.

Proposal 9: The real FR2 channel measurements and the ray tracing model can be considered to create the cluster-map with less clusters other than CDL-A model with 24 clusters.

Observation 6: Test case creation can be based on the illuminated clusters according to the UE location and gNB location. The clusters far away (a few hundred meters away, based on the scaling of delay in 38.901 table 7.7.3.-1) from the UE will not be illuminated.

Observation 7: Using the same TE chamber and the same probes to present the illuminated clusters. The setting of the weights for the probes need to be changed based on the illuminated clusters in each test case.

Proposal 10: Use the same probes of a TE chamber with different setting for weights of the probes to generate different test cases according to the illuminated clusters.

Proposal 11: A spatial filter should be applied to the CDL-C model first, which can significantly reduce the number of clusters. Next, clusters with insufficient power can be ignored, further reducing the number of clusters. Only the remaining clusters need to be represented in the chamber using probes.

Proposal 12: Endorse the above-mentioned 4-step procedure for representing the clusters in the chamber using probes.

Proposal 13: The reference sensitivity power levels can be reused to determine whether the power of a cluster is too weak and can therefore be ignored.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2514631, “WF on requirements for AIML air interface part 1”, Qualcomm Incorporated, RAN4#116bis, Prague, Czech Republic, 13th ‒ 17th October, 2025.

- R4-2521469, “CR on RRM core requirements to support beam management use case of NR AI/ML Air Interface”, Nokia, RAN4#117, Dallas, USA, November 17th-21st, 2025.

- R4-2514630, “Draft CR on RRM core requirements for support of AI/ML for NR Air Interface for beam management”, vivo, RAN4#116bis, Prague, Czech Republic, 13th ‒ 17th October, 2025.