3GPP TSG-RAN WG4 Meeting # 117	R4-2520926

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.15.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for On-demand SSB SCell operation

Document for:	Discussion

- Introduction

For Rel-19 NES, the performance requirements discussion was discussed in RAN4#116bis with progress captured in [1]. In this contribution, we provide our views on the undecided tests.

- Discussion

In last meeting, a TC list was approved in [1], however three TCs are still FFS.

- SCell activation No.; Test category; Test scenario; Test configuration; Test purpose. FFS; SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB; SCell activation Interruption at OD-SSB activation/deactivation; OD-SSB periodicity 10ms; OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1.

The first one regards to SCell activation TC. The proposed TC aims to verify:

- both scenario 2 and 2a in a single TC

- “OD-SSB case1 for known cell”

In legacy SCell activation TC of known SCell, the SCell is configured and detected during T1, however the test requirement does not verify the measurement period on deactivated SCell. The test requirement only test SCell activation delay. For OD-SSB based TC, there is separate test case to verify the measurement behaviour during scenario 2 (see below TC7). So we don’t observe the need to combine OD-SSB based L3 measurement on deactivated SCell in scenario 2 (before activated) and OD-SSB based SCell activation procedure in scenario 2a (during SCell activation procedure).

| TC7 | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 10ms | OD-SSB Case 1 FMW; UE behaviour relevant to T2 FR1 |
| --- | --- | --- | --- | --- |

If company intends to test SCell activation procedure for OD-SSB case1 in scenario 2a in FR1 rather than in both scenario 2 and 2a, it is a different story. In last meeting, TC4 for case 2 Alt Time-C1 has been agreed to be defined. In our understanding. TC4 is more complicated as it verifies that UE is able to execute SCell activation procedure and perform corresponding behaviour based on EMP. In our understanding, UE who is able to execute EMP based SCell activation procedure can also pass OD-SSB only test, since the OD-SSB based test is easier than EMP based.

| TC4 | SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2a known SCell activation FR1 |
| --- | --- | --- | --- | --- |

Proposal 1: No need to define the below TC:

| Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- |
| SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1 |

Another undecided TC is related to direct SCell activation TC as below.

- Direct SCell Activation

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a FR1 |

RAN4 has agreed to define TC6 for case 2 Alt Time-C1. In our understanding, TC6 is more complicated, as it verifies that UE is able to execute direct SCell activation procedure and perform corresponding behaviour based on EMP. In our understanding, UE who is able to execute EMP based direct SCell activation procedure can also pass OD-SSB only test, since the OD-SSB based test is easier than EMP based.

| TC6 | Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 20ms AO-SSB periodicity 80ms | OD-SSB Case 2 Alt Time-C1 OD-SSB scenario 2a FR1 |
| --- | --- | --- | --- | --- |

Proposal 2: No need to define the below TC:

| Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- |
| Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a FR1 |

Another undecided TC is related to deactivated SCell L3 as below.

- Deactivated SCell L3 measurement

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FFS FR1 |

Although RAN4 has agreed to define TC8 which is for FR2, we understand that some operators only have FR1 deployment. So we are fine to define such TC.

| TC8 | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FR2 |
| --- | --- | --- | --- | --- |

Proposal 3: RAN4 to define below TC:

| Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- |
| Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FR1 |

- Conclusions

Proposal 1: No need to define the below TC:

| Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- |
| SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1 |

Proposal 2: No need to define the below TC:

| Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- |
| Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a FR1 |

Proposal 3: RAN4 to define below TC:

| Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- |
| Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FR1 |

References

[1] R4-2514808, WF on Netw_Energy_NR_enh_RRM_Part1, Ericsson