3GPP TSG-RAN WG4 Meeting # 117	R4-2520928

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.15.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for adaptation RS

Document for:	Discussion

- Introduction

For Rel-19 NES, the performance requirements discussion was discussed in RAN4#116bis with progress captured in [1]. In this contribution, we provide our views on some open issues.

- Discussion

SSB adaption

In RAN4#116bis meeting, it is agreed to specify three TCs for SSB adaptation as below [1]:

  - TC1: L3 intra-frequency measurement

  - TC2: L1-RSRP measurement

  - TC3: L1-SINR measurement

Regarding the first TC for L3 measurement, the test procedure is agreed [1]:

Issue 1-4: How to verify SSB adaption Agreement: - For L3 Intra-frequency serving Cell measurement: - UE is configured with Event A1 or A2. - Condition of serving Cell is changed to satisfy the Event threshold at the instance after DCI 2-9 received + the transition period. - It is verified whether UE can trigger event within the delay based on the mapped additional SMTC.

It shall be noted that the condition of serving cell is changed at the instance after DCI 2-9 received + the transition period. As per the L3 measurement requirement at transition, measurement period is based on the longer period when SSB adaptation is performed. So in test the transition period is alike an uncertainty duration. After the transition duration, UE performs L3 measurement based on new SSB configuration.

9.1.6 Minimum requirement at transitions <non-related content skipped> For UE supporting SSB burst periodicity adaptation for SCell operation, when SSB periodicity changes due to SSB adaptation, the cell identification and measurement period requirements apply based on the longer period before or after the transition.

Regarding L1-RSRP and L1-SINR measurement requirement in SSB adaptation, UE shall follow a different behaviour: L1 measurement period based on the indicated SSB applies from the first SSB after DCI decoding plus OD-SSB post processing time. To verity UE perform L1 measurement based on the new SSB, channel quality is changed at the same time instance of DCI 2-9 indication. UE shall periodically report L1 RSRP result with new value from the first SSB after DCI2-9 decoding plus 2 ms.

9.5.7 Minimum requirement at transitions <non-related content skipped> For UE supporting SSB adaptation, when UE receives DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission during one mesauremnet period, the L1-RSRP measurement period requirements based on the indicated SSB burst periodicity applies from the first SSB burst after the time point as defined in TS 38.213 plus 2 ms and 3 ms for FR1 and FR2 respectively.

To verify the behaviour, the details in test procedure are proposed as follows (shown in Figure 1):

- Treport=20ms (Periodic L1-RSRP reporting); only SSB#0 is configured and observed; timeRestrictionForChannelMeasurements is configured

- DCI 2-9 indicates SSB adaptation: SSB periodicity from 80ms to 20ms

- Condition of serving Cell is changed at the same time as DCI 2-9 indication

- UE shall report L1-RSRP result with new value based on the indicated SSB from the first SSB after DCI decoding plus 2 ms.

Figure 1. TC procedure for L1 measurement with SSB adaption

Proposal 1: For TC of L1-RSRP/SINR measurement of SSB adaptation, the details in test procedure are proposed as follows:

- Treport=20ms (Periodic L1-RSRP reporting); only SSB#0 is configured and observed; timeRestrictionForChannelMeasurements is configured

- DCI 2-9 indicates SSB adaptation: SSB periodicity from 80ms to 20ms.

- Condition of serving Cell is changed at the same time as DCI 2-9 indication

- UE shall report L1-RSRP result with new value based on the indicated SSB from the first SSB after DCI decoding plus 2 ms.

- Conclusions

Proposal 1: For TC of L1-RSRP/SINR measurement of SSB adaptation, the details in test procedure are proposed as follows:

- Treport=20ms (Periodic L1-RSRP reporting); only SSB#0 is configured and observed; timeRestrictionForChannelMeasurements is configured

- DCI 2-9 indicates SSB adaptation: SSB periodicity from 80ms to 20ms

- Condition of serving Cell is changed at the same time as DCI 2-9 indication

- UE shall report L1-RSRP result with new value based on indicated SSB from the first SSB after DCI decoding plus 2 ms.

References

[1] R4-2514809, WF on Netw_Energy_NR_enh_RRM_Part2, Huawei