3GPP TSG-RAN4 Meeting #117	R4-2520990

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | Draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft bigCR to TS38.104 Introduce the BS demodulation requirement for Rel-19 ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-11-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The Big draftCR contains the following draftCR for Rel-19 ATG RRM perf: - R4-2521179 - ZTE Corporation, Sanechips; - Draft CR on 38.104 static propagation condition for Rel-19 ATG demodulation requirements; - Annex G.1. - R4-2522756 - Nokia; - DraftCR on 38.104 for Rel-19 ATG Enhancement; - G.2.3.2.3. - R4-2522757 - Samsung; - Draft CR on FRC for ATG scenario in TS 38.104; - A.5. - R4-2522758 - Samsung; - Draft CR on PUSCH requirement for ATG scenario in TS 38.104; - 8.2. |
| Summary of change: |  | Introduce BS demodulation requirement for NR ATG enhancement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-19 ATG spec will not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.14, A.5, G.1.1(new), G.2.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-1, T8.141-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ==============First change==============

### 8.2.14	Requirements for PUSCH for ATG

#### 8.2.14.1	General

The performance requirement of PUSCH for ATG is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions.

Table: 8.2.14.1-1 Test parameters for testing PUSCH for ATG

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15kHz SCS: N/A 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0},{0,1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL pattern, e.g., 30D4S6U, S=14G for 30kHz SCS. |  |  |

#### 8.2.14.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in tables 8.2.14.2-1 for FDD duplex mode and 8.2.14.2-2 for TDD duplex mode at the given SNR for 1Tx and 2Tx transmission. FRCs are defined in annex A.

Table 8.2.14.2-1: Minimum requirements for PUSCH for ATG with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS, FDD duplex mode

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Frequency offset | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | AWGN | 200 | 70 % | G-FR1- A13-1 | pos1 | 19.1 |
|  |  | Normal | AWGN | 200 | 70 % | G-FR1- A14-1 | pos1 | 19.4 |
| 2 | 2 | Normal | AWGN | 200 | 70 % | G-FR1- A5-17 | pos1 | [14.5] |

Table 8.2.14.2-2: Minimum requirements for PUSCH for ATG with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS, TDD duplex mode

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Frequency offset | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | AWGN | 500 | 70 % | G-FR1- A13-2 | pos1 | 18.7 |
|  |  | Normal | AWGN | 500 | 70 % | G-FR1- A14-2 | pos1 | 19.7 |
| 2 | 2 | Normal | AWGN | 500 | 70 % | G-FR1- A5-18 | pos1 | [14.6] |

# ==============Next change==============

A.5	Fixed Reference Channels for performance requirements (64QAM, R=567/1024)

The parameters for the reference measurement channels are specified in table A.5-2, table A.5-5 and table A.5-6 for FR1 PUSCH performance requirements:

-	FRC parameters are specified in table A.5-2 for FR1 PUSCH with transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer.

-	FRC parameters are specified in table A.5-5 for FR1 interlaced PUSCH with transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer.

-	FRC parameters are specified in table A.5-6 for FR1 PUSCH with transform precoding disabled for ATG scenario, Additional DM-RS position = pos1 and 2 transmission layers.

The parameters for the reference measurement channels are specified in table A.5-3 to table A.5-4 for FR2-1 PUSCH performance requirements:

-	FRC parameters are specified in table A.5-3 for FR2-1 PUSCH with transform precoding disabled, Additional DM-RS position = pos0 and 1 transmission layer.

-	FRC parameters are specified in table A.5-4 for FR2-1 PUSCH with transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer.

The parameters for the reference measurement channels are specified in table A.5-4A for FR2-2 PUSCH performance requirements:

-	FRC parameters are specified in table A.5-4A for FR2-2 PUSCH with transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer.

Table A.5-1: Void

Table A.5-2: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (64QAM, R=567/1024)

| Reference channel | G-FR1-A5-8 | G-FR1-A5-9 | G-FR1-A5-10 | G-FR1-A5-11 | G-FR1-A5-12 | G-FR1-A5-13 | G-FR1-A5-14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Code rate (Note 2) | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 |
| Payload size (bits) | 12040 | 25104 | 50184 | 11528 | 24576 | 50184 | 131176 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 2 | 3 | 6 | 2 | 3 | 6 | 16 |
| Code block size including CRC (bits) (Note 2) | 6056 | 8400 | 8392 | 5800 | 8224 | 8392 | 8224 |
| Total number of bits per slot | 21600 | 44928 | 91584 | 20736 | 44064 | 91584 | 235872 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39312 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0= 2 and l =11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |  |

Table A.5-3: FRC parameters for FR2-1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 1 transmission layer (64QAM, R=567/1024)

| Reference channel | G-FR2-A5-1 | G-FR2-A5-2 | G-FR2-A5-3 | G-FR2-A5-4 | G-FR2-A5-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Code rate (Note 2) | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 |
| Payload size (bits) | 23568 | 47112 | 11528 | 23568 | 47112 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 7888 | 7880 | 5800 | 7888 | 7880 |
| Total number of bits per slot without PT-RS | 42768 | 85536 | 20736 | 42768 | 85536 |
| Total number of bits per slot with PT-RS (Note 3) | 40986 | 81972 | 19872 | 40986 | 81972 |
| Total symbols per slot without PT-RS | 7128 | 14256 | 3456 | 7128 | 14256 |
| Total symbols per slot with PT-RS (Note 3) | 6831 | 13662 | 3312 | 6831 | 13662 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |  |  |  |

Table A.5-4: FRC parameters for FR2-1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (64QAM, R=567/1024)

| Reference channel | G-FR2-A5-6 | G-FR2-A5-7 | G-FR2-A5-8 | G-FR2-A5-9 | G-FR2-A5-10 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Code rate (Note 2) | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 |
| Payload size (bits) | 21000 | 42016 | 10248 | 21000 | 42016 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 5 | 2 | 3 | 5 |
| Code block size including CRC (bits) (Note 2) | 7032 | 8432 | 5160 | 7032 | 8432 |
| Total number of bits per slot without PT-RS | 38016 | 76032 | 18432 | 38016 | 76032 |
| Total number of bits per slot with PT-RS (Note 3) | 36432 | 72864 | 17664 | 36432 | 72864 |
| Total symbols per slot without PT-RS | 6336 | 12672 | 3072 | 6336 | 12672 |
| Total symbols per slot with PT-RS (Note 3) | 6072 | 12144 | 2944 | 6072 | 12144 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1 with l0= 0 and l =8 as per Table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |  |  |  |

Table A.5-4A: FRC parameters for FR2-2 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (64QAM, R=567/1024)

| Reference channel | G-FR2-A5-11 | G-FR2-A5-12 | G-FR2-A5-13 |
| --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 | 480 |
| Allocated resource blocks | 66 | 264 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 |
| Modulation | 64QAM | 64QAM | 64QAM |
| Code rate (Note 2) | 567/1024 | 567/1024 | 567/1024 |
| Payload size (bits) | 21000 | 83976 | 21000 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 10 | 3 |
| Code block size including CRC (bits) (Note 2) | 7032 | 8424 | 7032 |
| Total number of bits per slot without PT-RS | 38016 | 152064 | 38016 |
| Total number of bits per slot with PT-RS (Note 3) | 36432 | 145728 | 36432 |
| Total symbols per slot without PT-RS | 6336 | 25344 | 6336 |
| Total symbols per slot with PT-RS (Note 3) | 6072 | 24288 | 6072 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1 with l0= 0 and l =8 as per Table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |  |

Table A.5-5: FRC parameters for FR1 interlaced PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (64QAM, R=567/1024)

| Reference channel | G-FR1-A5-15 | G-FR1-A5-16 |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 30 |
| Allocated resource blocks | 11 | 11 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 |
| Modulation | 64QAM | 64QAM |
| Code rate | 567/1024 | 567/1024 |
| Payload size (bits) | 5248 | 5248 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | - | - |
| Number of code blocks - C | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 5272 | 5272 |
| Total number of bits per slot (Note 3) | 9504 | 9504 |
| Total symbols per slot (Note 3) | 1584 | 1584 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0= 2 and l =11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: The calculation of the “Total number of bits per slot” and “Total symbols per slot” fields include the REs taken up by CG-UCI, if present. |  |  |

Table A.5-6: FRC parameters for FR1 PUSCH performance reuqirments, transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers (64QAM, R=567/1024)

| Reference channel | G-FR1-A5-17 | G-FR1-A5-18 |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 30 |
| Allocated resource blocks | 25 | 24 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 |
| Modulation | 64QAM | 64QAM |
| Code rate | 567/1024 | 567/1024 |
| Payload size (bits) | 24072 | 23040 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 |
| Number of code blocks - C | 3 | 3 |
| Code block size including CRC (bits) (Note 2) | 8056 | 7712 |
| Total number of bits per slot | 43200 | 41472 |
| Total symbols per slot | 7200 | 6912 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0= 2 and l =11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. |  |  |

# ==============Next change==============

G.1	Static propagation condition

The propagation for the static performance measurement is an Additive White Gaussian Noise (AWGN) environment. No fading or multi-paths exist for this propagation model.

G.1.1	BS Receiver with 2Rx

For 2 port transmission the channel matrix is defined in the frequency domain by

# ==============Next change==============

G.2	Multi-path fading propagation conditions

<Unchanged part removed >

G.2.3	MIMO Channel Correlation Matrices

<Unchanged part removed>

G.2.3.2	Multi-Antenna channel models using cross polarized antennas

<Unchanged part removed>

G.2.3.2.3	MIMO Correlation Matrices using cross polarized antennas

The values for parameters α, β and γ for low spatial correlation are given in Table G.2.3.2.3-1.

Table G.2.3.2.3-1:	Values for parameters α, β and γ

| Low spatial correlation |  |  |
| --- | --- | --- |
| α | β | γ |
| 0 | 0 | 0 |
| Note 1: Value of α applies when more than one pair of cross-polarized antenna elements at gNB side. Note 2: Value of β applies when more than one pair of cross-polarized antenna elements at UE side. |  |  |

The correlation matrices for low spatial correlation are defined in Table G.2.3.2.3-2 as below.

Table G.2.3.2.3-2:	MIMO correlation matrices for low spatial correlation

| 1x8 case |  |
| --- | --- |
| 2x2 case |  |
| 2x8 case |  |

In Table G.2.3.2.3-2,  is a  identity matrix.

# ==============End of change==============