# 3GPP TSG RAN WG4 Meeting#117	R4-2521005

# Dalls, USA, Nov 17 – 21, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements of LP-WUS/WUR for NR

Agenda item:	6.16.3

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, the scope of the performance requirements for LP-WUS/WUR including various simulation assumptions were discussed and finalized [1]. In this paper, we present our perspectives on the remaining open issues.

2. Testability Aspects

2.1 Whether to define separate test cases for idle/inactive and connected modes

Whether to define separate test cases for idle/inactive and connected modes

- Agreement: Define one set of requirements and add test applicability rule that the test is conducted in idle/inactive and/or connected modes based on UE supported capabilities. If UE support both RRC idle/inactive and connected modes, the test will be conducted:

  - Option 1: in connected mode

  - Option 2: in idle/inactive mode

  - Option 3: in both RRC idle/inactive and connected modes

- FFS whether to capture the test applicability w.r.t. RRC state in 38.101-4 or leave it to RAN5.

The issue of whether to define separate test cases for IDLE/INACTIVE and CONNECTED modes The WF is outlined below.

Our view is that when the UE supports both idle/inactive and connected modes, testing in connected mode is preferable due to potentially shorter test durations.

Observation 1: Testing in connected mode is generally preferred when the UE supports both idle/inactive and connected modes, as it may result in shorter test durations.

Proposal 1: If the UE supports both RRC states, conduct the test in connected mode (Support option 1).

3. Demodulation Requirements

3.1 Consideration of one-shot and repetition transmission mode

Consideration of one-shot and repetition transmission mode for OFDM-based receiver

- Proposals

  - Option 1

    - RAN4 to specify demodulation requirements for both one-shot and full-repetition cases for OFDM-based receivers

The WF is outlined below.

We note that one-shot mode lacks repetition gain, making it more sensitive to channel conditions and requiring higher SNR to meet target MDR level. On the other hand, repetition mode benefits from processing gain due to multiple transmissions, enabling improved reliability at lower SNRs. Due to the significant variation in MDR performance observed in the simulation results [2], adopting a single unified requirement may not be feasible.

Observation 2: One-shot mode lacks repetition gain, making it more sensitive to channel conditions and requiring higher SNR to meet target MDR level, while repetition mode benefits from processing gain due to multiple transmissions.

Observation 3: A substantial difference in MDR performance has been observed between one-shot and repetition processing modes.

Proposal 2: Further explore a methodology for defining MDR requirements for the OFDM receiver operating under both one-shot and repetition processing modes.

4. Conclusions

In this paper, we share our perspectives on the remaining open issues. The following proposals have been made.

Observation 1: Testing in connected mode is generally preferred when the UE supports both idle/inactive and connected modes, as it may result in shorter test durations.

Proposal 1: If the UE supports both RRC states, conduct the test in connected mode (Support option 1).

Observation 2: One-shot mode lacks repetition gain, making it more sensitive to channel conditions and requiring higher SNR to meet target MDR level, while repetition mode benefits from processing gain due to multiple transmissions.

Observation 3: A substantial difference in MDR performance has been observed between one-shot and repetition processing modes.

Proposal 2: Further discuss an approach for defining demodulation performance requirements for OFDM receivers operating in both one-shot and repetition processing modes.

5. References

[1] R4-2514813, “WF for [116bis][330] NR_LPWUS_Demod,” 3GPP TSG RAN WG4 Meeting #116bis, Prague, Czech Republic, Oct 13 – 17, 2025.

[2] R4-2521006, “Simulation results for demodulation performance requirements of LP-WUS/WUR for NR,” 3GPP TSG RAN WG4 Meeting #117, Dallas, USA, Nov 17 – 21, 2025.