# 3GPP TSG-RAN WG4 Meeting # 117                                  R4-2521192

Dallas, USA, Nov. 17-21, 2025

# Title: 	Discussion on demodulation performance requirements for FR1 SU-MIMO with spatial channel model

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.3

# Document for:	Approval

# 1 Introduction

In RAN #109 meeting, the new WID of UE performance requirements for FR1 SU-MIMO with spatial channel model, which was approved in [1], wherein the objectives related to FR1 SU-MIMO with spatial channel model are listed as below:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

In this contribution, we give some initial discussions on the UE performance requirements for SU-MIMO with spatial channel model based on the last meeting WF [3].

# 2 Discussion

UE antenna modelling

- <For INFORMATION> It is RAN4 common understanding that the following were added in Rel-19 version of TR 38.901: - #1: Candiadate location for UE antenna placements - #2: Directional antenna assumption for UE - Further discuss the two options in the next meeting: - Option 1: Update the UE antenna placement and assumption of rCDL based on v19.0.0 of 38.901. - Option 2: Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline - The moderator requests proponents of option 1 to provide detailed simulation assumptions in the next meeting.

As discussed in the last meeting, in the last version 38.901, UT antenna model is introduced for different UE types, e.g., handheld UT and CPE devices. The import thing is that antenna element/array at the placement candidate locations can be modelled relative to the centre of the UT. Different sizes and candidate locations are assumed for handheld UT and CPE devices. In the Rel-19 work, only one UE AAV (1,1) configuration (single element subarray) has been considered in the study. Compared with the assumption of 38.901, RAN4 used a different assumption in Rel9 evaluation. Antenna placement candidate locations make different distances when calculating the location vector for receive antenna element. Therefore, we believe that updating the UE antenna placement and assumptions for the rCDL channel model would be more realistic.

Currently, total 8 and 9 candidate antenna locations are recommended for UT and CPE devices. A candidate antenna location can use for multiple antenna. RAN4 needs to further discuss how to choose the candidate locations for different number of receive antennas, which is the common assumptions for antenna placement.

Observation 1. The candidate locations for the antenna placement result in different distances when the location vector is calculated for the receive antenna element.

Proposal 1. Propose to update the UE antenna placement and assumptions for the rCDL channel model would be more realistic.

Proposal 2. RAN4 needs to further discuss how to choose the candidate locations for different number of receive antennas.

Further rCDL-C1 channel alignment

- Candidate options: - Option 1: Measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4. - Option 2: Not to do further rCDL-C1 channel alignment

In the last meeting, RAN4 agreed to update the rCDL channel model based on the latest angular scaling as in TR 38.901, using ‘s’ values for tabulated desired AS values, such as AOD-0.6513, AOA-0.8585, ZOA-1.9504, ZOD-1.8219 to generate new rCDL channel model, which will differ from the parameters of the previous rCDL-C1 channel model. In order to better align the channel model, we believe that the SDPD, TC, FC are helpful and reflect the characteristics of the channel model.

Proposal 3. Propose to measure and align channel statistics such as SDPD, TC, and FC as described in TR 38.753 Section 6.4.

Precoding, MCS and test metric

- Agreement and candidate options: - For 4T4R 4L test, it is agreed to use the test metric: SNR @ 70% max TP. - For 8T8R 8L test, it is agreed to use test metric: Define single SNR and further discuss the SNR is based on: - Option a: Based on per CW thput with Different MCS per CW - Option b: Based on total thput with different MCS per CW - Option c: Based on total thput with same MCS per CW - Further discuss: How to define the SNR for option a. How to select the MCS for different CW.

Based on the evaluation and analysis of Rel-19, each CW has different performance on 8 MIMO layers test cases. According this phenomenon, RAN4 assumed the same MCS for different CWs in Rel-19. In Rel-20, our motivation is to define limited test cases to do conformance testing and verify the performance of CDL channel model. Compared with the above options, option c is the simplest and most legacy testing approach. Option a and Option b emphasize choosing different MCS for each CW. This is undoubtedly feasible. However, choosing an MCS is a complex process, which is based on the CQI reporting and outer loop link adaption. The performance of different MCS can be verified in CQI requirements. PDSCH requirements are used to evaluate whether UE can achieve maximum throughput testing. Therefore, we believe that there is no necessary to configure different MCSs for different CW.

Proposal 4. Propose to consider the test metric based on total throughput with same MCS per CW.

PMI bias

- Candidate options: - Option 1: RAN4 shall exploit approaches to address the PMI bias issue - Option 1a: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 - Option 1b: time variant mean angles (AOA/AOD/ZOA/ZOD) - Other options are not precluded - Option 2: No need to consider time-wise evolution for PMI reporting tests - Option 3: Defer a consideration of a comprehensive long-term solutions to the 6G Radio Study Item

In the Rel-19 study, the remain issue is that the CDL channel model fixes the angles information, and the channel properties remain essentially constant in long-term simulation. This makes it difficult to verify the UE’s performance in PMI testing. The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Although the legacy TDL channel model can not generate time-varying angle information, RAN4 incorporates changeable angle information through beam steering, resulting in a time-varying beam angle in the PMI test, thus making PMI selection effective. The legacy approach such as beam steering has been assumed in the TDL channel model. At least, beam steering may be a potential approach to exploiting solutions to the PMI bias issue. Also, time variant mean angles are mentioned in the last meeting. By adding time-varying information to change the angle information during the channel generation. In general, all the above approaches can be considered as potential approaches for Rel-20 to address the PMI bias issue.

Proposal 5. Propose to consider the beam steering approach or time variant mean angles to address the PMI bias issue.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for FR1 SU-MIMO with spatial channel mode, the conclusions are:

Observation 1. The candidate locations for the antenna placement result in different distances when the location vector is calculated for the receive antenna element.

Proposal 1. Propose to update the UE antenna placement and assumptions for the rCDL channel model would be more realistic.

Proposal 2. RAN4 needs to further discuss how to choose the candidate locations for different number of receive antennas.

Proposal 3. Propose to measure and align channel statistics such as SDPD, TC, and FC as described in TR 38.753 Section 6.4.

Proposal 4. Propose to consider the test metric based on total throughput with same MCS per CW.

Proposal 5. Propose to consider the beam steering approach or time variant mean angles to address the PMI bias issue.

# 4 References

- RP- 252957, WID: NR demodulation performance: Phase 6.

- 3GPP TR 38.753 V1.1.0 (2025-09).

- R4-2514823, WF on NR_demod_Ph6_Ph5.