# 3GPP TSG-RAN WG4 Meeting #117                                    R4-2521195

# Dallas, USA, Nov. 17-21, 2025

# Title: 	Views on 6G demodulation specifications imporovement

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	8.12.2.3

# Document for:	Approval

# 1 Introduction

In RAN4 #116 meeting, a new SI [1] on RAN4 6G topic study was approved, which provided a more detailed guidance from RAN4 perspective, as shown in the following.

In this contribution, we provide a detailed discussion on RAN4 demodulation specification improvement for 6GR study based on the last meeting discussion [2].

# 2 Discussion

In the last meeting, RAN4 discussed the detailed issues for the 6G specifications improvement, including general principles, modernization and new tools, and drafting rules, etc. [2]. Next, we would like to address in the following for 6G demodulation specification improvements.

Perf/Demod specs improvements

- Proposals - Proposal 1: Replacing broad applicability statements with clear, centralized mappings of test coverage. This would enhance consistency and reduce ambiguity across device types and configurations. - Proposal 2: A capability-aware test applicability framework should be intended, e.g., For devices lacking legacy TN support, test applicability should be designed to avoid dependency on TN-related procedures and corresponding test cases." - Proposal 3: FRC table improvements considering - necessary configuration and avoid derived information - methodology to generate FRC table efficiently

In the last meeting, RAN4 discussed the potential specification improvement aspects for demodulation from applicable rules, an FRC tables. Regarding FRC tables, which not only including performance core parts specs, but also including conformance performance parts specs. Currently, RAN4 defines FRC tables for every test case, in order to align the maximum throughput between the requirements definition and TE vendors. Because the different maximum throughputs have different SNR operation point for most performance test cases. Different configuration, such as MCS, MIMO layers, reference signal configurations also could have an impact on conformance testing results. That also is a guidance to align the company’s results, because the requirements definition based on the company’s collaboration.

Firstly, for the 38.101-4 spec, RAN4 introduced many different test cases for SU-PDSCH, CA, CSI requirements to verify the performance of different functionalities. When reviewing those above test cases, different MCSs, MIMO layers, bandwidths, subcarriers, and reference signal configurations determine the throughput. The encoding parameters follow the same calculation rules and depend on the RAN1 design.

Based on the above analysis, we can see that the FRC tables are configured by parameters. Therefore, RAN4 should study an effective approach to including more related test cases, such as SU-PDSCH, CA, CSI requirements, SDR and ATP scenarios. These test cases could be based on fundamental formula to calculate the maximum throughput.

Secondly, the BS side specifications include the performance core parts and the conformance testing parts, not only for 38.104, but also for repeaters such as NCR, and IAB, etc., which also have demodulation requirements. The same issues occur regarding the FRC tables in 38.104 as in 38.101-4, more FRC tables and the same calculation procedure. The calculation procedure is easier than on the downlink side. There is no need to derive the throughput; it also lists the TB size and the total number of transmitted symbol bits, which depends on the modulation order. It also includes some encoding parameters. Therefore, we believe that studying the efficiency of the FRC tables could be very useful and reduce the complexity of the specifications.

Another issue is the numbering of the FRC between core and conformance testing. During the 5G discussion, it was also required that the numbering of FRC tables be aligned between the core and conformance testing specifications for new BS types such as NCR and IAB, which have a single specification for introducing related functionality in 3GPP. Sometimes, demodulation requirements, being the final specification, define numerous baseband test cases, leading to many FRC tables to indicate test throughput. Conformance testing also includes many of these same tables. Because the core parts and test specifications differ, the FRC numbering in the test specs can be inconsistent, making it difficult to use the same numbering. This introduces a significant non-technical workload. Based in the above reasons, we propose to discuss whether it’s necessary to align the FRC numbers between the performance and conformance testing parts specifications, and how to adopt a unified, standardized naming convention in the 6G.

Observation 1. The FRC table is mainly about throughput calculations, different MCSs, MIMO layers, bandwidths, subcarriers, and reference signal configurations determine the throughput.

Observation 2. The calculation procedure of uplink side is easier than on the downlink side. There is no need to derive the throughput; it also lists the TB size and the total number of transmitted symbol bits, which depends on the modulation order. It also includes some encoding parameters.

Proposal 1. Propose to explore ways to improve the efficiency of FRC table calculation, such as introducing scalable calculation methods. Such improvements are expected to significantly reduce the number of FRC table definitions while lowering the overall complexity of the specification.

Proposal 2. Propose to discuss whether it’s necessary to align the FRC numbers between the performance and conformance testing parts specifications, and how to adopt a unified, standardized naming convention in the 6G.

# 3 Conclusion

In this contribution, we give some discussions on 6G demodulation specifications imporovement, the conclusions are:

Observation 1. The FRC table is mainly about throughput calculations, different MCSs, MIMO layers, bandwidths, subcarriers, and reference signal configurations determine the throughput.

Observation 2. The calculation procedure of uplink side is easier than on the downlink side. There is no need to derive the throughput; it also lists the TB size and the total number of transmitted symbol bits, which depends on the modulation order. It also includes some encoding parameters.

Proposal 1. Propose to explore ways to improve the efficiency of FRC table calculation, such as introducing scalable calculation methods. Such improvements are expected to significantly reduce the number of FRC table definitions while lowering the overall complexity of the specification.

Proposal 2. Propose to discuss whether it’s necessary to align the FRC numbers between the performance and conformance testing parts specifications, and how to adopt a unified, standardized naming convention in the 6G.

# 4 Reference

- R4-2511652, RAN4 6G Plan and Meeting Arrangement.

- R4-2514648, Way Forward for [116bis][111] 6G operation efficiency, CATT.