3GPP TSG-RAN WG4 Meeting #117	R4-2521233

Dallas, USA, 17 November – 21 November 2025

Agenda Item:	6.11.8

Source:	Ericsson

Title:	PMI and CSI-PAI reporting requirements with CSI prediction

Document for:	Discussion

# Introduction

RAN4#116bis agreed with the way forward on AI/ML for NR air interface [1].

RAN4#116bis [1] Issue 1-4: Core requirements for PMI prediction reporting Agreement: Introduce a new clause under Clause 6 (6.X) in 38.101-4 as “Reporting of predicted PMI” Issue 1-7: Doppler and MCS Agreement: Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD Introduce test for TDD with the details FFS Issue 1-8: Generalization Agreement: Test setup for the generalization will be further discussed based on the following options - Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points. - Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS. - Option 3: the combination of option 1 and 2. It is FFS on how to quantify the generalization performance. RAN4#116 Issue 1-5: Scheduling delay Agreement: The delay from when the UE prediction is send until the time the TE applies is n+4 (slots) for FDD RAN4#115 Issue 1-1: Requirement considerations Agreement: The performance of AI based CSI prediction should not be worse than R16 etypeII. - The exact test procedure and configurations are FFS - The impact of different timeline between AI and R16 etype II should be also carefully analysed Issue 1-3: Doppler handling Agreement: - De-prioritize 100Hz Doppler in R19 - Consider 100Hz Doppler in the future release if needed Issue 1-5: SNR levels for training data and generalization Agreement: No need to capture anything explicit in RAN4 about training data in the spec, - For simulation calibration, SNR range alignment for training can be discussed separately UE performance at more than one MCS should be tested - Model generalization can be discussed separately - Each individual test is expected to involve in single MCS

RAN4#116bis has also updated the simulation assumption of AI/ML-based PMI reporting requirements [2]. This contribution focuses on Step 3 and test setup of PMI reporting requirements with CSI prediction.

Step 3: Check PDSCH throughput with the predicted PMI.

- γ1_AI/ML: AI/ML based CSI prediction is compared with Rel-15 Type I random PMI

- γ2_AI/ML: AI/ML based CSI prediction is compared with Rel-16 eType II CB (sample and hold)

- γ3_S&H: Rel-16 eType II CB (sample and hold) is compared with Rel-15 Type I random PMI

# PMI reporting performance

## Simulation results

### TDLA30-20

Table 1	Summary of simulation results for 16 CSI-RS ports, FDD SCS=15kHz, TDLA30-20.

| CSI-RS ports | Channel model | UE RX | MCS | SNR to achieve 90% of maximum throughput with predicted PMI (SNR_AI/ML) | γ1_AI/ML | γ2_AI/ML | SNR to achieve 90% of maximum throughput with eTypeII (SNR_S&H) | γ3_S&H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | TDLA30-20 | 2 | 17 | 4.7 | 9.82 | 1.28 | 8.9 | 3.87 |
|  |  |  | 19 | 6.3 | 8.88 | 1.27 | 10.6 | 3.68 |
|  |  | 4 | 17 | 1.8 | 7.30 | 1.05 | 2.4 | 6.30 |
|  |  |  | 19 | 3.4 | 6.74 | 1.03 | 3.9 | 6.20 |

Table 2	Summary of simulation results for 16 CSI-RS ports, TDD SCS=30kHz, TDLA30-20.

| CSI-RS ports | Channel model | UE RX | MCS | SNR to achieve 90% of maximum throughput with predicted PMI (SNR_AI/ML) | γ1_AI/ML | γ2_AI/ML | SNR to achieve 90% of maximum throughput with eTypeII (SNR_S&H) | γ3_S&H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | TDLA30-20 | 2 | 17 | 5.3 | 8.02 | 1.11 | 7.2 | 5.32 |
|  |  |  | 19 | 7.1 | 7.09 | 1.10 | 8.8 | 4.98 |
|  |  | 4 | 17 | 2.2 | 6.81 | 0.98 | 2.0 | 7.26 |
|  |  |  | 19 | 4.0 | 6.06 | 0.96 | 3.5 | 6.74 |

Table 1 and Table 2 summarize the simulation results for FDD SCS=15kHz and TDD SCS=30kHz, respectively. The results show that the range of SNR to achieve 90% of the maximum throughput (SNR_AI/ML) is from 1.8 to 7.1dB depending on the selected MCS index. The throughput ratios of the following predicted PMI and random Rel-15 TypeI PMI (γ1_AI/ML) are high enough (6.06 to 9.82). On the other hand, it is observed the larger gain of the predicted PMI to Rel-16 eTypeII PMI (γ2_AI/ML > 1.1) only for 2Rx cases. For 4Rx cases, the gain of the predicted PMI to Rel-16 eTypeII PMI is around 1.0. Especially for TDD 4Rx cases, Rel-16 eTypeII PMI is better than predicted PMI with the SNR corresponding to the 90% of the peak rate.

Observation 1: For TDLA30-20, throughput ratio of AI/ML-based PMI prediction over Rel-15 TypeI PMI (γ1_AI/ML) is high enough regardless of 2Rx/4Rx FDD/TDD.

Observation 2: For TDLA30-20, throughput ratio of AI/ML-based PMI prediction over Rel-16 eTypeII PMI (γ2_AI/ML) is more than 1.0 for 2Rx FDD/TDD and 4Rx FDD.

Observation 3: For TDLA30-20, both MCS17 and MCS19 show similar throughput ratio of AI/ML-based PMI prediction over Rel-15 TypeI PMI.

### TDLC300-20

We have also studied TDLC300-20 to compare follow PMI with AI/ML-based predicted PMI, follow PMI with Rel-16 eTypeII, and random PMI with Rel-15 TypeI.

Table 3	Summary of simulation results for 16 CSI-RS ports, FDD SCS=15kHz, TDLC300-20.

| CSI-RS ports | Channel model | UE RX | MCS | SNR to achieve 90% of maximum throughput with predicted PMI (SNR_AI/ML) | γ1_AI/ML | γ2_AI/ML | SNR to achieve 90% of maximum throughput with eTypeII (SNR_S&H) | γ3_S&H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | TDLC300-20 | 2 | 17 | 5.9 | 7.63 | 1.24 | 7.9 | 4.72 |
|  |  |  | 19 | 7.7 | 6.70 | 1.22 | 9.8 | 4.26 |
|  |  | 4 | 17 | 3.1 | 5.79 | 0.95 | 2.6 | 6.57 |
|  |  |  | 19 | 4.8 | 5.32 | 0.94 | 4.1 | 6.31 |

Table 4	Summary of simulation results for 16 CSI-RS ports, TDD SCS=30kHz, TDLC300-20.

| CSI-RS ports | Channel model | UE RX | MCS | SNR to achieve 90% of maximum throughput with predicted PMI (SNR_AI/ML) | γ1_AI/ML | γ2_AI/ML | SNR to achieve 90% of maximum throughput with eTypeII (SNR_S&H) | γ3_S&H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | TDLC300-20 | 2 | 17 | 7.9 | 4.97 | 0.97 | 7.7 | 5.34 |
|  |  |  | 19 | 10.0 | 4.31 | 0.96 | 9.6 | 4.65 |
|  |  | 4 | 17 | 4.5 | 4.72 | 0.90 | 2.8 | 7.24 |
|  |  |  | 19 | 6.3 | 4.16 | 0.90 | 4.5 | 6.39 |

Table 3 and Table 4 summarize the simulation results for FDD SCS=15kHz and TDD SCS=30kHz, respectively. The results show that the range of SNR to achieve 90% of the maximum throughput (SNR_AI/ML) is from 3.1 to 10.0dB depending on the selected MCS index, whose SNR test point slightly higher than TDLA30-20. Although the throughput ratios of the following predicted PMI and random Rel-15 TypeI PMI (γ1_AI/ML) are high enough (4.16 to 7.63), the gain is smaller than TDLA30-20. It is also observed the gain of the predicted PMI to Rel-16 eTypeII PMI (γ2_AI/ML) is more than 1.0 only for FDD 2Rx cases.

Observation 4: For TDLC300-20, throughput ratio of AI/ML-based PMI prediction over Rel-15 TypeI PMI (γ1_AI/ML) is high enough regardless of 2Rx/4Rx FDD/TDD.

Observation 5: For TDLC300-20, throughput ratio of AI/ML-based PMI prediction over Rel-16 eTypeII PMI (γ2_AI/ML) is more than 1.0 only for 2Rx FDD.

Observation 6: For TDLC300-20, both MCS17 and MCS19 show similar throughput ratio of AI/ML-based PMI prediction over Rel-15 TypeI PMI.

## Test cases

RAN4#116bis agreed to consider the following two scenarios:

- 16 CSI-RS ports, TDLA30-20, MCS17 or MCS19

  - FDD SCS=15kHz, 2Rx

  - TDD SCS=30kHz, 4Rx

RAN4 UE performance requirements in FR1 usually defines one test with the combination of FDD/TDD and 2Rx/4Rx, considering the UE capability. Since RAN4 has agreed to define more than one scenario to verify the model generalization, RAN4 may limit the test with FDD 2Rx and TDD 4Rx, to avoid increasing the total number of test cases.

Other than TDLA30-20, we think TDLA300-20 is a good candidate as shown in our simulation results to verify the model generalization. We prefer to use MCS17 for TDLA30-20 and MCS19 for TDLC300-10, considering the lower and higher SNR test points, respectively.

Proposal 1: RAN4 should define AI/ML-based PMI prediction performance requirements for FDD 2Rx and TDD 4Rx with the following configuration: 16 CSI-RS ports, TDLA30-20, MCS17.

Proposal 2: RAN4 should define AI/ML-based PMI prediction performance requirements for FDD 2Rx and TDD 4Rx with the following configuration: 16 CSI-RS ports, TDLC300-20, MCS19.

## Scheduling for PMI reporting requirements

Figure 1 and Figure 2 are the possible scheduling of CSI-RS, PMI reporting, and PDSCH used for Rel-19 AI/ML-based CSI prediction, for FDD and TDD scenarios, respectively. In these scheduling, we have the following assumptions:

- N4=1 (Doppler/time-domain basis vector length)

- Delta = 1 (Prediction delay)

- Periodic CSI-RS resources scheduled every 5 slots for FDD and 10 slots for TDD.

- CSI reporting delay is same as ‘typeII-Doppler-r18’.

Figure 1	Test setup for FR1 FDD case of AI/ML-based CSI prediction.

Figure 2	Test setup for FR1 TDD case of AI/ML-based CSI prediction.

In our understanding, RAN4 will define the PMI reporting requirements for UE capable of FG 58-3-1 “CSI prediction for UE-sided inference when N4=1” [3]. RAN1 has introduced additional CSI processing delay t to Z2/Z2’ in TS 38.214 5.4, and FG 58-3-1 includes Component 12 “supported value of t for the relaxation of Z and Z’ timeline”. However, RAN1 still sets Component 12 is FFS [3]. Because Figure 1 and Figure 2 are derived from the assumption t=0, if RAN1 agreed to introduce t < 0 and/or t > 0, the PMI reporting timing need to be revisited.

Observation 7: RAN1 is still discussing the CSI reporting delay for Rel-19 AI/ML-based PMI prediction.

Proposal 3: RAN4 should evaluate the AI/ML-based PMI prediction performance based on the CSI processing time with t=0. Once RAN1 concludes the UE capability of reporting delay t, RAN4 should revisit the scheduling of CSI-RS, PMI reporting, and PDSCH.

# CSI-PAI reporting requirements

RAN4#116 has already agreed to define the reporting ‘accuracy’ requirements for CSI prediction, although RAN4 does not define the performance monitoring reporting delay requirements for CSI prediction.

RAN4#116bis [1] Issue 1-1: Requirements for performance monitoring Agreement: No performance monitoring delay requirements are specified in RAN4 for CSI prediction. RAN4#116 Issue 1-1: Performance monitoring Agreement: - RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction - FSS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: - Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

## Overview of CSI-PAI reporting

RAN1 has specified that UE reports the following two SGCS values in CSI Prediction Accuracy Indicator (CSI-PAI) for the UE-assisted performance monitoring metric as illustrated in Figure 3.

- First SGCS (SGCSk1): calculated from the predicted PMI (wk) for one inference reporting and the non-predicted PMI (wk) based on the second Reporting Setting. UE shall report the first SGCS (SGCS1) for each MIMO layer k.

- Second SGCS (SGCSk2): calculated from the non-predicted PMI (wk) based on the second Reporting Setting and the PMI based on the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance (wk). UE shall report the second SGCS (SGCS2) for each MIMO layer k.

Figure 3	Illustration of first SGCS and second SGCS reported in CSI-PAI.

RAN1 has agreed that the performance monitoring for CSI prediction (FG 58-3-5) is optional feature even if UE supports CSI prediction for UE-sided inference when N4=1 (FG 58-3-1) [3]. In other words, if the UE supports the CSI-PAI reporting, this UE should support AI/ML-based CSI prediction.

Observation 8: The performance monitoring for CSI prediction model (FG 58-3-5) is optional feature even if UE supports the CSI prediction for UE-sided inference when N4=1 (FG 58-3-1).

Observation 9: If UE supports the performance monitoring for CSI prediction model, this UE supports the CSI prediction for UE-sided inference when N4=1.

It is important to define the CSI-PAI reporting performance requirements because gNB will use this information not only to monitor the prediction performance but also to deactivate the prediction function.

Proposal 4: For CSI-PAI reporting requirements, RAN4 should consider at least two cases: one case for the AI/ML-based CSI prediction works, and another case for the prediction does NOT work well.

On the other hand, it is not easy to judge whether the CSI prediction works or not with the reported SGCS values because we did not observe the clear relation between the reported SGCS values and PDSCH throughput with follow predicted PMI [4]. For example, higher SGCS1 does not always mean higher throughput with predicted PMI compared with Rel-16 eTypeII. RAN4 may need to study further the metric(s) to judge the AI/ML-based CSI prediction works.

Proposal 5: RAN4 should study further the metric of reported CSI-PAI to judge the AI/ML-based CSI prediction works or does not work well.

## Simulation results

### Statistics of SGCS1 and SGCS2

One test metric for CSI-PAI reporting is to check a statistic of reported SGCS1, where a requirement is that SGCS1 should be more than X with 90% of the test time, e.g., X=0.85, like the CQI fading test specified in TS38.101-4.

Figure 4 and Figure 5 show CDFs of quantized SGCS1 and SGCS2, respectively, for 16 CSI-RS ports with TDLA30-20 with different SNR test points, according the simulation assumption [2]. The simulation results show that the reported SGCSs become low as the SNR test point is decreased. It is also observed that the 90%-ile of SGCS1 is higher than that of SGCS2, which may indicate the AI/ML-based prediction works compared with sample-and-hold.

Figure 6 and Figure 7 also show CDFs of quantized SGCS1 and SGCS2, respectively, for 16 CSI-RS ports with TDLC300-20 with different SNR test points. We see the same observation as TDLA30-20.

| (a) Quantized SGCS1 | (b) Quantized SGCS2 |
| --- | --- |

Figure 4	CDF of CSI-PAI SGCS1 and SGCS2 for 16 CSI-RS ports, FDD SCS=15kHz, 2Rx, TDLA30-20.

| (a) Quantized SGCS1 | (b) Quantized SGCS2 |
| --- | --- |

Figure 5	CDF of CSI-PAI SGCS1 and SGCS2 for 16 CSI-RS ports, FDD SCS=15kHz, 4Rx, TDLA30-20.

| (a) Quantized SGCS1 | (b) Quantized SGCS2 |
| --- | --- |

Figure 6	CDF of CSI-PAI SGCS1 and SGCS2 for 16 CSI-RS ports, TDD SCS=30kHz, 2Rx, TDLC300-20.

| (a) Quantized SGCS1 | (b) Quantized SGCS2 |
| --- | --- |

Figure 7	CDF of CSI-PAI SGCS1 and SGCS2 for 16 CSI-RS ports, TDD SCS=30kHz, 4Rx, TDLC300-20.

### Statistics of SGCS1/SGCS2

Another test metric could be a ratio of the first SGCS and second SGCS, that is, SGCS1/SGCS2. Figure 8 and Figure 9 show CDFs of SGCS1/SGCS2 for 16 CSI-RS ports with TDLA30-20 and TDLC300-20, respectively.

If we focus on 90%-ile of CDF, the ratio becomes about 1.5 and 1.2 for SNR > 0dB for TDLA30-20 and TDLC300-20, respectively. For SNR<0dB, the 90%-ile of CDF becomes larger because of lower SGCS2 values.

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 8	CDF of ratio CSI-PAI SGCS1/SGCS2 (quantized), 16 CSI-RS ports, FDD SCS=15kHz, TDLA30-20

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 9	CDF of ratio CSI-PAI SGCS1/SGCS2 (quantized), 16 CSI-RS ports, TDD SCS=30kHz, TDLC300-20

## Test cases

We have shown our simulation results of two possible metrics for CSI-PAI reporting requirements above. However, the issue of these metrics is that UE can report any SGCS values. We therefore propose checking the reported CSI-PAI together with PDSCH throughput with the reported predicted PMI, because the UE supporting CSI-PAI reporting (FG 58-3-5) should also support the predicted PMI reporting (FG 58-3-1).

From our simulation results of PDSCH throughput with follow PMI, the SNR test point could be between 0 to 10dB. In this region, since the 90%-ile values of SGCS1 and SGCS2 are very close, we prefer to use the statistics of the ratio SGCS1/SGCS2 as the test metric.

Proposal 6: If UE supports the performance monitoring for CSI prediction model, check the statistics of the ratio SGCS1/SGCS2 together with throughput ratio of follow predicted PMI and random Rel-15 TypeI PMI.

Proposal 7: Study the feasibility to set the CSI-PAI reporting requirements with that the radio SGCS1/SGCS2 should be more than X in [90]% of the test time at SNR_AI/ML, where SNR_AI/ML is the SNR corresponding to the 90% of maximum throughput with follow predicted PMI, where X should be more than 0.

# Summary

Observations:

Observation 1: For TDLA30-20, throughput ratio of AI/ML-based PMI prediction over Rel-15 TypeI PMI (γ1_AI/ML) is high enough regardless of 2Rx/4Rx FDD/TDD.

Observation 2: For TDLA30-20, throughput ratio of AI/ML-based PMI prediction over Rel-16 eTypeII PMI (γ2_AI/ML) is more than 1.0 for 2Rx FDD/TDD and 4Rx FDD.

Observation 3: For TDLA30-20, both MCS17 and MCS19 show similar throughput ratio of AI/ML-based PMI prediction over Rel-15 TypeI PMI.

Observation 4: For TDLC300-20, throughput ratio of AI/ML-based PMI prediction over Rel-15 TypeI PMI (γ1_AI/ML) is high enough regardless of 2Rx/4Rx FDD/TDD.

Observation 5: For TDLC300-20, throughput ratio of AI/ML-based PMI prediction over Rel-16 eTypeII PMI (γ2_AI/ML) is more than 1.0 only for 2Rx FDD.

Observation 6: For TDLC300-20, both MCS17 and MCS19 show similar throughput ratio of AI/ML-based PMI prediction over Rel-15 TypeI PMI.

Observation 7: RAN1 is still discussing the CSI reporting delay for Rel-19 AI/ML-based PMI prediction.

Observation 8: The performance monitoring for CSI prediction model (FG 58-3-5) is optional feature even if UE supports the CSI prediction for UE-sided inference when N4=1 (FG 58-3-1).

Observation 9: If UE supports the performance monitoring for CSI prediction model, this UE supports the CSI prediction for UE-sided inference when N4=1.

Proposals:

Proposal 1: RAN4 should define AI/ML-based PMI prediction performance requirements for FDD 2Rx and TDD 4Rx with the following configuration: 16 CSI-RS ports, TDLA30-20, MCS17.

Proposal 2: RAN4 should define AI/ML-based PMI prediction performance requirements for FDD 2Rx and TDD 4Rx with the following configuration: 16 CSI-RS ports, TDLC300-20, MCS19.

Proposal 3: RAN4 should evaluate the AI/ML-based PMI prediction performance based on the CSI processing time with t=0. Once RAN1 concludes the UE capability of reporting delay t, RAN4 should revisit the scheduling of CSI-RS, PMI reporting, and PDSCH.

Proposal 4: For CSI-PAI reporting requirements, RAN4 should consider at least two cases: one case for the AI/ML-based CSI prediction works, and another case for the prediction does NOT work well.

Proposal 5: RAN4 should study further the metric of reported CSI-PAI to judge the AI/ML-based CSI prediction works or does not work well.

Proposal 6: If UE supports the performance monitoring for CSI prediction model, check the statistics of the ratio SGCS1/SGCS2 together with throughput ratio of follow predicted PMI and random Rel-15 TypeI PMI.

Proposal 7: Study the feasibility to set the CSI-PAI reporting requirements with that the radio SGCS1/SGCS2 should be more than X in [90]% of the test time at SNR_AI/ML, where SNR_AI/ML is the SNR corresponding to the 90% of maximum throughput with follow predicted PMI, where X should be more than 0.

# References

- R4-2514631, “WF on [116bis][112] R19 AI for air interface.

- R4-2514634,” WF on updated simulation assumption for AI based CSI prediction”, OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, CMCC, CTC, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple.

- R1-2507979, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #122bis”, RAN1.

- R4-2513974, “PMI reporting requirements with CSI prediction”, Ericsson.

# Appendix

## Simulation results of PMI reporting

### TDLA30-20

Figure 10: FDD SCS=15kHz, 16 CSI-RS ports, 2Rx, TDLA30-20

Figure 11: FDD SCS=15kHz, 16 CSI-RS ports, 4Rx, TDLA30-20

Figure 12: TDD SCS=30kHz, 16 CSI-RS ports, 2Rx, TDLA30-20

Figure 13: TDD SCS=30kHz, 16 CSI-RS ports, 4Rx, TDLA30-20

All the results consider MCS17 and MCS19 according to the simulation assumption.

| (a) MCS17 | (b) MCS19 |
| --- | --- |

Figure 10	PDSCH demodulation with 16 CSI-RS ports, FDD SCS=15kHz, 2Rx, TDLA30-20.

| (a) MCS17 | (b) MCS19 |
| --- | --- |

Figure 11	PDSCH demodulation with 16 CSI-RS ports, FDD SCS=30kHz, 4Rx, TDLA30-20.

| (a) MCS17 | (b) MCS19 |
| --- | --- |

Figure 12	PDSCH demodulation with 16 CSI-RS ports, TDD SCS=30kHz, 2Rx, TDLA30-20.

| (a) MCS17 | (b) MCS19 |
| --- | --- |

Figure 13	PDSCH demodulation with 16 CSI-RS ports, TDD SCS=30kHz, 4Rx, TDLA30-20.

### TDLC300-20

Figure 14: FDD SCS=15kHz, 16 CSI-RS ports, 2Rx, TDLC300-20

Figure 15: FDD SCS=15kHz, 16 CSI-RS ports, 4Rx, TDLC300-20

Figure 16: TDD SCS=30kHz, 16 CSI-RS ports, 2Rx, TDLC300-20

Figure 17: TDD SCS=30kHz, 16 CSI-RS ports, 4Rx, TDLC300-20

All the results consider MCS17 and MCS19 according to the simulation assumption.

| (a) MCS17 | (b) MCS19 |
| --- | --- |

Figure 14	PDSCH demodulation with 16 CSI-RS ports, FDD SCS=15kHz, 2Rx, TDLC300-20.

| (a) MCS17 | (b) MCS19 |
| --- | --- |

Figure 15	PDSCH demodulation with 16 CSI-RS ports, FDD SCS=30kHz, 4Rx, TDLC300-20.

| (a) MCS17 | (b) MCS19 |
| --- | --- |

Figure 16	PDSCH demodulation with 16 CSI-RS ports, TDD SCS=30kHz, 2Rx, TDLC300-20.

| (a) MCS17 | (b) MCS19 |
| --- | --- |

Figure 17	PDSCH demodulation with 16 CSI-RS ports, TDD SCS=30kHz, 4Rx, TDLC300-20.