# 3GPP TSG-RAN WG4 Meeting #117                                                                R4-2521450
Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on 6G UE RF

Source:	OPPO

Agenda item:	8.3

Document for:	Approval

Introduction

RAN4 starts the 6GR study from this meeting. This paper shares our view on what can be and need to be studied at this stage. Based on the experiences that learned from 5G NR, the 6GR design in RAN4 needs to be more careful to make the spec concise and accurate.

Below, we will start with reviewing the UE RF requirement framework, and find out the enhancing areas and the requirements which may depends on RAN1 conclusions. Then we will analyse the detailed RF requirements which needs to be studied further and give the proposals based on our analysis.

Discussion

RF requirement framework

The UE RF requirement frameworks for OFDM system have been established in 4G, and then reused in 5G. It is expected that most of the requirements will continue to be reused in 6G, considering RAN1 has decided in their first meeting that the OFDM will be used the waveform in 6GR. And the waveform has much impact to RAN4 especially the Tx requirements.

- RAN1 has decided to continue use OFDM in 6GR. From RAN4 perspective, most of the 5G NR UE RF requirement frameworks can be reused for 6GR.

Below gives the initial view on the RF requirements for 6GR where some requirements may depend on RAN1/2 discussion while some requirements can reuse the 5G NR framework especially for the legacy bands. RAN4 discussion can start from the requirements which have less RAN1/2 dependants.

- Consider take below table 1 as starting point for 6G UE RF discussion.

Table 1 Initial view on UE RF requirements for 6GR

| RF requirements | RAN4 scope | Whether depends on RAN1/2 |
| --- | --- | --- |
| Max Tx power | - Discuss power class framework for single CC and for band combinations in 6G - Discuss default power class for FR1/2/3 - Discuss the tolerance for each possible PC | MIMO capability and UE types discussion in RAN1 may have impact on this topic |
| MPR | - Streamline MPR tables for 1T/2T/MIMO/TxD/different UE types… - Discuss MPR reduction and improvement schemes | Waveform, modulation, and UE types discussion in RAN1 may have impact on this topic |
| AMPR | - Depends on whether new special regulation requirements for the new bands | N |
| Pcmax | - Discuss how to solve “power class fallback” schemes in 5G, e.g., SAR related - Discuss how to simplify the Pcmax definitions, e.g., SRS IL, power boosting - Discuss the approaches to enable UE fully utilize its PA capabilities | RAN1/2 power control and PHR reporting may have impact on this topic |
| Min Tx power | - Most likely can reuse 5G framework | N |
| OFF power | - Most likely can reuse 5G framework | N |
| ON/OFF time mask | - Reuse 5G framework in general ON/OFF time mask - FFS on how to harmonize the switching related time masks | RAN1/2 Tx/Rx switching related schemes may have impact on this topic |
| Power control tolerance | - Most likely can reuse 5G framework | RAN1/2 power control may have impact on this topic |
| Freq error | - Most likely can reuse 5G framework | N |
| EVM | - Discuss EVM requirements for high modulation, and non-uniform constellations - Discuss EVM relaxation schemes for high modulation | RAN1/2 highest modulation order and constellation design may have impact on this topic |
| Carrier leakage | - Most likely can reuse 5G framework | RAN1 physical design may have impact on this topic, e.g., whether the DC will carry data |
| IBE | - Most likely can reuse 5G framework | N |
| Occupied BW | - Most likely can reuse 5G framework for some CBWs - The spectrum utilization discussion in system parameter part will impact this requirement | RAN1 spectrum utilization discussion may impact this topic |
| SEM | - FFS on whether new SEM requirement is needed for the new spectrum, if not can reuse 5G framework | N |
| ASEM | - Depends on whether new special regulation requirements for the new bands | N |
| ACLR | - Most likely can reuse 5G framework for PC3/2/1.5 ACLR - FFS on the new power class demands | RAN1 physical design may have impact on this topic |
| SE | - Most likely can reuse 5G framework - OOB boundary | N |
| Tx intermodulation | - Most likely can reuse 5G framework | N |
| Rx sensitivity | - Discuss the REFSENS for new high bands, e.g., NF and SNR - Review REFSENS for larger CBW and clarify the impact factors. | RAN1 physical design may have impact on this topic, e.g., the SNR demodulation target |
| Max input level | - Most likely can reuse 5G framework | N |
| ACS | - FFS | RAN1 physical design may have impact on this topic |
| IBB | - FFS | N |
| OOB | - FFS | N |
| Narrow band blocking | - FFS | N |
| Spurious response | - FFS | N |
| Rx intermodulation | - FFS | N |

FR1 TX requirements

Power class for FR1

      - General view on power class

Power class is the most important requirement among the UE RF Tx requirements. A good power class definition should make sure UE can fully utilize the PA power capabilities while distinguish different levels of UEs in terms of Tx power.

- Power class definition should make sure UE can fully utilize the PA power capabilities while distinguish different levels of UEs in terms of Tx power.

Currently in RAN4, the power class definition mainly includes the power class for single CC and power class for band combinations. Below will review the current status and see what can be optimised based on the experience in 5G.

      - Power class for 1CC

        - Power steps among power classes

In NR, the power class for FR1 1CC has specified PC5, PC3, PC2, PC1.5, PC1. Among them the PC5 is for unlicensed bands, and PC1 is for public safety or cab-radio UE. And for handheld UE, PC3/2/1.5 are mainly used.

In below figure 1, it summarizes all the power classes in FR1. It can be seen that 3dB power step has been followed among PC5/3/2/1.5, but the step becomes 2dB between PC1.5 and PC1. In Rel-20, there are some discussions in RAN#109 on whether to introduce a new PC1bis for UE with 4x26dBm PAs.

When PC1 was introduced in the past, it was based on one public safety PA which can only achieve 31dBm. Now for UE with multiple handheld PAs, it can achieve 32dBm which is 3dB step comparing to PC1.5. If further consider the +/-2dB tolerance, then the power levels will be much overlapped between PC1.5 and PC1 for handheld UE.

The power class definition in RAN4 should be streamlined from the beginning and it is proposed to keep 3dB power step for all the power classes.

- Less than 3dB power step makes the power classes are not well distinguished and has much overlapping power.

- Keep 3dB power step among power classes.

Figure 1 summary of power class definition in 5G

        - How to handle the power gap b/w UE max Tx power and power class reporting

For UE with multiple PAs in single CC, the total power sometimes is not exactly the same as the power class definition as shown in below table 2. It can be seen, in some PA implementations, the gap between max Tx power and power class can reach 1.8dB.

Although RAN4 has defined the +/-2dB power tolerance together with the power class, this tolerance is mainly for the purpose of accommodating massive production variations. It cannot be used to solve this power gap.

- For UE with multiple Tx PAs, the gap between max UE power capability and power class can reach 1.8dB.

- The +/-2dB power tolerance was introduced to accommodate massive production variations.

Table 2 Gap between max Tx power and power class

| UL MIMO | UE PA capability | Max Tx power (dBm) | Power class report | Gap between max Tx power and power class (dB) |
| --- | --- | --- | --- | --- |
| 2Tx | PC3+PC2 | 27.8 | PC2 | 1.5 |
| 3Tx | PC3+PC3+PC3 | 27.8 | PC2 | 1.5 |
|  | PC3+PC2+PC2 | 30 | PC1.5 | 1 |
|  | PC2+PC2+PC2 | 30.8 | PC1.5 | 1.8 |
| 4Tx | PC3+PC3+PC3+PC2 | 30 | PC1.5 | 1 |
|  | PC3+PC3+PC2+PC2 | 30.8 | PC1.5 | 1.8 |
|  | … | … | … | … |

This problem is not severe in the past because most of the commercial UE can only support one PA with PC3 or PC2 mostly or support two equal PC3/PC2 PAs. However, in 6G, it is very likely that UE will support more Tx antennas like 2Tx/3Tx/4Tx with various PA capabilities then the total power will be different from the defined power class.

Whether and how to solve this upcoming issue needs to be considered in the beginning of the 6G power class definition. One way might be to borrow the 5G power boosting scheme that allow UE to indicate the increased power comparing to power class and keep the +/-2dB power tolerance.

Figure 2 fully utilize UE PA capabilities by power boosting

- Study how to handle the issue of power gap b/w UE max Tx power capability and power class reporting in the beginning of 6G

- Consider reuse the power boosting scheme that allow UE to report the boosted power comparing to the reported power class.

Power class for band combination

In 5G, the total power of the band combination has been used to define the power class. However, this kind of power class definition has put many restrictions on fully utilize of UE power capabilities especially when the total power is not exactly the same as the power class. With many painful discussions in 5G era, it has been recognized that total power limitation is unnecessary for band combinations.

Therefore, in 6G, it is proposed to remove the power class definition of band combinations and allow UE to transmit the max power of each band in the BC. However, it should be noticed that the per band power capability in a band combination might be different from the single band.

As shown in figure 3, for band A and band B each has two PC2 PAs, and can reach PC1.5 for single band, then in the band combination, this UE can transmit with one PA in each band, i.e., 26dBm of each band, which is different from the single band power class. Therefore, the max power of a band combination is determined by per band per BC power class.

Figure 3 Power class definition for the band combination

- For band combination, not define the total power limitation. Rely on the power capability of each band in the BC (ue-PowerClassPerBandPerBC) to determine the max Tx power.

Power class for different MIMO layers

In 5G, UE only report one power class for each band and no differentiation of MIMO layers. However, as shown in figure 4, for UEs not support TxD (most of commercial UEs don’t support it), the max power capability for single port configuration is much lower than 2/3/4 Layer UL MIMO. This leads to UE MOP capability actually is different for different MIMO configurations.

If only report one power class for each band, then the UE in figure 4 can only report PC2 which makes 3dB power loss for the UL MIMO.

Today this problem might be not severe due to no (or not many) UE supports PC1.5 in the market, later on when PC1.5 is widely adopted in 6G, then this inconsistency will be widely shown. And this issue needs to be considered in the power class definition and allow UE to report different power class for different MIMO Layers.

Figure 4 Power class difference among MIMO layers

- For UEs not supporting TxD, the power capability could be different for different MIMO layer configurations, like PC2 in single port, but PC1.5 for UL MIMO. This power difference cannot be differentiated with 5G NR per band power class reporting.

- For single band power class reporting, allow UE to report different power class for different MIMO Layers.

Default power class

In 5G NR, PC3 was defined as the default power class, and then HPUE like PC2/1.5 and lower power class like PC5 were introduced in a later stage. Currently, PC2 has been widely supported on the market and be considered useful to improve the UL coverage.

In 6GR, considering the current industry status, it might be useful to define PC2 as the baseline especially for TDD bands. For the FDD bands, there might be still some difficulty in supporting PC2. Whether PC2 can be defined as the baseline is FFS.

- For TDD bands, PC2 has been widely supported on the market.

- For TDD bands, PC2 is defined as baseline. FFS whether PC2 or PC3 is baseline for FDD bands.

MPR

MPR definition

MRP requirements were defined based on configurations of waveform, modulation, RB allocation, power class, UE types (antenna isolation), MIMO configuration, and PA/RFIC models. And it has some dependent on RAN1 discussion especially the waveform and modulation. RAN4 may needs to wait for RAN1 conclusions before can really start the requirement discussions for 6G.

However, in the SI, RAN4 indeed can consider which aspects could be improved inside RAN4 like the PA models/RFIC impairments and how to align between measurements and simulations. Below table 3 has summarized the factors which impact MPR definition and also the status.

Table 3 Factors impacts MPR definition and its status

| Factors impact MPR | Status and what RAN4 can do |
| --- | --- |
| waveform | - RAN1 has decided to continue use DFT-S-OFDM and CP-OFDM as baseline for the UL, but still keep open for further optimisations or even new waveforms. - RAN4 can use DFT-S-OFDM and CP-OFDM as starting point in MPR discussion. |
| modulation | - RAN1 has decided to use uniform modulations up to 256QAM as basis for UL, but FFS on other enhancements like non-uniform modulations - RAN4 can use uniform modulations as starting point in MPR discussion. |
| power class | - RAN1 has some discussion on the mandatory power class in each device type - RAN4 needs to support all power classes in the MPR definition, therefore can start the work without impact from RAN1. |
| UE types (antenna isolation) | - RAN1 has discussed a lot on device types, however, mainly on capability reporting/mandator features perspective - The UE types considered in RAN4 mainly from UE requirement definition perspective which is somehow different from RAN1 discussion. In MPR discussion, RAN4 can focus on how much dB antenna isolation need to be considered and how this impacts the RIMD in the MPR simulation/measurements. |
| MIMO configuration | - Although RAN1 has not start the discussion in MIMO, the 5G NR MIMO configurations can be used as starting point. |
| PA model | - RAN4 has used the polynomial PA models to simulate the MPR/AMPR for a long time. RAN4 can discuss which kind of PA models to be used in 6G like fixed biased model, APT model, etc. to better align with the measurements. |
| RFIC models | - Carrier leakage, IQ image, CIM3 has been modeled in the MPR simulation to mimic the RFIC impairments. These factors were agreed in the beginning of 5G, RAN4 may need to review these RFIC impairments in 6G. |
| Simulation and measurements | - Both methods have been used in 5G to define the requirements, however, sometimes they are not aligned well. How to better align them might need to be further considered. |

- In MRP requirements definitions, waveform, modulation, RB allocation, power class, UE types (antenna isolation), MIMO configuration, and PA/RFIC models need to be considered. Among them some of the factors have some dependent on RAN1.

- RAN4 can focus on below aspects to study the 6G MPR

        - Waveform: use DFT-S-OFDM and CP-OFDM as starting point

        - Modulation: use uniform modulations as starting point

        - Power class: take PC2 as starting point

        - UE types: use handheld UE as starting point, and FFS on how much dB antenna isolation need to be considered for other UE types

        - MIMO: single port as starting point

        - PA model: discuss which kind of PA models to be used in 6G, e.g., fixed biased model, APT model, etc. and also consider the new high bands

        - RFIC impairment: use 5G carrier leakage, IQ image, CIM3 assumptions as starting point and further discuss whether some updates can be made

        - Study how to better align simulation and measurements results.

In 5G NR, the inner/outer RB allocations are defined based on QPSK and apply to all modulation. Actually， the higher-order modulation have different MPR format compared with lower-order modulation, as shown in Figure 5:

MPR reduction with requirement relaxation

In 5G Rel-19, MPR reduction has been discussed a lot under several requirement relaxation conditions like ACLR/SEM relaxation, and in Rel-20 the MPR reduction based on EVM relaxation will also be considered. However, all these MPR reductions are optional features based on UE capability report and NW enable/disable. It means, UE still needs to comply with the legacy requirements. In 6G, it might be more meaningful to study whether these requirement relaxation scenarios could be used as baseline to increase UE Tx power.

- Study whether the requirement (ACLR/SEM/EVM) relaxation scenarios could be used as default in 6G to make it more useful.

MPR calibration updates

        - CBW/RB allocations

In 5G, RAN4 has defined some PA calibration configurations (table 4) to align the simulation results and also measurement results. Many of these configurations are inherited from LTE like the CBW/RB allocation/waveform, etc. At the beginning of 6G, it might be a good chance to review these calibration conditions to better fit for 6G requirement definition.

Table 4 PA calibration configurations in 5G

|  | 5G |
| --- | --- |
| CBW, scs, RB allocation | 20MHz, 15khz scs, 100RB@0 |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Targe power level to meet ACLR | 1dB MPR |
| carrier leakage | -28dBc |
| IQ image | -28dBc |
| CIM3 | -60dB |

In RAN4#108 and following meetings, there were some discussions on whether 20MHz with 100RB allocation can be updated to 40MHz with full RB allocation considering the 20MHz with 100RB allocation is not the worst case in terms of spectrum utilization as can be seen in below figure [2]. And it pointed out that current less stringent PA calibration point will lead to worse PA calibration results and then lead to higher MPR defined. However, in the following meetings, some other paper [3] also did some analysis and pointed out that the calibration CBW impact might be different for different UEs and considering the potential impacts to legacy UE this discussion was ended without change of the calibration conditions.

Figure 5 Spectrum utilization comparison between 20M/100RB and 40M/full RB

- The CBW/RB allocation used in 5G MPR calibration is not the worst case in terms of spectrum utilization which makes the calibrated PA has worse linearity and finally large MPR requirements.

At the beginning of 6G, no legacy UEs need to be considered which makes it a good chance to further review this CBW and RB allocation used in the MPR calibration.

To better understand how much the impact of using 20MHz/100RB@0 comparing to 40MHz/full RB, we recalibrated the PAs with these two configurations, and under different RFIC impairments, as can be seen in figure 6. The calibrated PA linearity difference seems quite small.

Figure 6 PA calibration results with 20MHz/100RB@0 comparing to 40MHz/full RB

The MPR simulation results for QPSK and 64QAM with these two calibrated PA can be found in figure 7. It can be seen no much difference can be observed.

Figure 7 MPR results with PAs calibrated with 20MHz/100RB@0 comparing to 40MHz/full RB

- The PA calibration results and MPR simulation results don’t show obvious difference between the calibration conditions of 20MHz/100RB@0 and 40MHz/full RB.

Therefore, the CBW and RB allocations used in 5G (20MHz/100RB@0) can be reused in 6G.

- Continue to use 20MHz/100RB@0 as the MPR PA calibration condition in 6G.

        - Carrier leakage/IQ image

For carrier leakage and IQ image, in 5G NR, -28dBc has been used in the calibration with QPSK modulation. However, for the high modulations like 256QAM, this value needs to be tightened to -34dBc to pass the EVM requirements. Fundamentally, there is no difference in RFIC carrier leakage and IQ image in terms of modulation orders, therefore, it might be straight forward to tighten carrier leakage and IQ image from -28dBc to -34dBc.

- To pass the EVM requirements of 256QAM, -34dBc for carrier leakage and IQ image has been used in 5G MPR simulation/measurements. There is no difference in RFIC carrier leakage and IQ image in terms of modulation orders.

The impacts of tightening carrier leakage/IQ image to PA calibration have been analysed with simulations as shown in figure 6, and MPR requirement impacts to 64QAM and 256QAM as shown in figure 8. It can be seen, the carrier leakage/IQ image tighten to -34dBc can improve the MPR for 64QAM around 0.6dB, and is necessary for 256QAM in central RB allocations.

Figure 8 MPR results with PAs calibrated with -28dBc comparing to -34dBc

- Carrier leakage/IQ image tighten to -34dBc can improve the MPR for 64QAM around 0.6dB in some allocations, and is necessary for 256QAM.

- The carrier leakage and IQ image condition for MPR calibration is tightened from -28dBc to -34dBc in 6G.

        - PA models

In MPR requirement definition, PA models are used in the simulation platforms to derive the final MPR requirements. The PA models used in the simulation play a key role. However, there is no aligned single PA model in the LTE and NR MPR discussions due to various PAs on the market and implemented in the UE. In reality, the PA models used in each company’s platform are different, for example, some are based on measured PA models used in the product, others are based on PA models shared in papers. This is one of the root causes that even after PA calibration the MPR simulation results are still different. It is expected this trend will continue in 6G.

- Due to various PAs on the market and implemented in UE, no single PA model can be universally used in the simulations. This trend will continue in 6G.

- The PA models used in 6G MPR simulation is still based on each company’s own choice.

Regarding the detailed PA models, fixed biased PA model has been widely used in the past, however, this kind of PA is different from the real PAs used in the products which is either APT PA or ET PA. This is one of the reasons that lead to MPR difference between simulation and measurement.

To better align with the real products, APT PA is preferred in principle. The question is how much difference between fixed bias PA and APT PA which is unknow up to now. To better understand this issue, the APT PA model from real PA and also fixed bias PA from real PA are both implemented in our MPR simulation platform. Some comparisons have been made in terms of MPR requirements.

It is understood that for a typical APT PA, it may include several operating modes/statuses like high power mode (HPM), middle power mode (MPM), and low power mode (LPM), etc. The APT PA will change its operating status when the desired output power is different, for example change from HPM to MPM when the output power is below the targeting power to reduce power consumption but with the sacrifice of PA linearity. This will lead to higher MPR desired comparing to the fixed bias PA.

- APT PA is widely used in UE implementation, and the APT PA will change its working modes when the output power is different which could reduce the power consumption but with the sacrifice of PA linearity, i.e., larger MPR.

Based on the simulation results with fixed bias PA and APT PA, it was observed that when the required MPR is not large with fixed bias PA, e.g., less than 7dB for PC3 (this value could be different for different APT PA), then there is no difference comparing to APT PA. However, when the MPR is larger than that it will be different. Among the simulation results, it was observed that the difference could be larger than 5dB in some configurations.

Therefore, in 6G the APT PA should be used in MPR simulations especially when the MPR is large to better align with the real products and also measurements.

- From simulation results it can be seen that the MPR required for APT PA could be 5dB higher than fixed bias PA in some configurations.

- The APT PA models is encouraged to be used in 6G MPR simulation to better align with the real products and measurements.

MPR summary

To summarize, the below table is proposed as starting point for 6G MPR simulations.

- The 6G MPR simulation calibration conditions is as below table.

|  | 6G |
| --- | --- |
| CBW, scs, RB allocation | 20MHz, 15khz scs, 100RB@0 |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Targe power level to meet ACLR | 1dB MPR |
| carrier leakage | -34dBc |
| IQ image | -34dBc |
| CIM3 | -60dB |

Pcmax

Pcmax,h and power boosting

Pcmax gives the UE Tx power higher and lower limit under certain configurations. And the lower limit has drawn more attention usually than the higher limit since the lower limit gating UE from working worse. However, the higher limit now also has become more and more limiting UE performance especially for UEs with several Tx PAs as discussed in section 2.2.2.2.

- Pcmax,h has become the gating factor for UE to transmit higher power especially when UE has multi-Tx PAs.

To allow UE to transmit higher Tx power whenever possible, power boosting could be one of the solutions which has been adopted in 5G, e.g., delta Ppowerboost in the equation. Another solution is to remove this Pcmax,H and allow UE to transmit as high Tx power as possible, this may draw the concern of coexistence. Both solutions could be discussed in 6G study phase, and using power boosting could be more feasible.

- Power boosting or remove the Pcmax,h are two potential approaches to allow UE transmit higher power whenever possible.

- Study how to enable UE to fully utilize its PA capabilities. And power boosting approach is preferred comparing to remove the Pcmax,h limits.

SAR and power fallback

Schemes to solve SAR issue have been widely studied in 5G including single CC HPUE and band combination HPUE, and for different power classes. Much meeting time has been spent on this “SAR solutions”, but unfortunately none of the SAR solutions except PAPR has been implemented in the UE.

Meeting time has been wasted in debating how the duty cycle solutions could be designed, and what is the duty cycle relationships among different power classes, and its power fallback. This has made the spec very complex. This kind of “SAR solution” discussion shall be avoided in 6G.

- Duty cycle-based SAR solutions in RAN4 spec is very complex, but it hasn’t been implemented in UE. Only PMPR is applied.

- In 6G, only rely on PMPR to solve SAR issue and no other SAR solution is to be defined.

EVM

General EVM

EVM requirement is tightly connected with RAN1 modulation definitions, and in last RAN1 meeting, it has been decided that the uniform modulations are supported as basis. And other modulation schemes are FFS.

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Actually, the agreed uniform modulations are same as 5G, and it is expected that the EVM requirements will be the same for these modulations. What RAN4 needs to pay attention is the potential enhancements in 6G no matter even higher modulations like 1024QAM in UL with AI/non-AI based modulation, or non-uniform modulations like probabilistic shaping as shown in below figure 9 or geometric shaping as shown in figure 10.

All these higher modulation, new non-uniform modulations may need RAN4 to fully study its impacts in terms of implementation complexity, requirements and performance.

Figure 9 Example of probabilistic shaping for 64QAM

Figure 10 Example of geometric shaping for 256QAM

- Besides of the uniform modulations already defined in 5G, some enhancements including even higher modulation, and non-uniform modulations are under discussion in RAN1 which will have impact to RAN4 EVM requirements.

- RAN4 needs to closely follow the RAN1 discussion on even higher modulations in UL like 1024QAM with AI/non-AI based schemes, and the non-uniform modulations like probabilistic shaping or geometric shaping. And study the impacts in terms of implementation complexity, requirements and performance.

EVM relaxation

        - General work scope/arrangement

During the last RAN4#116bis meeting, the TX EVM has been discussed and the WF agreement is captured as below:

- Agreements

  - For Tx EVM relaxation

    - Discuss non-AI based UL demod in UE RF thread

      - The impact on BS receiver could be considered.

    - Do not discuss AI based solution(s) in UE RF thread, except the potential common part.

    - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible.

      - Strive to study TX non-linearity sources, including PA, in this thread

      - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G.

    - Strive to avoid overlapping work between different threads

    - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g.

      - Which aspects need to be considered to conclude on feasibility

      - Need for network control

As can be seen from the WF agreement, companies would like to discuss the common part for AI and non-AI based solutions for this TX EVM relaxation. From the RAN4 AI progress perspective, it is always that to have non-AI based solution first and then the AI-based solution come out and the gain of using AI is compared as using the non-AI based solution as baseline. For this TX EVM relaxation, we see similar procedure still works. Also consider it is the timeline of SI of 6G, to save the duplicated discussion, it is proposed in the WF that the common part of both AI and non-AI based common parts can discussed. We believe this approach is quite time efficient and it is a good wayforward to continue this topic discussion.

- Usually, RAN4 non-AI based solution will be done prior to AI based solution.

- To avoid duplicated discussion, the common part for AI and non-AI based solution will be studied.

With the above observation, we believe the procedure of RAN4 work arrangement should be:

1, The common part of non-AI and AI based solution is discussed and aligned.

2, Under the UE RF thread, the non-AI based solution can go prior to AI-based solution to provide a baseline performance.

3, The evaluation assumption including PA non-linearity, UE RF requirements for MPR simulation will be based on the available information on the table. Further change can be applied if new PA model and/or UE RF requirement is introduced.

- Below RAN4 work arrangement to be followed:

- The common part of non-AI and AI based solution is discussed and aligned.

- Under the UE RF thread, the non-AI based solution can go prior to AI-based solution to provide a baseline performance.

- The evaluation assumption including PA non-linearity, UE RF requirements for MPR simulation will be based on the available information on the table. Further change can be applied if new PA model and/or UE RF requirement is introduced.

With the above proposal 1, we can further discuss the TX EVM relaxation.

        - Common part for AI and non-AI based solution

To consider the common part for AI and non-AI based solution, we can start with the specific part for AI-based solution and the rest of the evaluation will be the common part. From our understanding, the AI based solution for TX EVM relaxation especially refer to the AI DPoD algorithm for the BS receiver. Besides the AI DPoD algorithm part, the rest is the evaluation method, simulation assumption and the justification metric are all the common part for AI and non-AI based solution.

- The specific part for AI based solution is the BS AI DPoD algorithm.

- The common part of AI and non-AI based solution is evaluation method, simulation assumption and the justification metric.

For the evaluation method, the TX EVM relaxation directly brings the output power increase. This helps to improve the UL coverage of the UE. However, it is foreseeable that the SNR will be worse with worse EVM as signal quality even with the BS DPoD. So together there are two parts, one is the UE output power increase after EVM relaxation compared to the output power without EVM relaxation. The other one is the SNR degradation (potential) for UE with higher output power and relaxed EVM compared to UE with lower output power and legacy EVM.

- One of the evaluations of relaxed EVM is the UE output power increase after EVM relaxation compared to the output power without EVM relaxation.

- The other evaluation of relaxed EVM is the potential SNR degradation for UE with higher output power and relaxed EVM compared to UE with lower output power and legacy EVM.

Hence the above two metrics should be used to evaluate the TX EVM relaxation gain.

- To evaluate the UE output power with EVM relaxation.

- To evaluate the potential SNR degradation for UE with higher output power and relaxed EVM compared to UE with lower output power and legacy EVM.

With the above agreed two parts for evaluation, then we can further discuss the evaluation assumptions. For the UE output power, the PA model and UE RF assumptions needs to be aligned. From our understanding, currently the 6GR is also discussing the PA model under system parameter agenda and this can be one of the potential UE PA models. Another one is to reuse the NR PA model seems also acceptable. Considering the RAN4 MPR simulation work only require to calibrate the PA at specific point and no further alignment is needed, we believe still use such method is proceedable.

- RAN4 MPR simulation only require to calibrate the PA at specific point.

- Same PA calibration point as NR is proposed.

- Same RF impairment of NR MPR simulation is proposed.

For the potential SNR degradation, link level simulation is needed. Hence it is proposed to further align the link level simulation assumption.

- To further align the link level simulation assumption.

OOB boundary

The boundary between out of band and Spurious emission is defined in ITU-R SM.1539, which is generally specified as being separated from the centre frequency by 250% of the necessary bandwidth, needs to be modified for narrow-band and wideband (including multicarrier) systems, as shown in Table 5:

Table 5: OOB boundary defined in ITU-R SM.1539

And Table 6 provides guidance for determining general values of separation between the centre frequency of an emission and the beginning of its spurious domain. A minimum separation applies to the narrow- band case, while the normal and wideband cases have separate formulas for determining the boundary.

Table 6 Guideline values for frequency separation between the centre frequency and the boundary of the spurious domain

- For wideband case, 1.5BN+BU boundary can be applied only when BN>BU in related frequency range.

According to the description in ITU-R SM.1539, BN is the UE CBW from RAN4 perspective. Therefore, here based on CBW, we analysed which CBWs can apply for the boundary of 1.5BN+BU and 2.5BN separately, as listed in Table 7:

Table 7 The FR1 OOB boundary for NR CBW based on ITU-R SM.1539

| NR Bands | BU | NR OOB boundary |  |
| --- | --- | --- | --- |
| f < 1 GHz | 10 MHz | 2*CBW for 5MHz, 10 MHz CBW | CBW+10MHz for CBW >10MHz |
| 1GHz< f< 2.7 GHz | 50 MHz | 2*CBW for CBW <= 50 MHz | CBW+50MHz for CBW > 50 MHz |
| f >3 GHz | 100 MHz | 2*CBW for CBW <= 100 MHz | - |

Currently, NR OOB boundary in Spec was defined as listed in Table 8 for all FR1 bands:

Table 8 NR OOB boundary in Spec

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWChannel + 5 |

According to the analysis in Table 7, NR OOB boundary maybe need modify as list in Table 9:

Table 9 Modified NR OOB boundary for FR1 bands

| Channel bandwidth | Frequency | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | f < 1GHz | 2*CBW |  |  | CBW + 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1GHz< f< 2.7GHz | 2*CBW |  |  |  |  |  |  |  |  |  |  | CBW+50 MHz |  |  |  |  |
|  | f >3GHz | 2*CBW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- NR FR1 OOB boundary in current Spec does not align with the definition of ITU-R SM.1539.

- In 6G, RAN4 need study whether FR1 OOB boundary need to define according to different CBWs in different frequency ranges, separately, as shown in Table 9.

Coherent MIMO

Coherent UL MIMO is one of the important MIMO schemes defined in RAN1 in 5G, and RAN4 has specified the requirements in 38.101-1 as below table, however, this scheme hasn’t been implemented in UE. In the past releases, there were some discussions on how to enable this feature in the real network like calibration in UE side, etc. but was not able to be well studied. In 6G, probably this can be studied.

Figure 11 Coherent UL MIMO requirement

- Coherent UL MIMO was defined in 5G, but no be supported in UE/network.

- Study how to enable coherent UL MIMO to improve UE performance.

FR1 Rx requirements

REFSENS

REFSENS is the basis of UE RF Rx requirements. In 5G, it has been defined based on below equation where the SNR is determined by the demodulation simulations which depends on RAN1 physical design, and the NF is connected with the targeting bands.

For 6G, the equation can be reused, and RAN4 can start with the discussion of targeting bands and its related NF. After the spectrum utilization in system parameters is clear then the REFSENS will be much clearer.

REFSENS (dBm) = -174dBm/hz + 10lg(RxBW) + NF + 10lg(SU) – DivGain + SNR + IM - SNR = -1dB - DivGain = 3dB - IM = 2.5dB - NF = 10 for n78 and n79

- The REFSENS calculation equation below can be reused in 6G, and 3dB for diversity gain with 2Rx antennas, 2.5dB implementation margin. For the NF, RAN4 can start with the discussion of targeting bands and analyze its related NF.

- REFSENS (dBm) = -174dBm/hz + 10lg(RxBW) + NF + 10lg(SU) – DivGain + SNR + IM

5G REFSENs for TDD bands are defined using scaling method based on the REFSENs of 5MHz or 10MHz, e.g., -97 + 10log10(NRB/25), there is no problem for larger CBW, 6G could adopt the same method to define the REFSENs for TDD bands.

For FDD bands in 5G, some relaxations are added to the REFSENs of larger CBW based on the scaling method due to the impact of UL transmitter to its own DL receiver. Things got fuzzy as time goes by, few people can know the relaxed reasons clearly. Therefore, from 6G day one, the relaxations for the REFSENs of larger CBW of FDD bands especially for new larger CBW should define clearly and carefully.

- The REFSENs relaxation for larger CBW of FDD bands is not very clearly in 5G.

- Review REFSENS for larger CBW and clarify the impact factors.

Other Rx requirements

5G Rx requirements including ACS, in-band blocking, out of band block, narrow band block, spurious response and Wide band Intermodulation are defined for below 2700MHz and above 3300MHz, separately.

For ACS and in-band blocking, the interferer is one bandwidth signal, as shown in Figure 12:

a. below 2700MHz

b. above 3300MHz

Figure 12 The frequency location of ACS and in-band blocking

The bands below 2700MHz are re-farming from 3G/4G, to consider the existence with 3G/4G, the bandwidth of interferer used 5MHz. and the bands above 3300MHz are 5G spectrum, there is no legacy RATs, it only needs consider the intra-RAT co-existence, the bandwidth of interferer adopt the same CBW with wanted signal, as shown in Table 10.

Table 10 The interferer bandwidth of ACS and in-band blocking

| Frequency range | Channel bandwidth for wanted signal (CBW) | Bandwidth for interferer |
| --- | --- | --- |
| Below 2.7GHz | 5, 10, 15, 20, 25, 40, 50, 60,80, 90, 100MHz | 5MHz |
| Above 3.3GHz | 10, 15, 20, 25, 40, 50, 60,80, 90, 100MHz | CBW |

- ACS and in-band blocking in 5G have different interferer bandwidth for below 2700MHz and above 3300MHz bands.

In 6G, the same situation will exist for re-farming bands, especially, for those re-farming bands are still exist in 3G/4G, it still needs consider the co-existence among different RATs. In 6G, to unified the requirements, RAN4 could study whether can adopt the same the bandwidth of interferer for all FR1 bands for ACS and in-band blocking.

- For ACS and in-band blocking, RAN4 could study whether can adopt the same the bandwidth of interferer between the bands below 2700MHz and above 3300MHz including 6G new bands.

For out of band blocking, spurious response and wide band intermodulation, the setting of the wanted signal power are different for the bands below 2700MHz and above 3300MHz, as shown in Table 11 and Table 12:

Table 11 The parameters for out of band blocking and spurious response

| Frequency range | Wanted signal |  |
| --- | --- | --- |
|  | Power | Channel bandwidth |
| Below 2.7GHz | REFSENs + reused LTE value for CBW ≤20MHzscaling the value of 20MHz to other CBW | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60,70, 80, 90, 100MHz |
| Above 3.3GHz | REFSENs + reused LTE value for CBW ≤20MHz9 for other CBW | 10, 15, 20,30, 35, 40, 45, 50, 60, 70, 80, 90, 100MHz |

Table 12 The parameters for wide band intermodulation

| Frequency range | Wanted signal |  |
| --- | --- | --- |
|  | Power | Channel bandwidth |
| Below 2.7GHz | REFSENs + reused LTE value for CBW ≤20MHzscaling the value of 20MHz to other CBW | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60,70, 80, 90, 100MHz |
| Above 3.3GHz | REFSENs + 6 | 10, 15, 20,30, 35, 40, 45, 50, 60, 70, 80, 90, 100MHz |

- RAN4 should consider to unify the parameters for out of band blocking, spurious response and wide band intermodulation between the bands below 2700MHz and above 3300MHz including 6G new bands.

For narrow band blocking, it is to consider the interference coming from GSM for the bands below 2700MHz. In 6G, RAN4 should confirm whether the GSM interference also need consider.

- Discuss whether narrow band blocking can be removed from 6G.

  - Requirements for different UE types

Device types have been widely discussed in RAN1 and last RAN plenary in terms of the mandatory features. From RAN4 UE RF requirement perspective, the UE types need to be considered is how RAN4 should group the UEs and define requirements for them. In other words, RAN1 may define many device types with different mandatory features, but in RAN4 they might share same UE RF requirements. This is the difference from RAN1 discussion.

Therefore, the discussion of UE types here will focus on the applicable requirements instead of mandatory features which will be covered by system parameter agenda.

- Device types discussed in RAN1 and RAN plenary mainly focus on the mandatory features for each device type which is different from RAN4 requirement definition. RAN1 may define many device types but in RAN4 they may share same RF requirements.

In RAN4 spec, especially 38.101-1, many UE types have been mentioned like Handheld, FWA, FWA form factor, CPE, vehicle, XR, IoT, Redcap, ATG, UAV, UE with large/small antenna isolations…Some of these UE types were introduced into RAN4 spec with some specific purposes, while some are not even carefully considered. This leads to confusions in the industry on which requirement should be applied and what is the difference among these UE types. And for the UE types that are not explicitly mentioned in the spec like laptop and pad which requirement should be applied.

This has led to some chaos in the industry and frequently CRs are brought to RAN4 to clarify this but with no conclusion due to impact on many legacy UEs. For example, whether to use FWA form factor or FWA like UE or FWA UE or CPE or FWA CPE…

- RAN4 has defined many UE types but not be well organized in the 5G spec. This has caused many confusions in the industry in terms of which requirement to be applied to which UE types.

In 6G, RAN4 shall think about the UE types very carefully on how to differentiate UEs and requirements. The UE types used in RAN4 to differentiate requirements should be as less as possible, and these UE types should cover the potential future UEs on the market.

From RF perspective,

- handheld is clearly a most important UE type which is different from other UE types in terms of compact size and sensitive to cost and power consumption.

- Among non-handheld UE,

  - there is no need to further differentiate FWA and CPE which are both with large size, and in the spec they are typically considered as with large (20dB) antenna isolation.

  - The vehicle UE was considered in 5G because of supporting 2Rx antennas in 4Rx mandatory bands and with 10dB antenna isolation.

  - XR introduced in 38.101-1 was also for the sake of relaxing Rx antennas from 4Rx to 2Rx

It can be seen that for most of UE types defined in RAN4, the antenna numbers and antenna isolations were the main concern in 5G.

- For the antenna numbers, it might be not avoidable to mention about specific UE type in REFSENS (as shown in figure 13) due to the concerns from NW side on the exception of supporting less antennas than the mandatory.

- For the antenna isolation, this mainly impacts the MPR requirements (as shown in figure 14), which is not bounded with UE types actually.

Figure 13 Two Rx UE types in REFSENS

Figure 14 FWA form factor special MPR requirement

- For most of UE types defined in RAN4, the antenna numbers and antenna isolations were the main concern in 5G. Antenna numbers mainly about the relaxation from mandatory 4Rx to 2Rx for some specific UE type, and antenna isolation is mainly for the MRP requirement definition.

Therefore, as starting point it might be possible to not specifically mention which UE type is applied for the MPR requirements, but use the technical wording like antenna isolations. Any UE types that meet the antenna isolation conditions can apply the corresponding requirements.

- Consider using antenna isolations as the discriminator to differentiate applicable MPR requirements only and no bounding of UE types. Any UE that meets the antenna isolation conditions can apply the corresponding requirements.

UE capability signaling simplification

As pointed out in section 2.4, in 5G RAN4 has defined many UE capabilities to differentiate which requirement UE is complied. And this kind of signaling is not useful in BS scheduling which is only for the testing purpose. However, in the testing/certification, UE declaration approach can be applied to make it clear which requirements should be tested. This makes these signaling are meaningless.

- In the testing/certification, UE declaration approach can be applied to make it clear which requirements should be tested. There is no need to define UE capabilities just for the purpose of differentiating the applicable requirements for a UE.

In 6G, this kind of testing purpose singling should be avoided and when new signaling is introduced it shall be justified that it is useful for BS scheduling.

Besides, the UE capability granularity (per band/BC/FS/CC) also needs to be carefully considered. And avoid defining capabilities with unnecessary small granularity

- In 6G, the testing purpose singling should be avoided and when new signaling is introduced it shall be justified that it is useful for BS scheduling.

- In 6G, avoid defining UE capabilities with small granularity when it is unnecessary.

  - FR2 UE RF

In 5G, RAN4 has spent much time in defining the requirements for FR2 in the first release. Then in the following releases some efforts on the enhancements like UL/DL CA and multi-panel transmission, etc. However, in the real field it seems these kinds of enhancements were not deployed. Even the FR2 single CC network deployment is rare, and not much UE support FR2.

In 6G, the FR2 spectrum is part of the targeting spectrum, and RAN4 will define requirements for the FR2 UEs. However, based on the progress on the market, it probably more convenient to reuse the 5G NR FR2 requirements whenever possible. And refrain from enhancements in FR2 like multi-carrier or multi-panel aspects.

- In 6G, reuse 5G FR2 requirements as much as possible and focus on 1CC in the beginning.

  - Spectrum aggregation

SCMC

NR CA implementation

Intra-band contiguous CA, intra-band non-contiguous CA, inter-band CA were introduced in LTE. In LTE, intra-band contiguous CA was assumed all CCs were received by one RF chain, and intra-band non-contiguous CA was assumed each CC was received by separate RF chain. Inter-band CA was assumed each band used separate RF chain but need share antenna.

NR did some enhancement to resolve some deployment scenarios, e.g.,

- From R16, Tx switching was introduced to make CA can support MIMO through TDD mode.

- NR R19 researched whether intra-band non-contiguous CA of two CCs can share one RF chain to improve the utilization of RF Chain.

- In 5G NR Rel-19, the SDL switching was introduced to enable low-low band CA, that the implementation of the RF front-end presented challenges, these could be overcome through the switching method.

Regardless of the RF implementation using one RF chain or multiple RF chain, under CA framework, one cell only treated one CC by configured one active BWP, as shown in Figure 15:

Figure 15 The RF implementation and active BWP configurations for CA

- To resolve the deployment scenarios, Tx switching, SDL switching and intra-band NC CA sharing one RF chain were enhanced under NR CA framework, however, they were all configured all with one cell and one CC.

6G SCMC scheme and implementation

- SCMC concept

To solve the CA configuration complexity and redundant control configurations, there were proposals on Multiple carriers in single cell (SCMC) in RAN1 to enable more efficient Cell/UE management, comparing with CA, e.g., associated all CCs as one cell as show in Figure 16. It will simultaneously transmit for multiple UL CCs and/or simultaneously reception for multiple DL CCs.

Figure 16 SCMC comparing with CA

- SCMC concept is discussing in RAN1 mainly for the purpose of solving CA configuration complexity and redundant control configurations.

- Difference of CA and SCMC in implementation

It may extend from intra-band SCMC to inter-band SCMC to utilize the benefits it brings in 6G, as shown in Figure 17:

Figure 17 The RF implementation and possible active BWP configurations for SCMC

The difference from CA is one cell can treat multiple CCs. In addition, multiple CCs in one cell sharing one RF chain can be treated as one virtual CC by configured one BWP, if not, maybe multiple active BWP for multiple CCs are needed, it may depend on synchronization conditions between multiple RF chains. Anyway, how to handle the BWP depends RAN1’s design. From RAN4 RF perspective, the cell or the BWP cannot be seen, only the number of CC and RF chain can be identified. Compared the parts within the dashed frame of Figure 10 and Figure 18, it is clear that the RF implementations are similar between SCMC and CA, as shown in Figure 13 for inter-band as an example, it can support inter-band SCMC and inter- band CA.

Figure 18 RF implementation for inter-band

That is, the RF implementation supporting intra-band C CA, intra-band NC CA and normal inter-band CA can support intra-band C SCMC, intra-band NC SCMC and inter-band SCMC.

- From UE perspective, SCMC and CA can share one set of RF implementation, there is no significant difference between them.

- Study whether it is possible to define only one set of requirements for CA and SCMC when introduced in RAN1 considering CA and SCMC can share the same RF implementation.

- Mixing combination of SCMC and CA

For band combination Band A + Band B, from UE side, when a UE indicates it support inter-band SCMC Band A + Band B, it means the UE expect the Network schedule these two bands as one Cell. That is, the UE don’t expect the Network schedule these two bands as separate Cells. From Network side, when the Network schedule Band A + Band B as one cell as SCMC, it can’t schedule them as two separate cells at the same time, as shown in Figure 19:

Figure 19 Band combination Band A + Band B

Considering SCMC and CA both are aggregating carriers, there might be one situation that a band combination includes multi-carriers with SCMC and CA simultaneously as shown in figure 20. The question is whether this kind of band combination will be supported.

Figure 20 Mixing Band combination of SCMC and CA

From RF implementation perspective, the UE need support all bands at simultaneously, which caused the UE RF front-end is complex to support mixed SCMC and CA. and the simultaneously scheduling CA and SCMC is also complex.

Therefore, to simplify the band combination definition, it is proposed to not support such mixed SCMC and CA.

- The complexity of supporting SCMC + CA simultaneously is high.

- In 6G day one, mixed combination of SCMC + CA is not supported.

UL/DL decouple

Background of UL/DL decouple

In NR R19, the UL and DL is fixed for both FDD and TDD, the impact of UL to DL is guaranteed though the Tx-Rx isolation of duplexer.

For UL/DL decouple, the potential problem is that the harmonic of Band A UL may fall on Band B DL, this will cause a larger REFSENs degradation. To solve these issues, operators have proposed the concept of resource pool among different bands/CCs, NW configure one UL CC with the most suitable DL CC among these CCs, e.g., Band A UL +Band B DL.

In addition, the purpose of UL/DL decouple is to enhance coverage in the cell edge by flexible association of DL carrier and UL carrier, e.g., Low Band UL+ mid-High Band DL, or mid-High Band UL+ High Band DL.

- UL/DL decouple is discussed in RAN1 for the purpose of cell-edge performance and UL coverage.

Implementation of UL/DL decouple

RAN4 need discuss how to define the recouple pair of Band A UL and Band B DL and how to implement it.

If band A and Band B adopt separate antenna in UE RF implementation, UL/DL decouple have not any impact on UE RF implementation.

- UL/DL decouple have not any impact on UE RF implementation when band A and Band B adopt separate antenna in UE RF implementation.

Here only analysis the scenario that band A and Band B sharing one antenna. From our side, there are some options for UL/DL decouple sharing one antenna.

Option 1: Defining a new band number for the recouple pair of Band A UL and Band B DL, then it can work as a normal FDD band, the UE only needs to support this new band, don’t need to support band A and band B, as shown in Figure 21.

Figure 21 Defined a new band for recouple pair of Band A UL and Band B DL

From spectrum perspective, this new recouple band comes from Band A and Band B, if the operator has this new recouple band, it must have Band A and Band B, why the operator only deploys this new recouple band. From UE perspective, UE will support one operator’s all bands, so if the UE support this new recouple band, it must support Band A and Band B. it seems defining one new band number for this new recouple band is unnecessary and increase the complexity of UE implementation due to add new RF front-end component. In addition, introduce more new bands will cause more band combination and bring more workload.

- New RF front-end component will increase the complexity of UE implementation and new band number cause more band combinations and bring more workload.

- Avoid to define a new band for UL/DL decouple pair.

Option 2: Enable it work as one of configuration for inter-band CA Band A+ Band B, the UE needs to support CA between Band A and Band B as shown in Figure 22:

Figure 22 Recouple pair of Band A UL and Band B DL as one of CA configuration

- A UE supports inter-band CA (Band A + Band B), will support the recouple pair of Band A UL and Band B DL by CA enhancement.

- To enable more efficient Cell/UE management, UL/DL decouple based on non-CA mode can be studied.

Option 3: Association of Band A UL and Band B DL as a new aggregated scheme

From UE side, the recouple pair of Band A UL and Band B DL is also one combination, in principle, the UE will need support normal band A and Band B separately, then can support the recouple pair of Band A UL and Band B DL as one band combination.

If the cell only includes Band A UL and Band B DL without band A DL and band B UL and the UE don’t need support CA between A and Band B, then Band A UL and Band B DL can work simultaneously and share one set of RF chain band BB with Band A and band B, although new RF front-end component need introduce. The possible implementation as shown in Figure 23:

Figure 23 The recouple pair of Band A UL and Band B DL works simultaneously

To avoid impact the UE front-end implementation, the recouple pair of Band A UL and Band B DL can also work by switching between Band A UL and Band B DL, as shown in figure 24:

Figure 24 The recouple pair of Band A UL and Band B DL works by switching

- New RF front-end component will increase the complexity of UE implementation although can save one set of RF chain band BB.

- If both bands are within same band group (e.g., both are low bands), then for shared antenna UEs, this will need a new quadplexes which will increase much complexity in implementation.

- The recouple pair of Band A UL and Band B DL can work simultaneously or by switching between Band A UL and Band B DL.

Carrier-switching

Low band FDD-SDL carrier switching has been introduced in Rel-19, and FDD-FDD switching was also proposed by some companies, due to the implementation of the RF front-end presented challenges.

This kind of switching has some benefits from NW side in terms of scheduling flexibility and load balance. From single UE side, no obvious throughput gain but maybe useful when the cell is too crowded and UE can move to another cell.

Therefore, carrier switching can be further simplified from RF band baseband, because of switching bands don’t need to work simultaneously. Such as with single carrier processing capability (both RF and baseband) and function shared by switched carriers and the switching bands also can be associated as one cell to utilize the benefits it brings in 6G, as shown in Figure 25.

Figure 25 Carrier switching within inter-band

- Carrier switching is under discussion in RAN1 which is targeted to simplify from RF and baseband especially for the band combinations which cannot supported simultaneously like low band FDD + FDD.

From RAN4 perspective, carrier switching is similar as SDL switching that also can enable low-low band combination, that the implementation of the RF front-end presented challenges. The difference is the bands within carrier switching are considered as one cell but the bands within SDL switching are considered as multiple cells.

- Study the RF impacts for carrier/band switching in terms of considering the switched carriers as one cell or two cells.

The relationship among the aggregated schemes

Based on above analysis from RF implementation and utilization of the benefits of 6G, SCMC, UL/DL decouple and carrier switching can be considered as one cell, Figure 26 shows the similarities and differences:

Figure 26 The similarities and differences among different aggregated technologies

The purpose of all these new candidates aggregated technologies have some similar with enhanced CA in NR, consider enabling more efficient Cell/UE management, RAN4 and RAN1 need further down select from CA and these new candidates aggregated technologies.

- All these new candidates aggregated technologies have similar purpose with enhanced CA in NR, except the different of Cell/UE management.

- For one band combination, which aggregated technologies will be adopt need further down selection, and UE implementation complexity need to be well considered.

Band combination definition simplification

BCS simplification

In additional, the definition of BCS for each band combination increased a lot of workload and also took up a lot of space in Spec. In NR, BCS4 and BCS5 were introduced to reduce the workload. BCS4 is to indicate UE to support up to all of the channel bandwidths for the band in the band combination. To reduce the UE capability, 6G can directly define the band in band combination can support all of the channel bandwidths for this band.

- BCS4 is introduced in NR in Release 17. It can be supported in 6G day one.

- Consider remove BCS definition for band combination.

Intra-band contiguous CA simplification

In 5G, the maximum aggregated bandwidth of intra-band contiguous CA is smaller than the max bandwidth of single CC for this band. For example, the max channel bandwidth of band n1 is 50MHz, but the max aggregated bandwidth of intra-band CA_n1B is 40MHz, as shown in Table 13:

Table 13 The max aggregated bandwidth of intra-band CA_n1B

Actually, intra-band CA_n1B with 40MHz can be replaced by one single CC with 40MHz. the smaller channel bandwidth aggregation in 5G is to take into account backward compatibility due to 5G was introduced in day one is based on none stand-alone architecture.

6G will be designed based on a stand-alone architecture from day one. To solve the partial spectrum re-farming issue, 6G can work together with 5G in the re-farming band through MRSS operating.

- In NR, there are many intra-bands congruous CA with smaller aggregated bandwidth, which will not be a typical application.

Therefore, RAN4 should optimise the max aggregated bandwidth for intra-band contiguous CA to reduce the amount of band combination. At least, the max aggregated bandwidth of 2 carriers or intra-band contiguous CA should be large than the maximum channel bandwidth of single carrier in this band

- In 6G, strive to avoid using multiple small contiguous CCs aggregation to derive the large CBW, and use one CC with large CBW as much as possible.

Band combination requirements simplification based on band group concept

MSD simplification

RAN4 has specified the MSD look up table for HPUE in Rel-19, although the MSD for PC3 are still defined case by case in 5G.

Actually, in LTE, RAN4 has treated the inter-band CA requirements based on CA classes, as below list:

1UL/2DL

- A1: L-H band combination without harmonic problem

- A2: L-H band combination with harmonic problem

- A3: L-L or H-H band combination without intermodulation problem

- A4: L-L or H-H band combination with intermodulation problem

- A5: Combination except for A1-A4

2UL/2DL

- A1: L-H band combination without harmonic or intermodulation problem

- A2: L-H band combination with harmonic or intermodulation problem

- A3: L-L or H-H band combination without intermodulation problem

- A4: L-L or H-H band combination with intermodulation problem

- A5: Combination except for A1-A4

We can see the CA classes are divided based on frequency range the bands in CA belong to, e.g.,  Low band < 1GHz, 1.7GHz< High band < 2.7GHz in LTE. Here summarized the reasons caused the MSD for CA in LTE, as shown in Table 14.

Table 14 The reasons caused the MSD for CA in LTE

| The reasons for MSD | BC |
| --- | --- |
| Harmonic with 1UL | L-H band |
| Harmonic mixing with 1UL | L-H band |
| Intermodulation with 2UL | L-L band L-H band H-H band |
| Cross band isolation with 1UL | L-L band H-H band |

- LTE has treated the inter-band CA requirements based on CA classes.

In 6G, to avoid define MSD case by case, we can adopt the similar classification with LTE to simplify the MSD for BC, and introduce “band group” concept, for example, Low band group (<1GHz), Mid band group (1-3.5GHz), High band group (3.5-4.9GHz), Super high band group (4.9-8GHz).

- RAN4 can simplify the MSD definition for BC based on look up table and “band group” concept.

- Further study how to define the band groups.

      - ΔTIB,C  and ΔRIB,C simplification

ΔTIB,C  and ΔRIB,C are to treat the IL issues due to additional RF components introduced in UE front -end for inter-band CA. ΔTIB,C  and ΔRIB,C are defined case by case due to different BC have different UE front-end implementation.

Actually, ΔTIB,C  and ΔRIB,C for two bands BC were defined based on CA class originally in LTE, as summarized in Table 15:

Table 15 ΔTIB,C  and ΔRIB,C for two bands based on CA classes

| CA classes in LTE | Bands | Delta TIB | Delta RIB |
| --- | --- | --- | --- |
| A1: L-H without harmonic | L/H | 0.3 | 0 |
| A2: L-H with harmonic | L | 0.3 | 0 |
|  | H | 0.6 | 0.2 |
| A3/A4: L-L or H-H | L/H | 0.5 | 0 |

And RAN4 also agreed the principle that how to define ΔTIB,C  and ΔRIB,C for LTE CA larger than two bands as following:

- When UE support > 1 inter-band CA combos,

- If this band is ≤1GHz, average ΔTIB,C  and ΔRIB,C  for this band in these CA combos. In case.

- there is a harmonic relation between L band UL and H band DL, then the maximum ΔTIB,C  and ΔRIB,C is selected.

- If this band is >1GHz, maximum ΔTIB,C  and ΔRIB,C  for this band in these CA combos.

- ΔTIB,C  and ΔRIB,C were studied based on CA classes in the beginning of LTE.

In 6G, to avoid define ΔTIB,C  and ΔRIB,C case by case, RAN4 can simplify the definition of ΔTIB,C  and ΔRIB,C by combination of the “band group” concept and look up table method.

- RAN4 can simplify the definition of ΔTIB,C  and ΔRIB,C by combination of the “band group” concept and look up table method.

Frequency range between FR1 and FR2-1

WRC-23 had allocated 6425-7125 MHz and 10-10.5 GHz to international mobile communications (IMT) spectrums.

For 6425-7125 MHz is totally overlapping with NR band n104.

For 10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly.

- 6425-7125 MHz is totally overlapping with NR band n104.

- 10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly.

The WRC-27 issue 1.7 aims to find new frequency allocations for future mobile communications (IMT) in the following new frequency bands: 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz.  Based on results of studies, the identification of frequency band(s), to consider for the terrestrial component of IMT:

- 4400-4800 MHz, or parts thereof, in Region 1 and Region 3;

- 7125-8400 MHz, or parts thereof, in Region 2 and Region 3;

- 7125-7250 MHz and 7 750-8 400 MHz, or parts thereof, in Region 1;

- 14.8-15.35 GHz;

Currently, ITU only identified potential new spectrum ranges of 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz.

- Candidate 6G Spectrum in WRC-27 includes 6 425-7 125 MHz, 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz which also need to be studied in RAN4.

For 4400-4800 MHz and 7125-8400 MHz, there is no concern that the UE can adopt omnidirectional antenna.

For 14.8-15.35GHz, one of the main challenges is the size of potential panel. As the researching in TR 38.922, the array antenna for different frequencies were compared as shown in Figure 27 from TR 38.922:

Figure 27 Patch array dimensions vs. frequency

We can observe that

- The size of single element for 15GHz is almost 1.5 times that of 24GHz, correspondingly, the array length of 15GHz band is almost 1.5 times that of 24GHz.

- The array length in the 15GHz band is almost twice as long as that of a single element of 7GHz.

The larger array antenna will increase the difficulty of UE implementation. In addition, the number of antennas and RF chains required for 6G UE will be larger than those used in 5G NR, e.g., new 6G bands of 8GHz and 15GHz need separate antennas from 3.5GHz bands. Therefore, FR1-like antenna is more feasible for 14.8-15.35 GHz, from UE design flexibility perspective.

- Around 7GHz and around 15G can adopt FR1-like antenna and conducted RF requirements.

Conclusion

This paper analyzed the UE RF aspects that need to be considered in the study item and got below initial views and proposals.

RF requirement framework

Observation 1: 	RAN1 has decided to continue use OFDM in 6GR. From RAN4 perspective, most of the 5G NR UE RF requirement frameworks can be reused for 6GR.

Proposal 1: 	Consider take below table 1 as starting point for 6G UE RF discussion.

Table 1 Initial view on UE RF requirements for 6GR

| RF requirements | RAN4 scope | Whether depends on RAN1/2 |
| --- | --- | --- |
| Max Tx power | - Discuss power class framework for single CC and for band combinations in 6G - Discuss default power class for FR1/2/3 - Discuss the tolerance for each possible PC | MIMO capability and UE types discussion in RAN1 may have impact on this topic |
| MPR | - Streamline MPR tables for 1T/2T/MIMO/TxD/different UE types… - Discuss MPR reduction and improvement schemes | Waveform, modulation, and UE types discussion in RAN1 may have impact on this topic |
| AMPR | - Depends on whether new special regulation requirements for the new bands | N |
| Pcmax | - Discuss how to solve “power class fallback” schemes in 5G, e.g., SAR related - Discuss how to simplify the Pcmax definitions, e.g., SRS IL, power boosting - Discuss the approaches to enable UE fully utilize its PA capabilities | RAN1/2 power control and PHR reporting may have impact on this topic |
| Min Tx power | - Most likely can reuse 5G framework | N |
| OFF power | - Most likely can reuse 5G framework | N |
| ON/OFF time mask | - Reuse 5G framework in general ON/OFF time mask - FFS on how to harmonize the switching related time masks | RAN1/2 Tx/Rx switching related schemes may have impact on this topic |
| Power control tolerance | - Most likely can reuse 5G framework | RAN1/2 power control may have impact on this topic |
| Freq error | - Most likely can reuse 5G framework | N |
| EVM | - Discuss EVM requirements for high modulation, and non-uniform constellations - Discuss EVM relaxation schemes for high modulation | RAN1/2 highest modulation order and constellation design may have impact on this topic |
| Carrier leakage | - Most likely can reuse 5G framework | RAN1 physical design may have impact on this topic, e.g., whether the DC will carry data |
| IBE | - Most likely can reuse 5G framework | N |
| Occupied BW | - Most likely can reuse 5G framework for some CBWs - The spectrum utilization discussion in system parameter part will impact this requirement | RAN1 spectrum utilization discussion may impact this topic |
| SEM | - FFS on whether new SEM requirement is needed for the new spectrum, if not can reuse 5G framework | N |
| ASEM | - Depends on whether new special regulation requirements for the new bands | N |
| ACLR | - Most likely can reuse 5G framework for PC3/2/1.5 ACLR - FFS on the new power class demands | RAN1 physical design may have impact on this topic |
| SE | - Most likely can reuse 5G framework - OOB boundary | N |
| Tx intermodulation | - Most likely can reuse 5G framework | N |
| Rx sensitivity | - Discuss the REFSENS for new high bands, e.g., NF and SNR - Review REFSENS for larger CBW and clarify the impact factors. | RAN1 physical design may have impact on this topic, e.g., the SNR demodulation target |
| Max input level | - Most likely can reuse 5G framework | N |
| ACS | - FFS | RAN1 physical design may have impact on this topic |
| IBB | - FFS | N |
| OOB | - FFS | N |
| Narrow band blocking | - FFS | N |
| Spurious response | - FFS | N |
| Rx intermodulation | - FFS | N |

Power class for FR1

Proposal 2: 	Power class definition should make sure UE can fully utilize the PA power capabilities while distinguish different levels of UEs in terms of Tx power.

Observation 2: 	Less than 3dB power step makes the power classes are not well distinguished and has much overlapping power.

Proposal 3: 	Keep 3dB power step among power classes.

Observation 3: 	For UE with multiple Tx PAs, the gap between max UE power capability and power class can reach 1.8dB.

Observation 4: 	The +/-2dB power tolerance was introduced to accommodate massive production variations.

Proposal 4: 	Study how to handle the issue of power gap b/w UE max Tx power capability and power class reporting in the beginning of 6G

Proposal 5: 	Consider reuse the power boosting scheme that allow UE to report the boosted power comparing to the reported power class.

Proposal 6: 	For band combination, not define the total power limitation. Rely on the power capability of each band in the BC (ue-PowerClassPerBandPerBC) to determine the max Tx power.

Observation 5: 	For UEs not supporting TxD, the power capability could be different for different MIMO layer configurations, like PC2 in single port, but PC1.5 for UL MIMO. This power difference cannot be differentiated with 5G NR per band power class reporting.

Proposal 7: 	For single band power class reporting, allow UE to report different power class for different MIMO Layers.

Observation 6: 	For TDD bands, PC2 has been widely supported on the market.

Proposal 8: 	For TDD bands, PC2 is defined as baseline. FFS whether PC2 or PC3 is baseline for FDD bands.

MPR

Observation 7: 	In MRP requirements definitions, waveform, modulation, RB allocation, power class, UE types (antenna isolation), MIMO configuration, and PA/RFIC models need to be considered. Among them some of the factors have some dependent on RAN1.

Proposal 9: 	RAN4 can focus on below aspects to study the 6G MPR.

        - Waveform: use DFT-S-OFDM and CP-OFDM as starting point

        - Modulation: use uniform modulations as starting point

        - Power class: take PC2 as starting point

        - UE types: use handheld UE as starting point, and FFS on how much dB antenna isolation need to be considered for other UE types

        - MIMO: single port as starting point

        - PA model: discuss which kind of PA models to be used in 6G, e.g., fixed biased model, APT model, etc. and also consider the new high bands

        - RFIC impairment: use 5G carrier leakage, IQ image, CIM3 assumptions as starting point and further discuss whether some updates can be made

        - Study how to better align simulation and measurements results.

Proposal 10: 	Study whether the requirement (ACLR/SEM/EVM) relaxation scenarios could be used as default in 6G to make it more useful.

Observation 8: 	The CBW/RB allocation used in 5G MPR calibration is not the worst case in terms of spectrum utilization which makes the calibrated PA has worse linearity and finally large MPR requirements.

Observation 9: 	The PA calibration results and MPR simulation results don’t show obvious difference between the calibration conditions of 20MHz/100RB@0 and 40MHz/full RB.

Proposal 11: 	Continue to use 20MHz/100RB@0 as the MPR PA calibration condition in 6G.

Observation 10: 	To pass the EVM requirements of 256QAM, -34dBc for carrier leakage and IQ image has been used in 5G MPR simulation/measurements. There is no difference in RFIC carrier leakage and IQ image in terms of modulation orders.

Observation 11: 	Carrier leakage/IQ image tighten to -34dBc can improve the MPR for 64QAM around 0.6dB in some allocations, and is necessary for 256QAM.

Proposal 12: 	The carrier leakage and IQ image condition for MPR calibration is tightened from -28dBc to -34dBc in 6G.

Observation 12: 	Due to various PAs on the market and implemented in UE, no single PA model can be universally used in the simulations. This trend will continue in 6G.

Proposal 13: 	The PA models used in 6G MPR simulation is still based on each company’s own choice.

Observation 13: 	APT PA is widely used in UE implementation, and the APT PA will change its working modes when the output power is different which could reduce the power consumption but with the sacrifice of PA linearity, i.e., larger MPR.

Observation 14: 	From simulation results it can be seen that the MPR required for APT PA could be 5dB higher than fixed bias PA in some configurations.

Proposal 14: 	The APT PA models is encouraged to be used in 6G MPR simulation to better align with the real products and measurements.

Proposal 15: 	The 6G MPR simulation calibration conditions is as below table.

|  | 6G |
| --- | --- |
| CBW, scs, RB allocation | 20MHz, 15khz scs, 100RB@0 |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Targe power level to meet ACLR | 1dB MPR |
| carrier leakage | -34dBc |
| IQ image | -34dBc |
| CIM3 | -60dB |

Pcmax

Observation 15: 	Pcmax,h has become the gating factor for UE to transmit higher power especially when UE has multi-Tx PAs.

Observation 16: 	Power boosting or remove the Pcmax,h are two potential approaches to allow UE transmit higher power whenever possible.

Proposal 16: 	Study how to enable UE to fully utilize its PA capabilities. And power boosting approach is preferred comparing to remove the Pcmax,h limits.

Observation 17: 	Duty cycle-based SAR solutions in RAN4 spec is very complex, but it hasn’t been implemented in UE. Only PMPR is applied.

Proposal 17: 	In 6G, only rely on PMPR to solve SAR issue and no other SAR solution is to be defined.

EVM

Observation 18: 	Besides of the uniform modulations already defined in 5G, some enhancements including even higher modulation, and non-uniform modulations are under discussion in RAN1 which will have impact to RAN4 EVM requirements.

Proposal 18: 	RAN4 needs to closely follow the RAN1 discussion on even higher modulations in UL like 1024QAM with AI/non-AI based schemes, and the non-uniform modulations like probabilistic shaping or geometric shaping. And study the impacts in terms of implementation complexity, requirements and performance.

Observation 19: 	Usually, RAN4 non-AI based solution will be done prior to AI based solution.

Observation 20: 	To avoid duplicated discussion, the common part for AI and non-AI based solution will be studied.

Proposal 19: 	Below RAN4 work arrangement to be followed:

•	The common part of non-AI and AI based solution is discussed and aligned.

•	Under the UE RF thread, the non-AI based solution can go prior to AI-based solution to provide a baseline performance.

•	The evaluation assumption including PA non-linearity, UE RF requirements for MPR simulation will be based on the available information on the table. Further change can be applied if new PA model and/or UE RF requirement is introduced.

Observation 21: 	The specific part for AI based solution is the BS AI DPoD algorithm.

Observation 22: 	The common part of AI and non-AI based solution is evaluation method, simulation assumption and the justification metric.

Observation 23: 	One of the evaluations of relaxed EVM is the UE output power increase after EVM relaxation compared to the output power without EVM relaxation.

Observation 24: 	The other evaluation of relaxed EVM is the potential SNR degradation for UE with higher output power and relaxed EVM compared to UE with lower output power and legacy EVM.

Proposal 20: 	To evaluate the UE output power with EVM relaxation.

Proposal 21: 	To evaluate the potential SNR degradation for UE with higher output power and relaxed EVM compared to UE with lower output power and legacy EVM.

Observation 25: 	RAN4 MPR simulation only require to calibrate the PA at specific point.

Proposal 22: 	Same PA calibration point as NR is proposed.

Proposal 23: 	Same RF impairment of NR MPR simulation is proposed.

Proposal 24: 	To further align the link level simulation assumption.

OOB boundary

Observation 26: 	For wideband case, 1.5BN+BU boundary can be applied only when BN>BU in related frequency range.

Observation 27: 	NR FR1 OOB boundary in current Spec does not align with the definition of ITU-R SM.1539.

Proposal 25: 	In 6G, RAN4 need study whether FR1 OOB boundary need to define according to different CBWs in different frequency ranges, separately, as shown in Table 9.

Coherent MIMO

Observation 28: 	Coherent UL MIMO was defined in 5G, but no be supported in UE/network.

Proposal 26: 	Study how to enable coherent UL MIMO to improve UE performance.

REFSENS

Proposal 27: 	The REFSENS calculation equation below can be reused in 6G, and 3dB for diversity gain with 2Rx antennas, 2.5dB implementation margin. For the NF, RAN4 can start with the discussion of targeting bands and analyze its related NF.

- 	REFSENS (dBm) = -174dBm/hz + 10lg(RxBW) + NF + 10lg(SU) – DivGain + SNR + IM

Observation 29: 	The REFSENs relaxation for larger CBW of FDD bands is not very clearly in 5G.

Proposal 28: 	Review REFSENS for larger CBW and clarify the impact factors.

Other Rx requirements

Observation 30: 	ACS and in-band blocking in 5G have different interferer bandwidth for below 2700MHz and above 3300MHz bands.

Proposal 29: 	For ACS and in-band blocking, RAN4 could study whether can adopt the same the bandwidth of interferer between the bands below 2700MHz and above 3300MHz including 6G new bands.

Proposal 30: 	RAN4 should consider to unify the parameters for out of band blocking, spurious response and wide band intermodulation between the bands below 2700MHz and above 3300MHz including 6G new bands.

Proposal 31: 	Discuss whether narrow band blocking can be removed from 6G.

Requirements for different UE types

Observation 31: 	Device types discussed in RAN1 and RAN plenary mainly focus on the mandatory features for each device type which is different from RAN4 requirement definition. RAN1 may define many device types but in RAN4 they may share same RF requirements.

Observation 32: 	RAN4 has defined many UE types but not be well organized in the 5G spec. This has caused many confusions in the industry in terms of which requirement to be applied to which UE types.

Observation 33: 	For most of UE types defined in RAN4, the antenna numbers and antenna isolations were the main concern in 5G. Antenna numbers mainly about the relaxation from mandatory 4Rx to 2Rx for some specific UE type, and antenna isolation is mainly for the MRP requirement definition.

Proposal 32: 	Consider using antenna isolations as the discriminator to differentiate applicable MPR requirements only and no bounding of UE types. Any UE that meets the antenna isolation conditions can apply the corresponding requirements.

UE capability signaling simplification

Observation 34: 	In the testing/certification, UE declaration approach can be applied to make it clear which requirements should be tested. There is no need to define UE capabilities just for the purpose of differentiating the applicable requirements for a UE.

Proposal 33: 	In 6G, the testing purpose singling should be avoided and when new signaling is introduced it shall be justified that it is useful for BS scheduling.

Proposal 34: 	In 6G, avoid defining UE capabilities with small granularity when it is unnecessary.

FR2 UE RF

Proposal 35: 	In 6G, reuse 5G FR2 requirements as much as possible and focus on 1CC in the beginning.

SCMC

Observation 35: 	To resolve the deployment scenarios, Tx switching, SDL switching and intra-band NC CA sharing one RF chain were enhanced under NR CA framework, however, they were all configured all with one cell and one CC.

Observation 36: 	SCMC concept is discussing in RAN1 mainly for the purpose of solving CA configuration complexity and redundant control configurations.

Observation 37: 	From UE perspective, SCMC and CA can share one set of RF implementation, there is no significant difference between them.

Proposal 36: 	Study whether it is possible to define only one set of requirements for CA and SCMC when introduced in RAN1 considering CA and SCMC can share the same RF implementation.

Observation 38: 	The complexity of supporting SCMC + CA simultaneously is high.

Proposal 37: 	In 6G day one, mixed combination of SCMC + CA is not supported.

UL/DL decouple

Observation 39: 	UL/DL decouple is discussed in RAN1 for the purpose of cell-edge performance and UL coverage.

Observation 40: 	UL/DL decouple have not any impact on UE RF implementation when band A and Band B adopt separate antenna in UE RF implementation.

Observation 41: 	New RF front-end component will increase the complexity of UE implementation and new band number cause more band combinations and bring more workload.

Proposal 38: 	Avoid to define a new band for UL/DL decouple pair.

Observation 42: 	A UE supports inter-band CA (Band A + Band B), will support the recouple pair of Band A UL and Band B DL by CA enhancement.

Proposal 39: 	To enable more efficient Cell/UE management, UL/DL decouple based on non-CA mode can be studied.

Observation 43: 	New RF front-end component will increase the complexity of UE implementation although can save one set of RF chain band BB.

Observation 44: 	If both bands are within same band group (e.g., both are low bands), then for shared antenna UEs, this will need a new quadplexes which will increase much complexity in implementation.

Proposal 40: 	The recouple pair of Band A UL and Band B DL can work simultaneously or by switching between Band A UL and Band B DL.

Carrier-switching

Observation 45: 	Carrier switching is under discussion in RAN1 which is targeted to simplify from RF and baseband especially for the band combinations which cannot supported simultaneously like low band FDD + FDD.

Proposal 41: 	Study the RF impacts for carrier/band switching in terms of considering the switched carriers as one cell or two cells.

The relationship among the aggregated schemes

Observation 46: 	All these new candidates aggregated technologies have similar purpose with enhanced CA in NR, except the different of Cell/UE management.

Proposal 42: 	For one band combination, which aggregated technologies will be adopt need further down selection, and UE implementation complexity need to be well considered.

BCS simplification

Observation 47: 	BCS4 is introduced in NR in Release 17. It can be supported in 6G day one.

Proposal 43: 	Consider remove BCS definition for band combination.

Intra-band contiguous CA simplification

Observation 48: 	In NR, there are many intra-bands congruous CA with smaller aggregated bandwidth, which will not be a typical application.

Proposal 44: 	In 6G, strive to avoid using multiple small contiguous CCs aggregation to derive the large CBW, and use one CC with large CBW as much as possible.

MSD simplification

Observation 49: 	LTE has treated the inter-band CA requirements based on CA classes.

Proposal 45: 	RAN4 can simplify the MSD definition for BC based on look up table and “band group” concept.

Proposal 46: 	Further study how to define the band groups.

ΔTIB,C  and ΔRIB,C simplification

Observation 50: 	ΔTIB,C  and ΔRIB,C were studied based on CA classes in the beginning of LTE.

Proposal 47: 	RAN4 can simplify the definition of ΔTIB,C  and ΔRIB,C by combination of the “band group” concept and look up table method.

Frequency range between FR1 and FR2-1

Observation 51: 	6425-7125 MHz is totally overlapping with NR band n104.

Observation 52: 	10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly.

Observation 53: 	Candidate 6G Spectrum in WRC-27 includes 6 425-7 125 MHz, 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz which also need to be studied in RAN4.

Proposal 48: 	Around 7GHz and around 15G can adopt FR1-like antenna and conducted RF requirements.

Reference

R4-2514642 WF on [116bis][102] 6G general RF and UE RF, Qualcomm, RAN4 #116bis 2025-10

R4-2311151 PA Calibration using 40MHz CBW waveforms for FR1, Skyworks, #108 2023-08

R4-2318781 Reference waveform for PA calibration, Qualcomm, #109 2023-11

R4-2520006 LS on PA models in 6GR waveform evaluations, R1->R4, R4#117 2025-11

R4-164542 Response LS on realistic power amplifier model for NR waveform evaluation, R4->R1, R4#79, 2016-05