3GPP TSG RAN WG4 Meeting #117	R4-2521537

Dallas, USA., 17th – 21st, Nov. 2025

# Agenda item:	8.8.2

# Source: 	ZTE Corporation, Sanechips

# Title: 	Views on AI study in 6G RAN4-driven use cases

# Document for:	Approval

# 1 Introduction

6G Radio Study Item has been approved in the RAN plenary #108 meeting [1]. The RAN4 discussion has already been kicked off from RAN4 #116bis meeting. This contribution addresses to select use case by focusing on  RF/Demod/RRM domains, prioritizing scenarios that demonstrate significant performance gains.

# 2 Discussion

During the last meeting, RAN4 commenced the discussion on RAN4-driven use cases selection, focusing on RF, Demod and RRM domains. This initiative aims to identify and prioritize the most impactful AI use cases to enhance system performance and efficiency. The following sections provide a detailed clarification on RAN4 driven use cases selection.

## 2.1 AI use cases in RF aspect

For AI applications in RF, AI-nonlinearity compensation has gained the most interest from companies. And the sub use case to be studied per agreement are listed below:

Agreement: - The scope of potential RAN4-led AI use case, AI-DPD/DPoD, will be further discussed in the next meeting. The candidate sub-use cases include - UL: - UE: AI-DPD - NW: AI-DPoD - DL: - NW: AI-DPD - UE: AI-DPoD To facilitate the 6G AI scope discussion, the interested companies are encouraged to timely provide more analysis, including the performance benefit, spec impact, the assumption behind their analysis, on the specific sub-use case(s).

In last meeting, nearly a full set of potential sub use case are collected for AI-NC, including DPD at Tx side and DPoD at Rx side for DL and UL respectively. DPD has been applied in 5G system for a long time, which is mainly beneficial for ACLR, SEM and EVM requirements improvement, but it is more an implementation issue rather than a specification issue, and RAN4 has never discussed the DPD related procedure before. Although AI based DPD provides us a more promising way to promote the power efficiency than legacy DPD algorithm, RAN4 should deproritize the AI DPD discussion unless the spec impact is justified.

For DPoD, which has been illustrated for significant signal quality improvement at Rx side, and the signal quality improvement can promote the relax of EVM requirement at Tx side conversely. As we know, EVM requirement  is usually the bottleneck for high order modulation. So we are more preferred to study DPoD in 6G day 1further.

Proposal 1: RAN4 to prioritize the DPoD study for AI-nonlinearity compensation use case.

Moreover, the evaluation methods are also discussed in last meeting, which can facilitate use case discussion. For DPoD evaluation, we need to determine the PA model firstly, two approaches can be considered for PA model：

-Alternative 1: To determine a common PA model for further evaluation

If a variety of PA models are provided, it seems difficult to select a common PA model for evaluation in the limited TU budget, for alternative1, we suggest to use the GMP model define in TR38.803.

-Alternative 2: multiple PA models can be used for further evaluation

For alternative2, each company can provided own PA model and use this model for further AI NC study and evaluation, as long as the relaxed EVM requirements can be derived reasonably. It is encouraged to report the PA model when the evaluation is conducted.

Proposal 2: For DPoD evaluation, two alternatives can be considered for PA model derivation:

-Alternative 1: To determine a common PA model for further evaluation

-Alternative 2: multiple PA models can be used for further evaluation as long as the relaxed EVM requirements can be derived reasonably

we select the Generalized Memory Polynomial PA model defined in TR38.803, the first one called 2.1 GHz PA model with memory (based on measurements of commercially available GaAs PA) is used as baseline, detail parameters can be seen in the appendix part.

In the following part, we provide some preliminary evaluation results for AI DPoD based on the GMP GaAs non-liner PA model.

Figure 1 illustration of AI-DPoD

In figure 1, we provided a SNR-bler curve of no PA case (or so-called linear PA) for reference. The bandwidth is set as 10M Hz and 25PRB/15k SCS are allocated for the transmitted signal. As can been see from the figure, to meet the Tx EVM requirement of 3.5%@256QAM, 2.5dB power back off is needed. If power back off is set as 0, the Tx EVM increase as 4.04% > 3.5%.

Taking SNR=16 as reference, the bler for BO=2.5 is 0.1101, then our goal is to seen whether the usage of AI DPoD at Rx side when Tx BO=0(EVM=4.04%) can improve the bler(0.1224,red line at SNR=16) to be not worse than the bler(0.1101,green line at SNR=16) of BO=2.5（EVM = 3.49%). As a result, it is surprised to find out that the bler is improved to 0.0917 (magenta point at SNR=16) after AI DPoD, that means the Tx EVM can be relaxed to 4.04% in our simulation assumption. Of course the EVM relaxation need more evaluations to apply most simulation assumptions such as different carrier frequency, bandwidth for a given MCS, this work can be left for further study. In table 1 we provide detail information for AI DPoD evaluation.

Table 1 AI DPoD information

| AI model input | Input in training | The estimated data signal time domain sample with 1st, 3rd , 5rd terms |
| --- | --- | --- |
|  | Input in inference | The estimated data signal time domain sample |
| AI model output | Label in training (if applicable) | original/known data signal |
|  | Output in inference | data signal with non-linear distortion compensated |
| Assumption on training | Training type | offline training |
|  | Label construction (if applicable) |  |
| model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | N/A |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameters; Value. Carrier frequency; 2.1 GHz. # of TX layers/users; 1. # of RX antennas; 2. Waveform; NR based. # of PRB; 25. # of symbol per slot; 12. # of DMRS per slot; 2. DMRS subcarrier position; Odd index (0 for even index). FFT size; 1024. CP length; 72. Modulation; 256 QAM. Coding rate; LDPC 0.7363. Transport Block size; 28800for 1Tx/256 QAM. Channel model; CDL-A 100ns. Subcarrier spacing; 15 kHz. Evaluation metric; MPR/EVM/BLER. Benchmark; Conventional receiver (without non-linearity compensation). |
| Evaluation KPI |  | BLER, MPR, EVM, throughput |
| Evaluation benchmark |  | Legacy receiver without DPoD |
| preliminary evaluation results |  | Tx EVM can be relaxed to 4.04% |
| High level potential specification impact |  | UE capability report on supported MPR relaxing RAN4 measurement requirement on MPR delta |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Complexity: FCN is used: Number of input neurons is 30 Number of output neurons is 2 Number of hidden layers is 4 Number of neurons in each hidden layer is 64 FLOPs: 0.59G, Params: 0.01M Performance benefit: AI-NC provides 2.5dB MPR improvement when using 256QAM for the given SNR value(SNR=16). |

## 2.2 AI use cases in Demod aspect

In the last meeting, RAN4 discussed the potential AI driven use cases from RAN4 demodulation perspective.             As captured in the WF [3], about three cases mentioned based on the company’s contribution, such as AI-SRS-assisted channel reconstruction, AI based SRS IL compensation, and AI-enable PRACH. In the following, we want to share our views on the above three uses cases from RAN4 demodulation perspective.

As is well known, the RAN4 demodulation work is based on the reasonability of baseband processing, and involves defining test cases to verify the functionality of features which come from RAN1, RAN2 or RAN4-driven topics. Topics driven by RAN4 demodulation include receiver definition, interference cancellation and spatial channel modelling in the legacy framework, at least. From this point of view, potential AI use cases driven by RAN4 can be discussed in the definition of the receiver and the performance could be verified. In the last meeting, some potential use cases are mentioned by companies, such as AI based SRS IL compensation and SRS-assist channel reconstruction, AI-enable PRACH receiver. In the following, we will give our initial opinions on those potential use cases.

AI based SRS IL compensation and SRS-assist channel reconstruction

As mentioned by companies, SRS IL power imbalance is caused by differences in power at each antenna port in UE side. Insertion loss due to antenna layout significantly affects the power at each antenna ports. When using SRS for channel estimation, the network side’s understanding of the channel will be biased, leading to a decrease in sub sequent demodulation performance. More importantly, incorrect selection of DL precoding can lead to beam direction errors or energy dispersion, reducing coverage and signal quality. Due to the impact of SRS power imbalance, RAN4 defines requirements in RF to limit the effect the SRS. It also points that the UE can perform pre-compensation to balance the power, but this depends on the implementation.

As is well known, the most fundamental and important value of SRS lies in its physical principle, in a TDD system, the uplink and downlink channels operate on the same frequency and are reciprocal. This means that uplink channel characteristics can reflect downlink channel characteristics. By receiving SRS data from UE side, the BS can accurately learn the state of the DL channel. Currently, with the increasing number of antennas on the network side, the overhead of CSI-RS has also increased dramatically, resulting in increased DL overhead. However, by leveraging reciprocity through SRS, the entire channel state information can be obtained by combing a small number of CSI-RS and SRS. There is no doubt that SRS will play an important role in 6G.

To ensure SRS performance is unaffected by power imbalance and to integrate it with CSI-RS, various companies have proposed AI based approaches, the power imbalance is considered in channel reconstruction. This is because accurate channel estimation is a prerequisite for reconstruction.

Here, we discuss the SRS-assist channel reconstruction, the motivation of this is to reduce the CSI-RS overhead, especially when deploying antennas on a larger scale. We expect that by using fewer CSI-RS and SRS, we can infer the entire channel information, thereby providing a reference for DL beamforming. Less CSI-RS overhead result in greater beamforming gain. Therefore, if we carefully consider the benefits of AI-SRS-assisted channel reconstruction, to reduce CSI-RS overhead. From this point of view, we believe that how to reduce the CSI-RS is very important, and the detailed assumption shall be studied, such as frequency domain density, number of symbols in the time domain. Furthermore, designing AI algorithm that can be obtain comprehensive channel state information through CSI and SRS is also crucial.

Observation 1: The motivation of SRS-assisted channel reconstruction is to reduce the CSI-RS overhead, especially when deploying antennas on the larger scale.

Observation 2: The SRS-IL power imbalance shall be included in the SRS-AI assist channel reconstruction. Accurate channel estimation is a prerequisite for this reconstruction.

Proposal 3: Propose to postpone the related AI SRS assist channel reconstruction discussion in RAN4, to be led by RAN1.

AI-enable PRACH receiver

Another issue is the AI-enable PRACH receiver to support multi-user collision detection when using same preamble. Currently, to avoid this collision, Msg 4 based contention resolution is a potential feasibility approach. The UE determines whether random access is successful by judging the unique identifier. This is the contention resolution in 5G system.

In the last meeting, companies mentioned that AI- based PRACH receiver, early contention resolution can be feasible. Based on company’s study, AI-based PRACH receiver can estimate the number of UEs by using same preamble. And then, BS can configure a RAR associated with the RA-RNTI corresponds to the preamble. The RAR includes multiple PUSCH grants, the number of PUSCH grants is correspond to the number of UEs, which using the same preamble in same PRACH resource.

However, based on our understanding, the above procedure is related to the RACH access of RAN1, which has a significant difference with RAN 1. Firstly, as mentioned in the above, multiple grants can be transmitted in Msg 2 correspond to the multiple UEs based on BS RACH detection. In this procedure, BS configures multiple UL grants, and UE receives Msg 2. A potential issue is how the UE knows its corresponding UL grant. BS transmits multiple UL grants to UE, but the UE does not know which grant corresponds to which UE. Secondly, currently each preamble in the RAR message corresponds to only one UL grant. If the preambles sent by the UE in Msg 1 conflict, the BS will allocate new resources to the winning UE through a contention resolution procedure (Msg 4), rather than directly providing multiple grants in Msg 2. Thirdly, based on the above analysis, if the collision can be resolved in Msg 2, the RACH access procedure may need to be discussed in RAN1 and RAN2.

Observation 3: Some potential technical issues need to be discussed in RAN1, and the RACH access procedure may need to be discussed in RAN1 and RAN2.

Proposal 4: Propose to postpone the related AI based RACH receiver discussion in RAN4, to be led by RAN1.

In legacy requirements definition, RAN4 only led very limited test cases such as receiver enhancement for UE and BS side, as well as interference cancellation scenarios in 5G NR system. These studies lay an important foundation for receiver architecture design (MMSE/MMSE-IRC, R-ML), performance baselines, technology development. In 5G NR, MMSE-IRC receiver is a mandatory feature, and R-ML receiver is an optional feature in UE device side. However, due to factors such as high chip implementation costs, insufficient network-assisted signaling support, and limited standardization, R-ML receivers have not yet been widely commercialized.

In recent years, with the rapid development of AI technology, the integration of AI and communication technology has continued to deepen. For example, in Rel-19, the AI-based CSI compression and prediction use cases was introduced for the first time, laying a good foundation for the standardization of AI. With the development of 6G technology, more and more AI use cases are being studied at the physical layer (such as coding, modulation, channel estimation) and network layer (beam management, energy saving). It is foreseeable that AI-enabled communication technology will become an important trend.

As the core module of wireless communication system, receiver technology has a direct impact on the communication quality and efficiency of the entire link. However, within the theoretical constraints of Shannon’s theorem, the performance improvements of traditional linear receivers and non-linear are gradually reaching their limits. This is especially true in complex channel environments, multi-user interference, and high modulation order scenarios, where traditional receiver architectures struggle to significantly improve spectral efficiency of reduce BLER. In this backdrop, exploring receiver technology based on AI is becoming a potential direction to break through the performance boundaries of classical theory.

Combining existing research literature, AI receiver can be roughly divided into two categories, data driven methods and model driven methods. As shown in the following figures, the model driven approach, based on the existing technology of the wireless communication system, does not change the model structure of the wireless communication system, but uses a deep learning network to replace a certain module or train related parameters to improve the performance of a certain module. The data driven approach regards multiple functional modules of the wireless communication system as an unknown and black box, replaces them with an AI network, and then relies on a large amount of training data input to output training. A large number of simulation and experimental studies have shown that both methods have been proven to be effective in different scenarios, and each has unique advantages and application potential.

From the perspective of RAN4, AI based receiver is undoubtedly one of the current potential research directions in the current 6G phase. We believe that conducting systematic research on AI receiver has significant value, based on the following considerations.

Figure 1: Model driven AI receiver

- Performance improvement and bottleneck breakthroughs

Traditional receivers face performance bottlenecks in complex environments (such as high frequency bands, high mobility, and multi-user interference). AI approaches demonstrate superior non-linear processing and environmental adaptability, potential further improving spectral efficiency, and system capacity, supporting the even more extreme performance targets of 6G.

- Interpretability and Reliability

Model-driven AI is built on mature communication theories (such as OFDM, MIMO, and detection theory). Its decision-making process is supported by clear mathematical and physical meanings, rather than being a pure "black box." This makes its performance easier to analyze, verify, and trust.

- Compatibility of model driven development with existing frameworks

Model driven based AI receiver can be embedded in existing communication process without changing the basic physical layers structure, making it easier to evolve during standardization process. Data driven based AI receiver is a black box models whose inputs and outputs may be completely different from those defined in existing standards. They rely heavily on large amounts of data training rather than explicit mathematical models, which typically require the transmitter and receiver to be jointly optimized as a single neural network, fundamentally changing the traditional modular architecture, e.g., channel coding, modulation schemes, channel estimation, detection, decoding, etc. This bring a revolutionary impact on existing baseband chips design, also resulting a significant impact on current signal processing and a longer and more challenging road to standardization.

What’s more, from performance of view, results convergence and functional testability are the important aspects for RAN4. In Rel-19, RAN4 discussed the testability issues of two side model for CSI compression, and achieved a conclusion that the functionality testability of two side model can be tested. On the other hand, the testability of one side model for CSI prediction is also verified. And regarding the testability of AI receiver, which is more similar as one side model. Therefore, the testability issues of AI receiver are undoubtedly non-existent.

Another question is whether the results converge. As we discussed before, for RAN4 requirements definition, convergence is the most important part. In the above discussion, we mentioned two different approaches to the current architecture of AI receivers. Based on current our view, we believe that the model driven approach is more suitable for RAN4 research or standard definition. The data driven approach changes too many classical communication module designs, making the results highly uncertain, which is also a huge challenge for convergence. Therefore, in general, we believe that the model driven AI architecture is more suitable for current standard promotion and research. It also makes it easier to evaluate, simulation and standardize technical in3GPP.

Observation 4: RAN4 defined the requirements of linear receivers and non-linear receiver in 5G.

Observation 5: Exploring receiver technology based on AI is becoming a potential direction to break through the performance boundaries of classical theory.

Observation 6: The data driven approach regards multiple functional modules of the wireless communication system as an unknown and black box, replaces them with an AI network, and then relies on a large amount of training data input to output training.

Observation 7: The model driven approach, based on the existing technology of the wireless communication system, does not change the model structure of the wireless communication system, but uses a deep learning network to replace a certain module or train related parameters to improve the performance of a certain module.

Proposal 5: Propose to discuss whether RAN4 can lead the discussion on AI receiver in RAN4, such as legacy release discussed MMSE-IRC and advanced receivers.

## 2.3 AI use cases in RRM aspect

In the last meeting, we had an intial discussion on AI use cases from RRM perspective in RAN4, and there was no down selection just some additional principles were agreed compared to general part [3]. In this meeting, we should give some priority among different use cases proposed by different companies according to the principle for AI/ML use cases selection.

As the overview of AI/ML overall the whole 5G, around the RRM measurement and event prediction, the RAN4 aspects such as measurement delay, prediction accuracy, reporting delay, reporting accuracy and measurement gaps were discussed in the RAN2-led WID. Given they would be ongoing development in NR Release 20, to prevent duplication of efforts, we propose to exclude any RAN4-related issues that have been identified/discussed in 5G-A from the scope of 6G AI topics led by RAN4. In addotion, some use cases have been identified in 5G-A but were not included in work scope due to limited time and large workload, such as RLF, spatial domain, non-collocated scenario and L3 beam level prediction. Even if these use cases not within the work scope of 5G, it is clear that they would be studied in RAN2 for 6G, and these use cases are the extension of 5GA. The issues that should be discussed with 5G as a whole, such as colocated and non-colocated scenarios, temproal/frequency/spatial domain, as well as L3 cell level and L3 beam level prediction.

- Issue 6-1c: Additional principle/method for AI-RRM use case selection Agreement: - Additional principle/method for AI-RRM use case selection - Whether the proposed AI-RRM use case should be driven by RAN4: - Existing 5G-A use cases included in RAN2 and RAN1 WIs should be precluded in RAN4 6G study. - Coordination with RAN1 and RAN2 when potential AI-RRM use cases are agreed in RAN4 to avoid overlapping scope.

Observation 8: Some use cases have been identified in 5G-A but were not included in work scope due to limited time and large workload, such as RLF, spatial domain, L3 beam level prediction and so on, which are better to be discussed with 5G as a whole.

Proposal 6: RRM relevant use cases that have been identified in 5G-A but not included in work scope due to limited time and large workload should be deprioritized in RAN4’s discussion.

From RAN4 RRM perspective, we intend to develop new use cases of AI/ML application starting from some pain points exposed in actual practice deployment. We classify all the potential use cases into several types:

Type 1: Whether and how to apply AI/ML to reduce measurement resource overhead

Type 2: Whether and how to apply AI/ML to reduce measurement/processing delay

Type 3: Whether and how to apply AI/ML to reduce the interruption/measurement gap

For Type 1, which facilitates both NW power saving and UE power saving since less measurement is expected, at the meanwhile, with the reduction of measurement resource overhead, the saved time/frequency domain resource could be used for traffic transmission, so the throughput gain is foreseen.

For Type 2, it is beneficial to the UE power saving since less measurement operation or processing is expected. Furthermore, the more efficient mobility procedure is supposed to improve the user experience. Take the RF re-tuning or RF warm up as example, if UE is relieved from performing frequency RF re-tuning/RF warm up, the UE power consumption and processing delay are saved.

For Type 3, obviously it could largely improve the system performance through less interruption and less measurement gap. In current practical deployment, the measurement gap/interruption overhead impacts the throughput performance more severe than expected especially in async network. If such measurement gap/interruption overhead could be reduced by AI/ML based prediction, significant throughput improvement is foreseen.

For example, in the non-AI way of measurement procedure, it takes several samples to complete the measurement for different time of the same cell or same time of different cells in order to obtain the measurement results. However, with the help of AI tools, we do not need to actually measure the results at that time and can directly complete the prediction through historical/other measurement results. For the prediction of different cells, measurements can be completed simultaneously on the same frequency, but for measurements on different frequencies, especially FR2 and with gap, measuring an additional layer means that the measurement delay is at least doubled. However, with the help of AI, there is no need to measure cells on another frequency layer, reducing the measurement delay and the need for gap. Additionally, if actual measurements are not required, the measurement restrication previously considered in the specification may make significant progress. The RS used for measurement is saved for transmission or other purposes, solving the scheduling restrication of NW and the measurement restrication of UE, improving the network throughput. Therefore, it is inevitable for measurement prediction to become an AI use case. So for the use case of RAN4 led, we also need to consider the improvement in overhead, latency and/or gap.

Proposal 7: Study and develop the potential AI/ML use cases for RRM from the following perspectives:

- Type 1: Whether and how to apply AI/ML to reduce measurement resource overhead

- Type 2: Whether and how to apply AI/ML to reduce measurement/processing delay

- Type 3: Whether and how to apply AI/ML to reduce the interruption/measurement gap

In this meeting, the scope of potential RAN4-led AI-RRM use cases will be futher discussed. And the follow table is provided to summarize AI-RRM proposals in the pervious meeting can be use as baseline [3].

| Index | use cases | Model location (Training method) | Proposed by companies |
| --- | --- | --- | --- |
|  | L1 beam measurement: Time domain (intra-cell) Time and spatial domain (intra-cell) Freq. and spatial domain (inter-cell) Spatial domain (inter-cell) Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c), Nokia ((d, e), as RAN1-led study), Apple (c, d, e) |
|  | L3 beam measurement: Time domain (intra-cell) Time and spatial domain (intra-cell) Freq. and spatial domain (inter-cell) Time and freq. domain (inter-cell) Freq. domain prediction for non-collocated cells (inter-cell) Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) | OPPO (a, b, c, e), Apple (c,d, e), CMCC (e), ZTE (e), Samsung (e, f), Nokia (c, d, e) |
|  | L3 cell measurement: Time domain (intra-cell) Freq. domain (inter-cell) Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c) Apple (c), Nokia (c) |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) | Apple, vivo, Samsung |
| 5a (5+6 combined) | Dynamic Adaptation of Measurement Procedure | UE sided model | MediaTek, Qualcomm, Apple |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Apple (L1 for scheduling restriction and delay reductions), vivo Qualcomm, Apple, CMCC, Xiaomi |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model | OPPO, vivo (AI for RLM) |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model | OPPO |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model | vivo |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model | ZTE |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model | ZTE |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model | CATT |

Use cases closely related to 5G

For use cases 1-3, we should preclude the cases which are existing 5G-A use cases included in RAN2 and RAN1 WIs. In our view, use cases (a)(b) in 1), use cases (a)(b) in 3) have been discussed in Rel-19 and Rel-20, so we don’t need to do the duplicated study on these use cases. Meanwhile, for L3 beam measurement, even not considered in 5GA, but it has been identified and it will be led by RAN2 with high possibility. To avoid duplication, it is better to consider these issues as second proprity. In addition, there are other cases that have already been identified in 5GA, such as RLF, spatial domain, non-collocated scenario and L3 beam level prediction. For the sake of completeness in the discussion, we prefer that these issues be led by other working groups, with RAN4 providing corresponding assistance.

For use case 4, Multi-Domain L3 level Prediction are introduced for Measurement-Gap Reduction. Compared to 5G, single domain such as temproal/frequency is considered for L3 level prediction, multi-domain prediction will bring more benefits by extending the scenario. We can use the measurement results of cell1 at t1 to predict the results of cell2 at t2, where t2>t1. However, from the perspective of feasibility and testability, is there any difference between the research on this case and previous 5G cases? We are not sure if we need to conduct independent research specifically on this case. Meanwhile, RAN4 should to figure out the single dimension prediction first (e.g., spatial domain, or frequency domain) and then to further consider the multiple dimension (e.g., spatial + frequency domain).

Proposal 8: For Multi-Domain L3 level Prediction, it should be deprioritized and considered after significant progress is achieved for single-domain L3 level prediction to avoid redundant discussion.

For use case 9, If I understand correctly, this is to apply AI to reduce L1 measurement and procedure, because our focus in 5G is on L3 measurement. From our perspective, this work may be more triggered by RAN2. Our discussion on LTM in 5G is led by RAN2, and AI mobility is also led by RAN2. Based on this, if AI is to be used in LTM procedure, RAN2 may be more reasonable to lead it. Meanwhile, AI used for LTM is also a key use case for RAN2 in 6G AI.

For use case 10, one of the expected benefits can be delay reduction for the related RRM procedure. Similar to the previous case where we wanted to extend AI to L1 measurement, here we aim to extend AI to CA scenario in order to reduce measurement delay. I am curious if this case can be classified into the first three cases, all of which use AI to reduce measurements without actually measuring the target cell, and obtain prediction results through historical information or measurement results from other cell(s). Different from 5G is how to update requirements, but from the perspective of measurement prediction, there seems to be no difference.

Observation 9: From the perspective of measurement prediction, there maybe no difference between AI/ML based SCell related enhancement by measurement prediction and AI based L3 cell-level measurement prediction. Both use AI to reduce measurements without actually measuring the target cell, and obtain prediction results through historical information or measurement results of other cell(s).

Use cases need further discussion

For use cases 5a and 11, the dynamic adaptation of measurement procedure or AI-based prediction for enhanced s-measure mechanism, UE will change it own behavior based on UE location or other information lacking in NW, which is different from NW configuration. We have some concerns on these use cases. As is well known, NW usually has more comprehensive information than UE because the network has a global view and centralized control capability, while the UE's view is usually limited to itself and the nodes directly communicating with it. From the perspective of NW, based on global information, provide UE related configurations and instructions. UE should complete corresponding tasks according to NW's instructions. If UE does not follow the instructions of NW, it may have an impact on the network performance.

If the UE does not follow the NW configuration, will it be unable to detect the deterioration of neighboring cell signal quality in a timely manner, resulting in the inability to report measurement reports in a timely manner to trigger handover. Alternatively, if the UE performs measurements according to its own wishes, it may cause the network to be unable to correctly determine the load situation based on the UE's measurement reports, resulting in the network being unable to make correct energy-saving decisions.

Observation 10: NW usually has more comprehensive information than UE because the network has a global view and centralized control capability, while the UE's view is usually limited to itself and the nodes directly communicating with it.

For use case 7a, spatial domain RX beam sweeping reduction (L1/L3) looks more like an extension of AI/ML beam management. In 5G, relevant content about Tx beam prediction was discussed, and it is hoped that this discussion can also be applied to Rx beam in 6G to reduce the number of Rx beam sweeping and thus reduce measurement time. This is very beneficial for RAN4, but we found that a similar issue was being discussed by RAN1 also includes predictions about Tx-Rx beam pair prediction [4], so conflicts may arise here. We may need to negotiate with RAN1 which work group should lead this use case.

Table E-1 AI/ML for beam management and extension

| Sub-use case | Sub-case A: Inter-Cell/TRP beam prediction and management | Sub-Case B: Cross frequency beam prediction | Sub-Case C: Tx-Rx beam pair prediction | Sub-Case D: Beam prediction for initial access | Sub-Case E: DL Tx beam prediction for spatial and/or temporal domain with additional local UE information | Sub-Case F: reinforcement learning-based approach beam selection |
| --- | --- | --- | --- | --- | --- | --- |

Observation 11: The sub-use cases of Inter-Cell/TRP beam prediction and management and Tx-Rx beam pair prediction are discussing in RAN1.

Proposal 9: RAN4 should discuss how to coordinate with RAN1 on Tx-Rx beam pair prediction.

Use cases from our side

Regarding the use cases 12 and 13 that we raised in the last meeting, we would like to provide some clarification here. Firstly, our principle is not to conflict with RAN1/RAN2, so we focus on the specific parts of RAN4, such as AGC and DL Sync. Secondly, in the current measurement procedure, we need several samples to complete AGC and DL Sync. If AI can be used to predict these values, it can reduce the overhead of reference signals and minimize measurement delay.

Before inter-frequency cell search, UE firstly needs to determine the general signal levels on the cell, in order to set the LNA to an operating point where saturation of the received signal is avoided, i.e. perform the SSB or CSI-RS based AGC to derive the AGC factor. Technically as long as the RF re-tuning happens, UE needs to perform AGC operation unless UE maintains the previous AGC factor for the target cell/frequency and it is still valid. Typically two RS samples are supposed to consume for AGC operation.

Observation 12: For any inter-frequency target cell/frequency, before the cell search, UE needs to perform AGC operation unless UE maintains the previous AGC factor for the target cell/frequency and it is still valid. The demand of AGC operation is universal as long as RF re-tuning happens.

The output of AGC operation is influenced by multiple factors, such as the Transmission power of NW side at the target cell/frequency, the pathloss of the signal propagation, the setting of LNA at UE side. If partial or all of these factors could be inferred through temporal domain or frequency domain prediction, both the RS overhead and UE operation could be saved. Therefore, RAN4 could study the AI/ML based AGC prediction in 6GR.

During the cell search or cell identification, before UE performing measurement to derive RSRP/RSRQ/SINR/RSSI, UE needs to perform PSS/SSS detection to track the coarse T/F information, to obtain the full time information, additional one sample of measurement is needed and the PBCH decoding is necessary. We want to explore whether the DL sync for an intra-frequency or inter-frequency cell/frequency layer could be derived through temporal/frequency domain prediction provided some side condition meet, i.e. lo-located deployment.

Proposal 10: Study the feasibility of AI/ML based AGC, DL sync in 6GR with consideration of some side conditions.

For use case 14, AI abstraction of the wireless environment to facilitate data scheduling and network power saving, which can be understood as applying the logic of the AI model. From our perspective, this may not be an effective use case.

For clarity, we have provided the following table as a reference based on the guidance from last meeting.

|  |  | Sub-use case 12: AI/ML based AGC through temporal/frequency domain prediction | Sub-use case 13: AI/ML based DL sync through temporal/frequency domain prediction |
| --- | --- | --- | --- |
| AI model input | Input in training | AGC values in historical measurements (t1) | Location of UE and two cells |
|  | Input in inference | AGC values in historical measurements (t2, where t2>t1) | Received Time difference |
| AI model output | Label in training (if applicable) | / | / |
|  | Output in inference | Corrected AGC values | Corrected Received Time difference |
| Assumption on training | Training type | offline training | offline training |
|  | Label construction (if applicable) | / | / |
| model location for inference |  | UE-sided model | UE-sided model |
| Collaboration/interaction between UE and NW |  | no collaboration/interaction | no collaboration/interaction |
| Evaluation methodology |  | system level simulation | system level simulation |
| Evaluation assumption |  | FFS | FFS |
| Evaluation KPI |  | - Difference between predicted AGC and measured AGC - Maximum difference among predicted AGCs | - Difference between predicted RTD and measured RTD 2. Maximum difference among predicted received Times |
| Evaluation benchmark |  | Use additional sample(s) to finish the AGC | Use additional sample(s) to finish the DL sync |
| Preliminary evaluation results |  | FFS | FFS |
| High level potential specification impact |  | Reduce measurement delay, reduce RS overhead and power saving | Reduce measurement delay, reduce RS overhead and power saving |
| Feasibility issues, including complexity, and other aspects related to implementation |  | FFS | FFS |

# 3 Conclusion

In this contribution, we give some initial views on the 6GR AI, the conclusions are summarized below:

Observation 1: The motivation of SRS-assisted channel reconstruction is to reduce the CSI-RS overhead, especially when deploying antennas on the larger scale.

Observation 2: The SRS-IL power imbalance shall be included in the SRS-AI assist channel reconstruction. Accurate channel estimation is a prerequisite for this reconstruction.

Observation 3: Some potential technical issues need to be discussed in RAN1, and the RACH access procedure may need to be discussed in RAN1 and RAN2.

Observation 4: RAN4 defined the requirements of linear receivers and non-linear receiver in 5G.

Observation 5: Exploring receiver technology based on AI is becoming a potential direction to break through the performance boundaries of classical theory.

Observation 6: The data driven approach regards multiple functional modules of the wireless communication system as an unknown and black box, replaces them with an AI network, and then relies on a large amount of training data input to output training.

Observation 7: The model driven approach, based on the existing technology of the wireless communication system, does not change the model structure of the wireless communication system, but uses a deep learning network to replace a certain module or train related parameters to improve the performance of a certain module.

Observation 8: Some use cases have been identified in 5G-A but were not included in work scope due to limited time and large workload, such as RLF, spatial domain, L3 beam level prediction and so on, which are better to be discussed with 5G as a whole.

Observation 9: From the perspective of measurement prediction, there maybe no difference between AI/ML based SCell related enhancement by measurement prediction and AI based L3 cell-level measurement prediction. Both use AI to reduce measurements without actually measuring the target cell, and obtain prediction results through historical information or measurement results of other cell(s).

Observation 10: NW usually has more comprehensive information than UE because the network has a global view and centralized control capability, while the UE's view is usually limited to itself and the nodes directly communicating with it.

Observation 11: The sub-use cases of Inter-Cell/TRP beam prediction and management and Tx-Rx beam pair prediction are discussing in RAN1.

Observation 12: For any inter-frequency target cell/frequency, before the cell search, UE needs to perform AGC operation unless UE maintains the previous AGC factor for the target cell/frequency and it is still valid. The demand of AGC operation is universal as long as RF re-tuning happens.

Proposal 1: RAN4 to prioritize the DPoD study for AI-nonlinearity compensation use case.

Proposal 2: For DPoD evaluation, two alternatives can be considered for PA model derivation:

-Alternative 1: To determine a common PA model for further evaluation

-Alternative 2: multiple PA models can be used for further evaluation as long as the relaxed EVM requirements can be derived reasonably

Proposal 3: Propose to postpone the related AI SRS assist channel reconstruction discussion in RAN4, to be led by RAN1.

Proposal 4: Propose to postpone the related AI based RACH receiver discussion in RAN4, to be led by RAN1.

Proposal 5: Propose to discuss whether RAN4 can lead the discussion on AI receiver in RAN4, such as legacy release discussed MMSE-IRC and advanced receivers.

Proposal 6: RRM relevant use cases that have been identified in 5G-A but not included in work scope due to limited time and large workload should be deprioritized in RAN4’s discussion.

Proposal 7: Study and develop the potential AI/ML use cases for RRM from the following perspectives:

- Type 1: Whether and how to apply AI/ML to reduce measurement resource overhead

- Type 2: Whether and how to apply AI/ML to reduce measurement/processing delay

- Type 3: Whether and how to apply AI/ML to reduce the interruption/measurement gap

Proposal 8: For Multi-Domain L3 level Prediction, it should be deprioritized and considered after significant progress is achieved for single-domain L3 level prediction to avoid redundant discussion.

Proposal 9: RAN4 should discuss how to coordinate with RAN1 on Tx-Rx beam pair prediction.

Proposal 10: Study the feasibility of AI/ML based AGC, DL sync in 6GR with consideration of some side conditions.

# 4	Reference

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

[2] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair

[3] R4-2514637, WF on 6G AI, Samsung

[4] R1-2508118, Moderator summary #5 on AI/ML for 6GR, Samsung (Moderator)

# Appendix

The parameters for the GMP model is:

yGMPn=k∈Kal∈Laaklxn−lxn−l2k+k∈Kbl∈Lbm∈Mbklmxn−lxn−m2k

akl;k∈[0,1,…,7]:

l=2:

[0.0145707+0.00223568i; 0.0166021+0.0884597i; -0.170987-0.889998i; 0.398012+4.25717i;
-0.922915-11.5296i; 1.51648+16.8822i; -1.31708-12.4992i; 0.443603+3.66282i]

l=1:

[-0.0730384-0.0608598i; 0.316437-0.130488i; -2.64289+1.95766i; 13.9617-8.92706i;
-35.9884+25.271i; 49.5323-38.9777i; -34.8388+30.1032i; 9.83576-9.12289i]

l=0:

[-0.369392-0.616894i; 0.582141-1.54129i; -4.2332+13.9746i; 15.4346-56.4738i;
-34.026+106.817i; 42.3779-83.2642i; -26.6004+5.86237i; 6.4982+15.2082i]

l=−1:

[-0.109009-0.0382752i; 1.34619-0.303139i; -7.57533+2.07457i; 30.8214-7.83883i;
-71.9119+13.7515i; 94.7172-10.8742i; -65.1891+3.05573i; 18.1882+0.14561i]

l=−2:

[0.0913878+0.029207i; -0.205695-0.0047561i; 0.436792+0.098933i; -0.0447736+0.802472i;
-1.91069-3.64271i; 3.53201+6.10853i; -2.64467-4.81807i; 0.741402+1.46945i]

bklm;k∈[0,1,…,7]:

l=1, m=0:

[-0.0732748-0.0617029i; 1.04861+0.216692i; -7.53774-2.85579i; 29.348+11.8762i;
-68.0727-19.8783i; 92.0079+4.93057i; -66.4247+17.5978i; 19.6384-12.2782i]

l=−1, m=0:

[-0.108885-0.0392921i; -0.65351-0.122316i; 2.7747+3.26333i; -6.41902-23.391i;
9.68476+79.745i; -10.5191-141.613i; 6.89414+125.231i; -1.92908-43.441i]