3GPP TSG-RAN WG4 Meeting #117	R4-2521694

Dallas, US, November 17th – 21st 2025

Agenda Item:	6.13.2.2

Source:	Ericsson

Title:	Impact on SBFD BS RF TX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on conformance testing of BS RF TX requirements for SBFD.

Discussion on conformance testing of BS RF RX requirements for SBFD and new requirements for SBFD are detailed in separate submissions [1-2].

# 2	Discussion

Conformance testing for both SBFD and non-SBFD slots

It is important to ensure that conformance testing is performed for both SBFD and non-SBFD slot. Even for requirements that remain unchanged, the transmitter and receiver operate under different conditions in SBFD, where simultaneous uplink and downlink transmission leads to different hardware behaviour. Therefore, separate conformance testing is necessary for both cases to capture any deviations in RF performance.

- The transmitter and receiver operate under different conditions in SBFD and non-SBFD slots, for which separate testing is necessary.

Conformance testing shall be performed separately for both SBFD and non-SBFD slots.

Manufacturer declarations

Conformance testing is based on predefined parameters such as carrier bandwidths, number of supported carriers, total output power, maximum power per carrier, etc. The current agreed declarations are not sufficient and need further clarification, e.g., it should be clearly declared whether the SBFD BS operates in single-carrier or multi-carrier mode, etc. As agreed in Rel-19 core phase, SBFD operation can be applied on only one carrier, regardless of how many carriers are supported. The declaration should show the association of SBFD carrier with the non-SBFD carriers.

Whether the SBFD BS operates in single-carrier or multi-carrier mode shall be declared, together with other necessary declarations as shown in draft CR [R4-2521696]. The declaration should show the association of SBFD carrier with the non-SBFD carriers.

RF channels to be tested for configuration-flexible SBFD-capable BS

Agreement: - For configuration-limited SBFD-capable BS, all declared RF channels that implement analogue filters shall be tested. - FFS RF channels to be tested for configuration-flexible SBFD-capable BS.

For conformance testing of configuration-flexible SBFD-capable BS, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed.

For conformance testing of SBFD slots, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed.

Receiver status during conformance testing of transmitter

When testing both conducted and OTA transmitter requirements for an SBFD-capable BS, the receiver should always be turned on. This ensures that the testing accurately reflects real-world operational conditions. The receiver’s presence during testing is critical, as it accounts for potential interactions between the transmitter and receiver, such as signal interference, leakage, and system stability. Additionally, keeping the receiver on during testing helps to simulate normal operating conditions, where the receiver continuously monitors and reacts to the transmitted signals. This approach ensures that the SBFD-capable BS is thoroughly evaluated for both transmitter and receiver performance under realistic scenarios.

When testing both conducted and OTA transmitter requirements for an SBFD-capable BS, the receiver should always be turned on, to ensure the transmitter is evaluated under realistic scenarios.

Test model

In RAN4#113, it was agreed to reuse the existing FRCs for conformance testing of the SBFD UL subband. While reusing the existing FRCs for UL subbands is a logical starting point, DL test models must be designed for scalability. Current DL test models do not consider flexible UL subband configurations, which are essential for SBFD operation. Therefore, a scalable test model for the DL subband, which is parameterized by the UL subband bandwidth, UL subband center frequency or its relative offset from the DL, etc., is needed to accurately evaluate SBFD performance under scalable conditions.

- The test model for SBFD DL subbband needs to be scalable.

- New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

In addition, an important technical concern is the impact of scalable test models on signal statistics. As DL subbands are scaled (e.g., by varying bandwidth or power), their signal statistics may change, potentially affecting the PA testing. To ensure accurate and comprehensive testing, quantitative assessment of signal statistics variations under different subband configurations/DL subband sizes and power levels.

- Signal statistics may change for SBFD test models. The impact of DL subband on signal statistics needs to be analyzed.

Signal statistics should be investigated to ensure the test model follows relevant properties of the waveform.

RAN4 use the TP below as a starting point for discussion on SBFD test model.

---------------------------------------------- Start of Text Proposal-----------------------------------------------------------

#### 4.9.2.2	FR1 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR FR1 test models (NR-FR1-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-1 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For FR1-TDD without NB-IoT operation in NR in-band, test models are derived based on the uplink/downlink configuration as shown in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon as defined in TS 38.331 [19].

Table 4.9.2.2-1: Configurations of TDD for BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

Common physical channel parameters for all NR FR1 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 and table 4.9.2.2-4 for PDSCH. Specific physical channel parameters for NR FR1 test models are described in clauses 4.9.2.2.1 to 4.9.2.2.8.

For DU:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

For UD:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 + NRB,SBFD,UL + NRB,SBFD,GB |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

For DUD:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL,1 – 3 + NRB,SBFD,DL,2 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for PDSCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| Mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | pos2 |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 1 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

For DU:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |

For UD:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 + NRB,SBFD,UL + NRB,SBFD,GB |
| Number of PRBs | 3 |

For DUD:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |
| PDSCH nRNTI=3 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |

##### 4.9.2.2.1	FR1 test model 1.1 (NR-FR1-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Transmit ON/OFF power

-	TAE

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Transmitter intermodulation

-	Receiver spurious emissions

Common physical channel parameters are defined in clause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM1.1 are defined in table 4.9.2.2.1-1.

For DU:

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

For UD:

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 + NRB,SBFD,UL + NRB,SBFD,GB |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 + NRB,SBFD,UL + NRB,SBFD,GB |

For DUD:

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| # of PRBs PDSCH nRNTI=3 | NRB,SBFD,DL,2 |
| Modulation PDSCH nRNTI=3 | QPSK |
| Starting RB location of PDSCH nRNTI=3 | NRB,SBFD,DL,1 + NRB,SBFD,UL + 2 * NRB,SBFD,GB |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 + NRB,SBFD,UL + NRB,SBFD,GB |

------------------------------------- End of Text Proposal--------------------------------------------------------------

Test signal

Issue 1-1-4: Test signal used to build Test Configurations when testing existing Tx requirements Agreement: - For configuration-limited SBFD-capable BS, Table 1 shall be used. FFS SCS in the table. - Table 1: Signal to be used to build NR SBFD TCs SBFD BS type; Configuration-limited. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; [30] kHz for FR1 [120] kHz for FR2-1. Subband configuration; All declared subband configurations. - For configuration-flexible SBFD-capable BS, use Table 2 as a starting point. FFS SCS in the table. - Table 2: Signal to be used to build NR SBFD TCs SBFD BS type; Configuration-flexible. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; [30] kHz for FR1 [120] kHz for FR2-1. Subband configuration; DUD; DU/UD. - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widese UL subband size, and the largest NRB,SBFD,DL,1/2.; - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL..

Regarding the test signals, agreements were reached in RAN4#116bis meeting, and SCS needs to be further studied. In our view, for each individual band, the typical numerology that is used should be used for testing, for example, for n77, n78, n104 in FR1, 30 kHz is used for SCS instead of 15 kHz. If any bands do not support 30 kHz, it could be listed as exceptions and use 15 kHz.

For FR1, use 30 kHz subcarrier spacing (SCS) for the test signal used to build test configuration (TC). For FR2, use 120 kHz subcarrier spacing (SCS) for the test signal used to build test configuration (TC).

Test configuration

As discussed above regarding the manufacturer declarations, whether the SBFD BS operates in single-carrier or multi-carrier mode should be clearly declared. For multi-carrier operation, the interaction between carriers should be tested together. The test configuration should include one SBFD carrier and the associated non-SBFD carriers. A new test configuration should be created, as presented in the submitted draft CR [R4-2521696].

For single-carrier and multi-carrier SBFD operation, a new test configuration for contiguous spectrum operation shall be created including one SBFD carrier and the associated non-SBFD carriers, as shown in draft CR [R4-2521696].

Measurement uncertainty (MU) and test tolerance (TT)

Issue 1-1-9: MU and Test tolerance when testing existing Tx requirements Agreement: - Re-use existing MU for SBFD symbols testing requirements as a starting point. - FFS re-use existing TT for SBFD symbols testing requirements.

While the existing measurement uncertainty (MU) is set to be re-used for SBFD symbols as a starting point, there is a need for further study to determine the applicability of re-using existing test tolerance (TT) for SBFD symbols testing. Test tolerance, which refers to the permissible deviation from nominal values, plays a crucial role in determining the acceptable limits for SBFD symbol performance. In our view, the existing TT could also be reused for SBFD conformance testing requirements as a starting point.

Re-use existing test tolerance (TT) for SBFD conformance testing requirements as a starting point.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The transmitter and receiver operate under different conditions in SBFD and non-SBFD slots, for which separate testing is necessary.

Observation 2	The test model for SBFD DL subbband needs to be scalable.

Observation 3	New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

Observation 4	Signal statistics may change for SBFD test models. The impact of DL subband on signal statistics needs to be analyzed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Conformance testing shall be performed separately for both SBFD and non-SBFD slots.

Proposal 2	Whether the SBFD BS operates in single-carrier or multi-carrier mode shall be declared, together with other necessary declarations as shown in draft CR [R4-2521696]. The declaration should show the association of SBFD carrier with the non-SBFD carriers.

Proposal 3	For conformance testing of SBFD slots, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed.

Proposal 4	When testing both conducted and OTA transmitter requirements for an SBFD-capable BS, the receiver should always be turned on, to ensure the transmitter is evaluated under realistic scenarios.

Proposal 5	Signal statistics should be investigated to ensure the test model follows relevant properties of the waveform.

Proposal 6	RAN4 use the TP below as a starting point for discussion on SBFD test model.

Proposal 7	For FR1, use 30 kHz subcarrier spacing (SCS) for the test signal used to build test configuration (TC). For FR2, use 120 kHz subcarrier spacing (SCS) for the test signal used to build test configuration (TC).

Proposal 8	For single-carrier and multi-carrier SBFD operation, a new test configuration for contiguous spectrum operation shall be created including one SBFD carrier and the associated non-SBFD carriers, as shown in draft CR [R4-2521696].

Proposal 9	Re-use existing test tolerance (TT) for SBFD conformance testing requirements as a starting point.

# References

R4-2521695, Impact on SBFD BS RF RX requirements, Ericsson

R4-2521693, New SBFD BS RF requirements, Ericsson

R4-2521696, DraftCR to 38.141-1 for introduction of SBFD, Ericsson