3GPP TSG-RAN WG4 Meeting#117                                R4-2521756

Dallas, USA, 17th – 17th, Nov. 2025

Agenda item:	7.2.4

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Rel-20 SBFD BS coexisting with SBFD BS

Document for:	Approval

Introduction

In the RAN#109 meeting, a new WID: New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2 has been approved with Rel-19 left over issue: SBFD BS coexisting SBFD BS in adjacent channel included. In this contribution, we would like to share our initial understanding how to conduct the SBFD BS coexisting with SBFD BS in the adjacent channel during the Rel-20.

SBFD BS to SBFD BS adjacent channel co-existence - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs

Discussion

## 2.1. General

First of all, we want to further discuss the spectrum allocation for the bands n41, n79 and n104 on the market.

For band n41, from our knowledge from the commercial deployment, this NR band n41 is mainly deployed in China, US, Japan and Latin America. As shown in Figure 2.1-1/2, it could be easily found that this band n41 in China is mainly hold by a single operator without any inter-operator adjacent channel coexistence scenario. This band n41 in US is also sharing the similar story which is hold by a single operator without any inter-operator adjacent channel coexistence scenario. For band n41 in Japan as shown in Figure 2.1-2, the spectrum of two operators are allocated with a 20 MHz gap in the middle, in other words, there is also no inter-operator adjacent channel coexistence scenario identified as far in Japan from our understanding.

Figure 2.1-1. The illustration of allocated spectrum at band n41 in China

Figure 2.1-2. The illustration of allocated spectrum at band n41 in Japan

Observation 1: For band n41 in the commercialized regions, there are no inter-operator adjacent channel coexistence scenarios identified so far from our understanding.

For band 79, from our knowledge from the commercial deployment, this NR band n41 is mainly deployed in China, Japan. As shown in Figure 2.1-3/4, it share the similar observation as the deployment of band n41 in China and Japan.

Figure 2.1-3. The illustration of allocated spectrum at band n79 in China

Figure 2.1-4. The illustration of allocated spectrum at band n79 in Japan

Observation 2: For band n79 in the commercialized regions, there are no inter-operator adjacent channel coexistence scenarios identified so far from our understanding.

As shown in Figure 2.1-5, for band n104, a contiguous 700 MHz spectrum band has been allocated so far, considering the multiple operators in the certain regions, it is quite possible to face the SBFD BS coexisting with SBFD BS in the adjacent channel case.

Figure 2.1-5. The illustration of potential spectrum from 6425-7125MHz for the convenience of discussion in RAN4

Observation 3: For band n104 in the commercialized regions, it is possible to have SBFD BS coexisting with SBFD BS in the adjacent channel case.

Usually in the past, RAN4’s BS RF RF requirement is somehow agnostic to the actual spectrum allocation in field to define the more general requirement to simplify the BS RF implementation instead of the customized design for certain specific deployment. But considering the much higher implementation difficulties for FR1 WA SBFD BS than the legacy NR BS, we also want to take the actual deployment in field in account to do more appropriate analysis and draw the conclusion on the potential much stringent Tx/Rx requirement due to BS2BS CLI problem. According to the above analysis across the example NR bands as listed in the WID, we would like to prioritize the band n104 in Rel-20 discussion with the adjacent channel deployment assumption. For band n79 and n41 scenario, it is proposed to discus the deployment assumption with the certain frequency gap between different operators which is more aligned with the practical deployment.

Proposal 1: Prioritize SBFD BS coexisting with SBFD BS in the adjacent channel in band n104 in Rel-20; for band n79 and n41, propose to consider the SBFD BS coexisting with SBFD BS with the certain frequency gap taken into account.

## 2.2. Coexistence study

- In the current WID, both co-located and non-collocated are included. For the co-located deployment, it is not appropriate to use the legacy pathloss model as captured in TR 38.901 which is based on the far field assumption. During the Rel-19 BS RF enhancement, there were some initial measurement results and simulation results between Array to sub-array and Array to Array captured in TR 37.941 clause 6.4.4 [5]. For the SBFD BS vs SBFD coexistence study in the co-located deployment, we propose to consider one typical value for the coexistence study which could be based on values from TR 37.941.

- Proposal 2: For the SBFD BS vs SBFD coexistence study in the co-located deployment, propose to consider one typical value for the coexistence study which could be based on values from TR 37.941.

- In addition, in the previous Rel-19 co-location analysis, it is just based on the assumption of 10cm separation distance between two BSs to mimic 30dBc coupling loss in the past. But when we check the practical AAU installation guidance in field as shown in Figure 2.2-1 for horizontal placement and Figure 2.2-2 for vertical placement, RAN4 assumption for the conformance testing in the horizontal domain is much more stringent than the recommended installation in field. In other words, we should also consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption.

Figure 2.2-1. The illustration of horizontal co-location deployment for two SBFD BSs from different operators

Figure 2.2-2. The illustration of vertical co-location deployment for two SBFD BSs from different operators

- Proposal 3: For BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption.

- Another important issue is to conduct the appropriate study for SBFD BS coexisting with SBFB BS in Rel-20. In the Rel-19, there were some simulation results with 10%, 20% and 50% grid shift in [4] provided for initial discussions and also some technical analysis for the co-located BS2BS CLI analysis per BS site.

- In the following Table E.2.1-1, we made some necessary modification change (e.g. grid shift, carrier frequency) on top of Annex E in TR 38.858 during the Rel-18 SBFD SI phase.

Table E.2.1-1: FR1 deployment parameters

| Parameter | Urban Macro (Note 1) | Remarks |  |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. |  |  |
| Inter-BS distance | 500 m |  |  |
| Grid shift | Co-located: 0% grid shift Non-colocated: 10%, 20%, 50% grid shift |  |  |
| Carrier frequency | [2.6GHz], 4 GHz, 6GHz | Discuss whether 2.6GHz and 4GHz should use the same evaluation results for it. |  |
| Path-loss model | BS-to-UE: UMa + penetration loss, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.828 UE-to-UE: UMi (h_BS = 1.5 m) + penetration loss, see TR 38.803 (Note 2). UMi model is not applicable when 2D distance is less than 10 m, instead free space path loss model is applicable. |  |  |
| LOS probability | BS-to-UE: UMa, see TR 38.803 BS-to-BS: Option 1: UMa (h_UT = 25 m), see TR 38.803. Option 2: If the 2D distance between two BS is less than or equal to the ISD, set the LOS probability to X; X = 0.75. Otherwise, UMa (h_UT = 25 m), see TR 38.828. UE-to-UE: UMi, see TR 38.803 |  |  |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value |  |  |
| BS height | 25 m |  |  |
| UE height | 1.5 m |  |  |
| UE distribution | Uniformly UE distribution (Note 1) Baseline: 20% indoor and 80% outdoor Optional: 80% indoor and 20% outdoor |  |  |
| O-to-I penetration loss model | 80% low-loss model and 20% high-loss model |  |  |
| Minimum BS-UE (2D) distance | 35 m |  |  |
| Minimum UE-UE (2D) distance | 3 m |  |  |
| DL power control | No |  |  |
| UL power control | Yes |  |  |
| Handover margin | 3 dB |  |  |
| BS mechanical down-tilt angle | 6 degrees |  |  |
| Note 1: The Urban Hotspot reuses the assumptions of Urban Macro, except Urban Hotspot assumes cluster-based UE dropping. See Table E.2.1-3 for cluster-based dropping. Note 2: For UE-UE penetration loss, the following is used to generate d2D−in for a UE-UE link associated with an indoor UE (the other UE could be an outdoor UE or an indoor UE in a different building) in order to calculate the inside loss component (PLin) of the UE-UE O-to-I building penetration loss: - d2D−in =Uniform(0,min⁡{25m,d2D2}) |  |  |  |

Table E.2.3-1: FR1 BS RF parameters

| Parameter | Urban Macro (Wide Area) 4GHz, 2.6GHz | Urban Macro (Wide Area) 7GHz | Remark |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | SBFD config 1: 46 dBm SBFD config 2: 49 dBm | SBFD config 1: 49 dBm SBFD config 2: 52 dBm |  |
| Antenna configuration | SBFD config 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) SBFD config 2 (baseline): (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | SBFD config 1: (Mg,Ng,M,N,P)= (1,1,8,8,2) SBFD config 2 (baseline): (Mg,Ng,M,N,P)= (1,1,8,16,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | For 7GHz, we propose to double the antenna array size compared with 4GHz assumption at least. For the extended AAS model in TR 38.803 and discuss whether to apply the larger antenna array size in TR 38.922 for 7-8.4GHz to 7GHz SBFD BS. |
| Receiver noise figure model parameters | Baseline: P1=-43 dBm P2=-25 dBm F1=5 dB F2=14 dB Optional (Enhanced NF model): P1=-33 dBm P2=-15 dBm F1=5 dB F2=14 dB See Figure E.2.3-1 | Baseline: P1=-43 dBm P2=-25 dBm F1=5 dB F2=14 dB Optional (Enhanced NF model): P1=-33 dBm P2=-15 dBm F1=5 dB F2=14 dB See Figure E.2.3-1 |  |
| ACLR | 45 dBc | 38 dBc |  |
| ACS | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc |  |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) |  |

- Proposal 4: For SBFD BS coexisting with SBFD BS, propose to use the simulation assumptions in Table E.2.1-1 and E.2.3-1 in this contribution and other relevant simulation assumptions in TR 38.858 to conduct the coexistence study.

## 2.3. Initial simulation results

### 2.3.1 DU vs DU case

#### 2.3.1.1. 10% grid shift

NOTE: for 7GHz simulation, due to the limited time, antenna configuration is the same as 4GHz and 2.6GHz. More simulation results could be provided later on for the larger antenna array size.

#### 2.3.1.1. 20% grid shift

NOTE: for 7GHz simulation, due to the limited time, antenna configuration is the same as 4GHz and 2.6GHz. More simulation results could be provided later on for the larger antenna array size.

#### 2.3.1.3. 50% grid shift

NOTE: for 7GHz simulation, due to the limited time, antenna configuration is the same as 4GHz and 2.6GHz. More simulation results could be provided later on for the larger antenna array size.

#### 2.3.1.4. 0% grid shift

For 0% grid shift case, since aggressive BS and victim BS are co-located and it is not appropriate to use the large scale pathloss model in far field for this case, then we assume the following isolation matrix according to the reported isolation value in TR 38.858 for the feasibility study.

Isolation matrix is defined for 3 sector case, therefore the dimension of isolation matrix between 3 aggressor sectors and 3 victim sectors are assumed as following:

Isolation matrix for A-A= [80 80 80; 80 80 80; 80 80 80];

Isolation matrix for A-SA= [70 70 70; 70 70 70; 70 70 70];

NOTE: for 7GHz simulation, due to the limited time, antenna configuration is the same as 4GHz and 2.6GHz. More simulation results could be provided later on for the larger antenna array size.

#### 2.3.1.5. Summary

- Observation 4: for aggressor BS and victim BS with 0% grid shift at 2.6GHz, 4GHz and 7GHz, the additional ACIR offset 30dB on top of baseline ACIR 42.6dB is needed which basically means that analog filter attenuation is needed.

- Observation 5: for aggressor BS and victim BS with 10% to 50% grid shift, the additional ACIR offset ranging from 30 to 40dB on top of baseline ACIR 42.6dB is needed. For different frequency, the additional ACIR offset varies.

- In the following table, we also summarized the in-band blocking level at 99% for different grid shift and different frequency range based on 4GHz and 2.6GHz antenna configuration.

- Table 2.3.1.5-1. the summary of in-band blocking at 99% of CDF curve

| Scenario | Grid shift | Per antenna elements [dBm] | Per antenna array [dBm] |
| --- | --- | --- | --- |
| 4GHz | 0% | -17.2092 | -26.7459 |
|  | 10% | -6.1030 | -6.4739 |
|  | 20% | -11.0219 | -11.9501 |
|  | 50% | -19.6189 | -22.7678 |
| 2.6GHz | 0% | -17.1836 | -26.5314 |
|  | 10% | -1.9850 | -3.0574 |
|  | 20% | -5.8347 | -7.9595 |
|  | 50% | -14.8446 | -18.0302 |
| 7GHz | 0% | -17.2163 | -27.0497 |
|  | 10% | -9.7003 | -11.1506 |
|  | 20% | -15.2622 | -17.1616 |
|  | 50% | -23.6256 | -25.8747 |
| NOTE 1: 3dB polarization gain is considered between aggressor BS and victim BS in the adjacent channel case as the worst case assumption. |  |  |  |

- Proposal 5: Propose to consider the analog filter to provide the additional ACIR protection e.g. 30-40dBc.

### 2.3.2 DUD vs DUD case

- To to added later.

### 2.3.3 DUD vs DU case

- To to added later.

## 2.4. RF requirement and feasibility study

- During the Rel-18 SBFD SI phase, the SBFD BS coexisting with SBFD BS in the adjacent channel scenario was not evaluated, however the CLI interference is quite similar as Scenario 1, Case 3 where NR DL interfering SBFD UL, the severe performance degradation could be observed for SBFD UL due to the strong CLI from NR DL in the adjacent channel. Even with ACIR requirement with 8dB tightening, the 5% throughput loss is still up to 43.5% and the mean throughput loss could be minimized down to 4.67%. However as we know that, for AAS based SBFD BS in FR1 high bands, due to the higher operating frequency and larger channel bandwidth and relative worse transceiver’s linearity, there are no much room for the improvements of ACLR/OBUE requirement. However by utilizing the following example configuration with the help of front-end filter from both aggressor’s transmitter side and victim’s receiver side as shown in Figure 2.2-1, this could efficiently alleviate the implementation burden from RF front-end perspective to achieve the potential more tightening transmitter unwanted emission /receiver blocking requirement defined in Rel-20. This should be also under the condition of DL-UL sub-band coordination/recommendation from the deployment in certain regions.

Figure 2.3-1. The illustration of the spectrum allocation of DL-UL sub-band and Tx/Rx front-end filter

- Proposal 6: For the potential tightening unwanted emission and blocking requirement for SBFD BS in Rel-20, it is proposed to consider the victim’s system DL sub-band as guard band to minimize the victim system’s performance degradation;

- As shown in Figure 2.2-1, regarding how to guarantee the SBFD vs SBFD coexistence performance between two different SBFD carries, it is highly dependent on the following factors:

- Potential more tightening emission and blocking requirements from coexistence study

- DL sub-band size of the victim system;

- Coupling loss between the aggressor and victim system for co-site case

- Analog filter performance of aggressor’s transmitter

- Analog filter performance of victim’s receiver;

- Proposal 7: For RF requirements of SBFD BS in Rel-20, propose to consider the following factors:

- Potential more tightening emission and blocking requirements from coexistence study

- DL sub-band size of the victim system;

- Coupling loss between the aggressor and victim system for co-site case

- Analog filter performance of aggressor’s transmitter

- Analog filter performance of victim’s receiver;

Here is one example for the initial discussion. If the analog filter attenuation at the certain freq offset away from the RF bandwidth edge could be achieved as 34dBc together with MCL as 67dB which will results in -116dBm/MHz. This leakage interference is supposed to be 7dB lower than victim’s noise figure which is also aligned with the legacy principle for co-location spurious requirement. However we might need to more time to verify this coupling loss based on the U6GHz BS in the OTA chamber. Meanwhile more analysis might be needed to get the baseline/default value for the required freq offset to achieve 34dBc attenuation, otherwise it will be very ambiguous to define this requirement with the declared freq offset to meet the requirement which will cause lots of product fragmentation.

Some initial analysis for analog filtering design at U6GHz is provided as following for the information, more data would be provided later on.

|  | Freq offset to the operators’ spectrum edge [U6GHz] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS RF bandwidth | 10MHz | 15MHz | 20MHz | 40MHz | F1 | F2 |
| 100MHz | 2.2 | 3 | 10 | 20 | 34? | TBD |
| 200MHz | TBD | TBD | TBD | TBD | TBD | TBD |
| 300MHz | TBD | TBD | TBD | TBD | TBD | TBD |
| 400MHz | TBD | TBD | TBD | TBD | TBD | TBD |

## 2.5. Work procedure for Rel-20 SBFD study

In the following Figure 2.5.1, we provide some initial understanding on how to organize the discussion in RAN4 from the procedure wise.

Figure 2.5-1. The illustration of working procedure for SBFD study in Rel-20

Proposal 8: propose to consider the work procedure in RAN4 for Rel-20 SBFD vs SBFD coexistence study.

Conclusions

In this contribution, we want to share some initial views on Rel-20 SBFD BS study and proposals/observations are made as following:

Observation 1: For band n41 in the commercialized regions, there are no inter-operator adjacent channel coexistence scenarios identified so far from our understanding.

Observation 2: For band n79 in the commercialized regions, there are no inter-operator adjacent channel coexistence scenarios identified so far from our understanding.

Observation 3: For band n104 in the commercialized regions, it is possible to have SBFD BS coexisting with SBFD BS in the adjacent channel case.

Proposal 1: Prioritize SBFD BS coexisting with SBFD BS in the adjacent channel in band n104 in Rel-20; for band n79 and n41, propose to consider the SBFD BS coexisting with SBFD BS with the certain frequency gap taken into account.

- Proposal 2: For the SBFD BS vs SBFD coexistence study in the co-located deployment, propose to consider one typical value for the coexistence study which could be based on values from TR 37.941.

- Proposal 3: For BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption.

- Proposal 4: For SBFD BS coexisting with SBFD BS, propose to use the simulation assumptions in Table E.2.1-1 and E.2.3-1 in this contribution and other relevant simulation assumptions in TR 38.858 to conduct the coexistence study.

- Observation 4: for aggressor BS and victim BS with 0% grid shift at 2.6GHz, 4GHz and 7GHz, the additional ACIR offset 30dB on top of baseline ACIR 42.6dB is needed which basically means that analog filter attenuation is needed.

- Observation 5: for aggressor BS and victim BS with 10% to 50% grid shift, the additional ACIR offset ranging from 30 to 40dB on top of baseline ACIR 42.6dB is needed. For different frequency, the additional ACIR offset varies.

- Proposal 5: Propose to consider the analog filter to provide the additional ACIR protection e.g. 30-40dBc.

- Proposal 6: For the potential tightening unwanted emission and blocking requirement for SBFD BS in Rel-20, it is proposed to consider the victim’s system DL sub-band as guard band to minimize the victim system’s performance degradation;

- Proposal 7: For RF requirements of SBFD BS in Rel-20, propose to consider the following factors:

- Potential more tightening emission and blocking requirements from coexistence study

- DL sub-band size of the victim system;

- Coupling loss between the aggressor and victim system for co-site case

- Analog filter performance of aggressor’s transmitter

- Analog filter performance of victim’s receiver;

Proposal 8: propose to consider the work procedure in RAN4 for Rel-20 SBFD vs SBFD coexistence study.

Figure 2.5-1. The illustration of working procedure for SBFD study in Rel-20

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- RP-252905, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, ZTE Corporation.

- R4-2507682, Discussion on existing Tx requirement for SBFD operation, Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom, Deutsche Telekom

- RP-252586, CR to TR 37.941: Rel-19 CLTA-related agreements, Approved.

- R4-2515091, Way Forward for [116bis][317] NR_BS_RF_Ph2, Ericsson.