3GPP TSG-RAN WG4 Meeting #117	R4-2521769

# Dallas, USA, November 17th – 21st 2025

Source:	Ericsson

Title:	Discussion on Rel-19 Ku band impact on NTN demodulation requirements

Agenda item:	5.10.3

Document for:	Discussion

# Introduction

In RAN4#116-bis meeting, companies have following agreements [1]:

Whether to define requirements

Agreement

Do not define SAN demodulation requirements

Whether to define requirements for FR1

Agreement

RAN4 could consider both PDCCH and PDSCH requirement could be defined for both 15KHz and 30KH SCS for Ku-band

Whether to define requirements for FR2

Agreement

Do not introduce new UE demodulation requirements for FR2-NTN Ku band

Whether to define CSI Requirements

Agreement

No CSI reporting requirements will be defined for NTN with Ku-band.

In this contribution, the detailed simulation cases and parameters for PDSCH and PDCCH for 15KHz and 30KH SCS on Ku bands are further discussed.

# Discussion

## 2.1	PDSCH

The tentative simulation parameters for PDSCH were discussed in previous meeting. The common configurations could follow Ka band configurations in NTN-FR2 requirements with some modifications according to different SCS configurations, see Table 2.1-1 in our simulation contribution [2].

Based on our simulation results [2], the NTN-TDLC5-1200 seems not feasible for 15kHz SCS with 1x1 antenna configuration which the peak throughput can’t be achieved. We also compared other Doppler shift models, such as 600Hz and 200Hz, and they are all feasible. In that case, we suggest companies to double check the channel feasibility and change Doppler shift to a lower value (e.g., 600Hz) for Ku band FR1 requirements.

Figure 2.1-1 Performance comparison among different channel models

The channel model NTN-TDLC5-1200 is not feasible for 15kHz SCS PDSCH requirements.

Proposal 1	Take lower Doppler shift for Ku band FR1 demodulation requirements, such as NTN-TDLC5-600.

## 2.2	PDCCH

The test case scope for PDCCH of Ku band FR1 was not discussed during previous meeting. Referring to Ka band PDCCH requirements, both DCI format 1_0 and 1_1 are covered. It could be followed for Ku band with different SCS configurations.

Proposal 2	Cover both DCI format 1_0 and 1_1 for Ku band PDCCH demodulation requirements.

The channel model could follow the discussion on PDSCH conclusion and use same model as PDSCH.

The reference channel can refer to Annex A.

In that case, the test configurations could take Table 2.2-1.

Table 2.2-1 Test configurations for PDCCH on Ku band

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 2-1 | 50 | 270 | 1 | 8 | R.PDCCH.A-1.1 FDD | TBD | 1x1 | 1 | TBD |
| 2-2 | 50 | 270 | 1 | 16 | R.PDCCH.A-1.2 FDD | TBD | 1x1 | 1 | TBD |
| 2-3 | 50 | 132 | 1 | 8 | R.PDCCH.A-1.3 FDD | TBD | 1x1 | 1 | TBD |
| 2-4 | 50 | 132 | 1 | 16 | R.PDCCH.A-1.4 FDD | TBD | 1x1 | 1 | TBD |

Proposal 3	Take cases in Table 2.2-1 for PDCCH demodulation requirement on NTN Ku band.

Proposal 4	Take same channel model as PDSCH demodulation for PDCCH requirement on NTN Ku band.

Proposal 5	Take FRC table in A-1 for PDCCH demodulation requirements on NTN Ku band.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The channel model NTN-TDLC5-1200 is not feasible for 15kHz SCS PDSCH requirements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Take lower Doppler shift for Ku band FR1 demodulation requirements, such as NTN-TDLC5-600.

Proposal 2	Cover both DCI format 1_0 and 1_1 for Ku band PDCCH demodulation requirements.

Proposal 3	Take cases in Table 2.2-1 for PDCCH demodulation requirement on NTN Ku band.

Proposal 4	Take same channel model as PDSCH demodulation for PDCCH requirement on NTN Ku band.

Proposal 5	Take FRC table in A-1 for PDCCH demodulation requirements on NTN Ku band.

# References

[1] 	R4-2515160, Way Forward for [116bis][312] Rel-19 Demodulation_Part2, Nokia

[2]	R4-2521770, Simulation results for Rel-19 NR NTN Ku band demodulation, Ericsson

# Annex A FRC table for PDCCH

Table A-1 PDCCH reference channels

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.A-1.1 FDD | R.PDCCH.A-1.2 FDD | R.PDCCH.A-1.3 FDD | R.PDCCH.A-1.4 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 30 | 30 |
| CORESET frequency domain allocation |  | 270 | 270 | 132 | 132 |
| CORESET time domain allocation |  | 1 | 2 | 1 | 2 |
| Aggregation level |  | 8 | 16 | 8 | 16 |
| DCI Format |  | 1_0 | 1_1 | 1_0 | 1_1 |
| Payload (without CRC) | Bits | 44 | 56 | 42 | 56 |