3GPP TSG-RAN WG4 Meeting #116bis                                           R4-2521887

# Prague, Czech Republic, October 13-17, 2025

Title:	Qualcomm views on 5G and 6G spectrum sharing

Source:	Qualcomm Incorporated

Agenda item:	8.10

Document for:	Approval

# Introduction

The 6G study [1] within RAN4 has been structured into multiple tracks [3], one of which focuses on 5G–6G spectrum sharing. This paper highlights RAN4 considerations that will be influenced by the spectrum-sharing and migration framework between 5G and 6G.

It is important to note that most design decisions for 5G–6G sharing will be made in RAN, RAN1 and RAN2, and the RAN4 study and subsequent design work will need to align with those outcomes. Consequently, significant progress in RAN4 is expected once the high-level requirements and details become clearer.

# 2. Discussion

### 2.1 Spectrum and bands

All spectrum currently used for NR or LTE is expected to transition to 6G over time. Therefore, RAN4 should examine how to enable smooth 6G deployment in bands already defined in 3GPP specifications. Some operators also plan to implement 5G–6G spectrum sharing in these bands. To support this migration, RAN4 should study the possibility of adopting the same or very similar RF requirements as those defined for NR, especially regulatory requirements such as emissions.

Proposal 1: RAN4 should study how to enable smooth migration to 6G and support 5G–6G sharing in existing bands.

Proposal 2: RAN4 should study the feasibility of adopting the same or similar RF requirements for 6G as for NR (particularly regulatory requirements) to facilitate easier refarming of existing bands.

### 2.1 Channel raster and spectrum sharing

NR currently supports two channel raster options: the 100 kHz raster and the SCS-based raster. Although the SCS-based raster offers several advantages, the 100 kHz raster was retained in most legacy LTE/3G bands to enable LTE–NR Dynamic Spectrum Sharing (DSS).

As noted in [4], the 100 kHz raster introduces several inefficiencies; however, it was originally defined to facilitate spectrum sharing. Whether this raster remains necessary—or can be phased out—will largely depend on the 5G–6G spectrum-sharing framework. Most aspects of this framework will be determined by other working groups (RAN, RAN1, RAN2) and will directly influence RAN4 studies and specifications.

Proposal 3: RAN4 should evaluate the implications of the 5G–6G spectrum-sharing framework on channel raster design and identify the most suitable solutions for the 6G channel raster (6GR).

### 2.2 RRM Related aspects

The RAN4 study on Radio Resource Management (RRM) for 6G will need to account for the 5G–6G spectrum‑sharing and migration framework. These aspects will have a direct impact on RRM requirements and procedures such as:

- Mobility management: Ensuring seamless 5G-6G operation across shared and dedicated spectrum, with minimal interruption and overhead, introduction of inter-RAT handover requirements, etc

- Measurement configurations: Measurement gaps, measurement periods, etc, will have to take into account the characteristics of signals to be measured (periodicity, signal transmission length, bandwidth, etc) for both 5G and 6G.

Proposal 4: RAN4 should study the impact of 5G–6G spectrum sharing and migration on RRM requirements, including mobility management and measurement procedures.

# 3. Conclusion

In this paper we provided a brief analysis of the 6G study directions for 5G-6G spectrum sharing and migration and some of the impact to the RAN4 discussions and future specifications. We made the following proposals for the 6G study:

Proposal 1: RAN4 should study how to enable smooth migration to 6G and support 5G–6G sharing in existing bands.

Proposal 2: RAN4 should study the feasibility of adopting the same or similar RF requirements for 6G as for NR (particularly regulatory requirements) to facilitate easier refarming of existing bands.

Proposal 3: RAN4 should evaluate the implications of the 5G–6G spectrum-sharing framework on channel raster design and identify the most suitable solutions for the 6G channel raster (6GR).

Proposal 4: RAN4 should study the impact of 5G–6G spectrum sharing and migration on RRM requirements, including mobility management and measurement procedures.

As noted in the previous sections, most design decisions for 5G–6G sharing will be made in RAN, RAN1 and RAN2, and the RAN4 study and subsequent design work will need to align with those outcomes.

# References

[1]	RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0, June 2025

[3]	R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple), 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[4]	R4-2513020, “Qualcomm Views on 6G System Parameters”, TSG-RAN WG4 Meeting#116Bis