# 3GPP TSG-RAN WG4 Meeting #117                                                                    R4-2522005

# Dallas, US,  November 17 - 21, 2025

Source:		Ericsson

Title:			VSAT RF requirement for HD-FDD

Agenda item:		7.6.4.1

Document for:		Approval

# Introduction

In this paper, we present our overview on HD-FDD Vsat terminal RF impact.

# Discussion

In WF[1], Reusing the ON/OFF time mask is considered.

#### Issue 1-1-3: Transmit ON/OFF time mask for HD-FDD

- WF

  - Consider reusing existing Transmit ON/OFF time mask requirement as starting point.

Last meeting some company propose to consider new ON/OFF mask for Tx/Rx or Rx/Tx switch. Indeed,  the Rx-Tx and Tx-Rx switch time is defined in Table 4.3.2-3 in 38.211 and quoted below.

“A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by Table 4.3.2-3.

A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where NTx-Rx is given by Table 4.3.2-3.”

As Tx or Rx switch is defined in RAN1 specification for HD-FDD, there is no need to define new one in RF specification.

Reuse the existing Transmit ON/OFF time mask requirement

For EIPR and EIS, the existing requirements are derived from link budget for GSO and LEO respectively and it seems there is no margin left to relax it further. Therefore, we propose to reuse the existing ones.

Reuse the existing EIRP/EIS requirements.

#### Issue 1-2-3: ACLR and ACS for the new VSAT with small antenna

- WF

  - Consider the existing Ku-band ACLR and ACS as the starting point

  - FFS on whether to modify.

    - Encourage companies to provide justifications for whether to re-use the existing specifications.

For the ACLR and ACS, this is specified from coexisting study. We think there is no need to re-do the coexisting study to derive new one as EIRP/EIS may not be changed.

Reuse the existing ACLR/ACS requirements and no new coexisting study needed.

# Conclusions

In this contribution, we present our overview on HD-FDD VSAT terminal RF impact.with below proposals:

Proposal-1:Reuse the existing Transmit ON/OFF time mask requirement

Proposal-2:Reuse the existing EIRP/EIS requirements.

Proposal-3:Reuse the existing ACLR/ACS requirements and no new coexisting study needed.

# References

- R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MediaTek